3GPP TSG SA WG 4 Meeting #134 	S4-251768

17-21 November 2025, Dallas, Texas, USA

Source:	Tencent

Title:	[FS_3DGS_MED] Pseudo-CR on use case 3: playback of dynamic 3DGS content

Spec:	3GPP Draft TR 26.958 v0.0.1

Agenda item:	9.7

Document for:	Agreement

1. Introduction

3D Gaussian Splatting (3DGS) has recently emerged as a photorealistic 3D representation that is lightweight enough for real-time rendering on mobile GPUs. FS_3DGS_MED study has been created to evaluate the support of 3DGS content in 3GPP systems, including capture on device, transmission over 3GPP services, decoding, and rendering on mobile devices.

This contribution proposes Use Case 3 (UC3): playback of dynamic 3DGS content, such as short volumetric performances or capture or a person. The focus is on time-varying 3DGS sequences delivered for on-demand to mobile devices.

We propose that this use case is used as baseline for requirement derivation, traffic analysis, and reference implementation for the following FS_3DGS_MED study evaluation.

2. Reason for Change

We propose that FS_3DGS_MED studies the following scenario: playback of dynamic 3DGS content.

In this use case, a user launches an application and selects a 3DGS dynamic scenes, then the UE receives time-varying 3DGS content representing dynamic subjects such as a dancer, athlete, singer, band, sporting action, etc…  The content is streamed or downloaded in real time, decoded on the UE, and rendered locally with free navigation option.

The type of animated 3DGS scenes is not limited and the previous list is not exhaustive.

3. Proposal

It is proposed to agree the following changes to the draft 3GPP TR 26.958.

* * * First Change * * * *

# 5	Use cases

[Editor’s note: Placeholder for the description of the use cases]

## 5.4	Playback of dynamic 3DGS content

### 5.4.1	Description

A UE receives time-varying 3DGS content depicting a dynamic subject (e.g., a performer, dancer, singer, exhibition moment, band, sport action …). The UE renders the pre-recorded sequence in real time. The user is offered the ability to adjust the viewpoint locally within a constrained navigation volume while the subject itself evolves over time. This is analogous to volumetric video streaming, except the rendering primitive is 3D Gaussian splats rather than textured meshes or voxels.

Delivery is limited to on-demand streaming or file download of pre-recorded sequences.

Capture processes and real-time communication are out of scope. This use-case is focused on the delivery, decoding, and real-time rendering of pre-recorded dynamic 3DGS sequences.

This use case aligns with 3GPP TR 26.928 [aa], specifically Use Case 3: Streaming of Immersive 6DoF (non-live/on-demand variant).

### 5.4.2	Working assumptions

This section outlines the end-to-end processing chain covering playback of dynamic 3DGS content, emphasising adaptive delivery and device capability requirements.

-	Acquisition and content generation

-	The capture and the generation of dynamics 3DGS models are not addressed in this use case.

-	Compression and packaging

-	Sequence serialization with time indices

- 	The 3DGS sequence is compressed to meet the service and bandwidth constraints in term of bitrate.

[Editor’s note: workflow is expected to be documented because it has different uplink/downlink traffic and latency profiles.]

[Editor’s note: characterize which Gaussian parameters need to be signalled (position, scale, orientation, color, spherical harmonics, opacity, etc.) and what level of precision and number of gaussians is required for acceptable quality meeting the EU's performance capabilities. This directly impacts file size and bitrate]

[Editor’s note: To be considered whether existing 3GPP media delivery frameworks (e.g. MMS, messaging, file transfer) can carry a static 3DGS models without new protocol work, or whether new signalling is needed]

-	Transport and delivery

-	On-demand streaming and file delivery is used to transmit the 3DGS compressed data.

-	Decoding and decompression

-	The UE parses the dynamic scene, fetches/decompresses 3DGS data, and manages GPU residency for visible gaussian splats.

-	Rendering

-	Real-time splat-based renderer.

-	Navigation is constrained to the allowed-view volume derived from the capture information.

-	Navigation may be further constrained to collision detection with 3DGS objects in the scene. For example, by analysing the local splats density or using objects bounding boxes, or with a pre-defined authorized navigation area.

[Editor’s note: how “allowed navigation volume” is expressed to the receiver for safety, privacy, and quality reasons]

* * * End of Changes * * * *