3GPP TSG-SA4 Meeting #134	S4-251818

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0008 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED]: Solution for KI1 for collecting and reporting an Energy of Service Metric to authorized 3rd parties |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital France R&D, SAS |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addtion of a solution to Key Issue 1 on Energy-related Collection and Reporting to the 5G media |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Consider the comments during the SA4-SA4-e (AH) MBS SWG post 133-e (2025-10-23 - Online) - Consider the BBC comments - Propose architecture mappings in clause 7.5.2.2.1 and 7.5.2.2.2 - Propose procedures in clause 7.5.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No collection and reporting mechanisms for Energy-related information |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S4aI250197 |  |  |  |  |  |  |  |  |

1ST Change

…

[88]	3GPP TS 26.510: "Media delivery; interactions and APIs for provisioning and media session handling".

[89]	3GPP TS 26.531: “Data Collection and Reporting”

2ND Change

## 7.5	Solution #10: Energy-related metrics collection and reporting

### 7.5.1	Key Issue mapping

This Candidate Solution addresses Key Issue #1.

### 7.5.2	Functional description

#### 7.5.2.1	Introduction

In line with [PR 1-1] of TR 26.942, this Candidate Solution to Key Issue #1intends to reuse existing and extend 3GPP mechanisms for reporting an Energy-Related metrics as illustrated in the Figure 1 for 5GMS System.

Similar to QoE metrics, the new Energy-of-Service metric (i.e. EoS) is defined in the context of the Metrics Reporting provisioning feature at reference point M1 (see [88]). The Metrics Reporting Configurations provided by the Media Application Provider determines what energy-related information is to be collected and reported (to the instances listed) by the UE, Application Server and the Core Network, and how often.

Figure 7.5.2.1-1 : Architecture for collection and reporting of Metrics in the 5G Media System

The reference points as defined as follows are involved in the Metrics Reporting:

M1	Provisioning of Metrics Reporting configuration in Media AF.

M5	Configuration of Media Client Metrics reporting via the Media Session Handler and Media AF.and collection of Metrics Reports

M3	Configuration of Media AS Metrics reporting by Media AF.and collection of Metrics Reports

#### 7.5.2.2	Metrics Reporting Configuration: extensions for Energy-of-media Service (EoS)

Up to and including Release 19, the Metrics Reporting Configuration resource is limited to Quality of Experience (QoE) metrics. The present clause specifies extensions enabling Energy-related reporting (EoS metrics) in addition to QoE in table 7.5.2.2-1. Consequently, the scope of the Metrics Reporting Configuration is extended to cover either QoE metrics reporting or EoS metrics reporting.

Table 7.5.2.2-1: EoS extensions of the Metrics Reporting Configuration

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |
| Metrics scheme | Three additional EoS schemes are defined: - carbon intensity; for which metrics are reported in g CO2-e / Wh - energy consumption for which metrics are reported in Wh - power consumption, for which metrics are reported in. W. The Media Application Provider may provision any number of the above metrics schemes, each with the separate set of Metrics Reporting Configuration parameters. All three schemes indicate that the metrics to be collected and reported pertain to energy-related information reporting and determine the format of the Energy-Related metrics report submitted by the Media Session Handler for the UE energy information, by the EIF for the Core Network energy information and by the Media AS for the Application Server energy information.(see clause 7.5.2.2.1 and 7.5.2.2.2 for architecture mapping proposal) |
| Reporting scope | Scope of EoS metrics to be collected and reported: - For each media delivery session, the sum of the energy metric value of all the components of the session during the sampling period. - For each media delivery session, the energy metric value of all the components of the session or only the ones of content type given by the element “component content type” below - aggregated energy metric value across all the active sample media delivery sessions |
| Component content types filter | MIME content types (e.g., video/mp4) used to filter components included in per‑component reporting. |
| Contribution ratio flag | If true, include contribution ratio information across UE, RAN, and CN in the report. |
| Metrics to be reported | A controlled vocabulary of EoS energy metrics to be reported. All three EoS schemes indicated by Metrics scheme share the same controlled vocabulary of metrics. |
| NOTE: When the metrics scheme of the Metrics Reporting Configuration indicates a QoE metrics scheme, behaviour remains as specified in Release 19; the EoS specific constraints above do not apply. |  |

#### 7.5.2.2	Architecture mapping

The solutions proposes two approaches for the EoS Metrics Reporting.

#### 7.5.2.2.1	Approach A

The first approach combines the Metrics reporting architecture of the Release 19 and a part of the architecture for collection and exposure of energy (see clause 7.6.2.2).

Figure 7.5.2.2.1-1 illustrates how the approach A could be instantiated

Figure 7.5.2.3.1-1: Metrics reporting feature in the Release 19 Metrics Reporting architecture updated with the generic reference architecture for collection and exposure of energy-related information (clause 7.6.2.2)

The following reference points are involved in the proposed approach A EoS Metrics Reporting:

M1	Network API used by the Media Application Provider to provision the Energy-of-Service Metrics Reporting Configuration to the Media AF. This determines whether and which energy-related information submitted by the Media Session Handler for the UE, the Media AS and the EIF for the Core Network. The Energy Information Media AF collects the Energy-related information and creates the Energy related information reports to be exposed to the Media Entry Point URLs or by default the Media Application Provider.

E12	This reference point is used to get the Energy-related information from the EIF network function. The Energy Information Media AF receives the information and creates the report aligned with the Energy of Service scheme of the configuration.

M3	After configuration of the Metrics Reporting by the Media AF and the establishment of the Streaming session, the Media AS obtains the Metrics reporting configuration embedded in the Service Access Information, fetches the Energy of Service collected Metrics, creates the Metrics report aligned with the Energy of Service scheme of the configuration and uses this reference point to transmit the report to the Media AF and subsequently, the Energy information Media AF.

M5	Network API used by the Media Session Handler to obtain an Energy-of-Service Metrics Reporting configuration of the Media AF. The configuration information is embedded in Service Access Information. The Energy Information Collector instantiated in the Media Session Handler fetches the Energy of Service collected Metrics, creates the Metrics report aligned with the Energy of Service scheme of the configuration and uses the reference point to transmit the report to the Media AF and subsequently, to the Energy information Media AF.

E5	Network API used by the Energy Information Collector in the UE to subscribe to and receive Network Energy Information from the Energy Information Media AF. Network Energy Information exposed to the Energy Information Collector relates to a specific Application Service Provider. In particular, this API allows the UE to receive the reports aligned with the Energy of Service scheme of the configuration from the Energy Information AF and propagate to the Media-aware application (thru E6) and optionally, to the Media Application Provider(thru E8).

E4	Network API used by the Data Collection in Media AF to subscribe to and receive Network Energy Information from the Energy Information Media AF. Network Energy Information exposed to the Data collection relates to a specific Application Service Provider. In particular, this API allows the Data collection to receive the reports aligned with the Energy of Service scheme of the configuration from the Energy Information Media AF and expose them to event consumers.

#### 7.5.2.2.2	Approach B

The approach B is based on the architecture for collection and exposure of energy (see clause 7.6.2.2) for Energy of Service Metrics reporting.

Figure 7.5.2.2.2-1 illustrates how the approach B could be instantiated

Figure 7.5.2.2.2-1: Metrics reporting feature with the overall generic reference architecture for collection and exposure of energy-related information (clause 7.6.2.2)

The following reference points are involved in the proposed EoS Metrics Reporting approach B:

M1	Network API used by the Media Application Provider to provision the Energy-of-Service Metrics Reporting Configuration to the Media AF. This determines whether and which energy-related information submitted by the Energy information Collector for the UE, the Media AS and the EIF for the Core Network. The Energy Information Media AF collects the Energy-related information and creates the Energy related information reports to be exposed to the Media Entry Point URLs or by default the Media Application Provider.

E12	This reference point is used to get the Energy-related information from the EIF network function. The Energy Information Media AF receives the information and creates the report aligned with the Energy of Service scheme of the configuration.

E3	After configuration of the Metrics Reporting by the AF and the establishment of the Streaming session, the Energy information AF provides the Metrics reporting configuration embedded in the Service Access Information to the Media AS which will fetch the Energy of Service collected Metrics, will create the Metrics report aligned with the Energy of Service scheme of the configuration and will use this reference point to transmit the report to the Energy Information Media AF.

E5	Network API used by the UE Energy Information Collector instantiated in the Media Session Handler to obtain an Energy-of-Service Metrics Reporting configuration from the Energy Information AF. The configuration information is embedded in Service Access Information. The Energy Information Collector fetches the Energy of Service collected Metrics, creates the Metrics report aligned with the Energy of Service scheme of the configuration and uses the reference point to transmit the report to the Energy Information AF. This API also allows the UE Energy Information Collector to receive the aggregated reports aligned with the Energy of Service scheme of the configuration from the Energy Information AF and propagate to the Media-aware application (thru E6) and optionally, to the Media Application Provider(thru E8).

E4	Network API used by the Data Collection in AF to subscribe to and receive Network Energy Information from the Energy Information AF. Network Energy Information exposed to the Data collection relates to a specific Application Service Provider. In particular, this API allows the Data collection to receive the aggregated reports aligned with the Energy of Service scheme of the configuration from the Energy Information AF and expose them to event consumers..

### 7.5.3	Procedures

Figure 7.5.3-1 below details the different steps for Energy-of-Service Metrics Reporting for the two approaches A and B  in the system outlined in clause 7.5.2.2.1 or 7.5.2.2.2:

Figure 7.5.3-1: Procedures for Energy-of Service collection and reporting

1:	The AF is provisioned with the Energy of Service metrics reporting configuration by the Media Application Provider, each pertaining to metrics collection by the Media Player and the EIF and the Media AS functions. The configuration is transmitted to the Energy Information AF to collect and generate the metrics reports regarding required post-processing functionality and deliver to the Media Application Provider. The Energy Information AF subscribe to the EIF network function to get the Energy-related information from the Core Network.

2:	The Media-Aware Application triggers the Service Announcement and Content Discovery procedure. The Service Announcement includes the whole Service Access Information that contains metric configuration information for Energy of Service scheme

3:	Time passes until the UE initiates session establishment and media playback.

4:	The Media-Aware Application informs the Media Player of impending media playback.

5a:	The Media Player requests the establishment of a streaming session with the Media Session Handler which acknowledges the request.

5b:	The Media Session Handler requests the establishment of a streaming session with the Media AF which confirms the streaming session establishment.

5c:	The Media Session Handler informs the Media Player of the successful set-up of the streaming session.

6:	Media playback pipeline is set up between the Media Player, the Media AS and the Application Provider.

Begin approach A:

7:	The Service Access Information is retrieved by the Media AS to get the Energy of Service Metrics Reporting configuration.

8:	The Service Access Information is retrieved by the Media Session Handler to get the Energy of Service Metrics Reporting configuration.

9:	The Media Session Handler queries the Media Player on its capability to perform metrics collection (measurement and logging function) in accordance with Energy of Service scheme as defined by its Metrics Reporting configuration. At this step it is checked if the Media player can support the reporting of Energy of Service metric.

10:	The Media Player acknowledges its support for the collection of the required metric of the configured scheme.

During the course of media playback, steps 11-23 below may be repeated, depending on the duration of the playback and the frequency of metrics reporting as set by the metrics configuration for the Energy of Service scheme.

11:	Assuming a live streaming service, media content is delivered via push ingest from the Media Application Provider to the Media AS,

12:	Causing the Media AS to initialize and begin countdown of Energy of Service metrics reporting interval timer for Energy of Service scheme.

13:	The Media Player fetches media content from the Media AS and begins media playback.

14:	The Media Player notifies the Media Session Handler of the start of media playback, causing the Media Session Handler to initialize and begin countdown of separate metrics reporting interval timers for the Energy of Service scheme.

15:	Upon expiration of the timer (associated with the Energy of Service scheme), the Media AS retrieves the logged metrics measurements according to scheme

16:	The Media AS reports the logged metrics measurements to the Media AF and subsequently to the Energy Information AF sub-entity.

17:	The EIF network function of the Core Network publishes the Energy related Information to the Energy Information AF sub-entity.

18:	The Energy Information AF processes the information according to the Energy of Service scheme

19-20:	Upon expiration of the timer (associated with the Energy of Service scheme), the Media Session Handler retrieves the logged metrics measurements from the Media Player according to Energy of Service scheme.

21:	In accordance with its metrics reporting configuration as provisioned in step 1, a metrics report for Energy of Service scheme is sent from the Media Session Handler to the AF and subsequently to the Energy Information AF sub-entity.

22:	In accordance with its metrics reporting configuration as provisioned in step 1, the Media AF performs the aggregation of the logged metrics measurements received from the Media Session Handler, the Media AS and the EIF network function and perform post-processing in accordance with the Energy of Service scheme(e.g. filtering, aggregation, reformatting, privacy)

23:	The Energy Information AF publishes the final Energy of Service Metric reports to the Media Application Provider.

24:	Optionally, the Energy Information AF publishes the final Energy of Service Metric reports to the UE Energy Information Collector thru E2 reference point to allow the UE to report to the App.

25:	Optionally, the Energy Information AF publishes the final Energy of Service Metric reports to the AF Data Collection thru E4 reference point to allow the existing Data Collection function to expose the Energy of Service information as it is in the state of the art.

End approach A:

Begin Approach B:

7:	The Service Access Information is retrieved by the Media Session Handler to get the Energy of Service Metrics Reporting configuration.

8:	The Media AF set the Energy of Service Metrics Reporting configuration to the Energy Information AF.

9:	The Energy Information AF set the Energy of Service Metrics Reporting configuration to the Media AS thru the E3 reference point.

10: The Media Session Handler set the Energy of Service Metrics Reporting configuration to the Energy Information Collector

11: The Energy Information Collector queries the Media Player on its capability to perform metrics collection (measurement and logging function) in accordance with Energy of Service scheme as defined by its Metrics Reporting configuration. At this step it is checked if the Media player can support the reporting of Energy of Service metric.

12:	The Media Player acknowledges its support for the collection of the required metric of the configured scheme.

During the course of media playback, steps 13-23 below may be repeated, depending on the duration of the playback and the frequency of metrics reporting as set by the metrics configuration for the Energy of Service scheme.

13:	Assuming a live streaming service, media content is delivered via push ingest from the Media Application Provider to the Media AS,

14:	Causing the Media AS to initialize and begin countdown of Energy of Service metrics reporting interval timer for Energy of Service scheme.

15:	The Media Player fetches media content from the AS and begins media playback.

16:	The Media Player notifies the Media Session Handler of the start of media playback, causing the Media Session Handler to initialize and begin countdown of separate metrics reporting interval timers for the Energy of Service scheme.

17:	Upon expiration of the timer (associated with the Energy of Service scheme), the Media AS retrieves the logged metrics measurements according to scheme

18:	The AS reports the logged metrics measurements to the Energy Information AF.

19:	The EIF network function of the Core Network publishes the Energy related Information to the Energy Information AF sub-entity.

20:	The Energy Information AF processes the information according to the Energy of Service scheme

21-22:	Upon expiration of the timer (associated with the Energy of Service scheme), the UE Energy Information Collector  retrieves the logged metrics measurements from the Media Player according to Energy of Service scheme.

23:	In accordance with its metrics reporting configuration as provisioned in step 1, a metrics report for Energy of Service scheme is sent from the UE Energy Information Collector to the Energy Information AF.

24:	In accordance with its metrics reporting configuration as provisioned in step 1, the Energy Information AF performs the aggregation of the logged metrics measurements received from the UE Energy Information Collector, the Media AS and the EIF network function and perform post-processing in accordance with the Energy of Service scheme(e.g. filtering, aggregation, reformatting, privacy)

25:	The Energy Information AF publishes the final Energy of Service Metric reports to the Media Application Provider.

26:	Optionally, the Energy Information AF publishes the final Energy of Service Metric reports to the UE Energy Information Collector thru E5 reference point to allow the UE to report to the App.

27:	Optionally, the Energy Information AF publishes the final Energy of Service Metric reports to the AF Data Collection thru E4 reference point to allow the existing Data Collection function to expose the Energy of Service information as it is in the state of the art.

End approach B:

Upon the termination of media playback (as notified by the Media-Aware Application to the Media Player) a final round of metrics collection and reporting is performed:

### 7.5.4	Impacts on existing services, entities and interfaces

#### 7.5.4.1	Media AF

The Media AF would be required to support an Energy of Service Metrics Reporting Configuration as part of the Metrics Reporting Configuration.

#### 7.5.4.2	Energy Information AF

The Energy Information AF is instantiated as a subfunction of the Media AF to collect energy-related information per application, per media session, and per media component, at multiple granularities and across UE, CN, and AS domains at reference points M3, M5 and E12 for approach A and at reference points E3, E5 and E12 for Approach B.

Collected information can be reported to Media Entry Point URLs specified in the Metrics Reporting Configuration.

Collected information can be reported to the UE Energy Information Collector instantiated within the Media Session Handler to be propagated to the Media Aware Application at the reference point E6 and optionally to the Media Application Provider at reference point E8.

Collected information can be reported to a Data Collection AF instantiated within the 5GMS AF at the reference point E4 prior to exposure to authorized third parties, privacy-preserving post-processing being applied.

#### 7.5.4.3	Reference point M1

At reference point M1, the Media Application Provider shall instantiate a Metrics Reporting Configuration resource associated with the target Provisioning Session to enable Energy-of-Service-Service collection and reporting

End of changes