# 3GPP TSG-SA WG4 Meeting #134																  S4-251836

Dallas, USA, 17 – 21 November, 2025											revision of S4-251315

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR | 0588 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to IVAS SDP parameters and RTCP APP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IVAS_Codec |  |  |  |  |  | Date: |  |  | 2025-11-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | IVAS SDP parameters have been updated |  |  |  |  |  |  |  |  |
| Summary of change: |  | SDP parameters for IVAS are renamed and new are added to align with the updated RTP payload format in TS 26.253. RTCP APP requests are also updated and added, including SDP parameters for negotiation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk for incompatible implementations resulting in fallback to a non-immersive codec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.1, 6.2.2.2, 10.2.1.12, 10.2.1.14 (new), 10.2.1.15 (new), 10.2.3, Annex M.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 26.253 CR 0016 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depends on TS 26.253 CR 0016. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Re-submission of endorsed CR Rev 2: Added updates the clauses 5.2.1.1, 10.2.1.12, 10.2.1.14, 10.2.1.15, 10.2.3 and Annex M.4 |  |  |  |  |  |  |  |  |

First change

#### 5.2.1.1	General codec requirements

MTSI clients in terminals offering speech communication shall support narrowband, wideband and super-wideband communication and should support immersive audio communication. The only exception to this requirement is for the MTSI client in constrained terminal offering speech communication, in which case the MTSI client in constrained terminal shall support narrowband and wideband, and should support super-wideband communication.

NOTE:	MTSI clients in terminals refers to the definition in clause 3.1.

In addition, MTSI clients in terminals offering speech communication shall support:

-	AMR speech codec (TS 26.071 [11], TS 26.090 [12], TS 26.073 [13] and TS 26.104 [14]) including all 8 modes and source controlled rate operation ‎TS 26.093 [15]. The MTSI client in terminal shall be capable of operating with any subset of these 8 codec modes. More detailed codec requirements for the AMR codec are defined in clause 5.2.1.2.

MTSI clients in terminals offering wideband speech communication at 16 kHz sampling frequency shall support:

-	AMR-WB codec (TS 26.171 ‎‎[17], TS 26.190 ‎[18], TS 26.173 ‎[19] and TS 26.204 [20]) including all 9 modes and source controlled rate operation ‎TS 26.193 [21]. The MTSI client in terminal shall be capable of operating with any subset of these 9 codec modes. More detailed codec requirements for the AMR-WB codec are defined in clause 5.2.1.3. When the EVS codec is supported, the EVS AMR-WB IO mode may serve as an alternative implementation of AMR-WB as defined in clause 5.2.1.4.

MTSI clients in terminals offering super-wideband or fullband speech communication shall support:

-	EVS codec ( TS 26.441 [121], TS 26.444 [124], TS 26.445 [125], TS 26.447 [127], TS 26.451 [131], TS 26.442 [122], TS 26.452 [165] and TS 26.443 [123]) as described below including functions for backwards compatibility with AMR-WB ( TS 26.446 [126]) and discontinuous transmission ( TS 26.449 [129] and TS 26.450 [130]). More detailed codec requirements for the EVS codec are defined in clause 5.2.1.4.

MTSI clients in terminals offering immersive audio communication:

-	shall support IVAS codec (TS 26.250 [186], TS 26.252 [187], TS 26.253 [188], TS 26.254 [189], TS 26.255 [190], TS 26.256 [191] and TS 26.258 [192]) as described below, including functions for backwards compatibility with EVS and AMR-WB interoperable mode as described above. More detailed codec requirements for the IVAS codec are defined in clause 5.2.1.7;

-	 may support dual-mono based on super-wideband or fullband speech communication.

Encoding of DTMF is described in Annex G.

Next change

#### 6.2.2.2	Generating SDP offers

When speech is offered, an MTSI client in terminal sending a first SDP offer in the initial offer-answer negotiation shall include at least one RTP payload type for AMR-NB according to RFC4867 [28] and the MTSI client in terminal shall support and offer a configuration, where the MTSI client in terminal includes the parameter settings as defined in Table 6.1. When EVS-NB is also offered, the MTSI client in terminal shall support and offer a configuration, where the MTSI client in terminal includes the parameter settings for EVS (both EVS Primary and AMR-WB IO modes) as defined in Table 6.2a.

If wideband speech is also offered, then the SDP offer shall also include at least one RTP payload type for AMR-WB according to RFC4867 [28] and the MTSI client in terminal shall support and offer a configuration, where the MTSI client in terminal includes the parameter settings as defined in Table 6.1. When EVS-WB is also offered, the MTSI client in terminal shall support and offer a configuration, where the MTSI client in terminal includes the parameter settings for EVS (both Primary and AMR-WB IO modes) as defined in Table 6.2a. AMR-WB and EVS (including the EVS AMR-WB IO mode) are thus offered using different RTP payload types.

If super-wideband speech is also offered, the SDP offer shall include at least one RTP payload type for EVS and the MTSI client in terminal shall support a configuration where the MTSI client in terminal includes the parameter settings as defined in Table 6.2a.

If fullband speech is also offered, the SDP offer shall include at least one RTP payload type for EVS and the MTSI client in terminal shall support a configuration where the MTSI client in terminal includes the parameter settings as defined in Table 6.2a.

When EVS is offered, the RTP payload type for EVS shall also use parameters for EVS AMR-WB IO mode as defined in Table 6.2a, except for the ‘ecn-capable-rtp’ and ‘leap ect’ parameters. AMR-WB and EVS (including the EVS AMR-WB IO mode) are thus offered using different RTP payload types.

NOTE 1:	RFC4867 can also be used for EVS AMR-WB IO when EVS is supported. This may happen after SRVCC when the EVS payload format is used between the ATGW and the MTSI client in terminal while RFC4867 is used between the CS-MGW and the ATGW.

NOTE 2:	ECN-triggered adaptation is currently undefined for EVS. This does not prevent ECN-triggered adaptation from being negotiated and used for AMR or AMR-WB.

NOTE 3:	When EVS is offered, the audio bandwidths may be different for different directions for the EVS Primary mode, even for ‘sendrecv’ media.

When IVAS is offered, the RTP payload type shall include parameters for immersive audio, EVS Primary mode and EVS AMR-WB IO mode as defined in Table 6.2b.

Clause 5.2.1.6 describes the preference order for how different configurations should be ordered in the list of payload type numbers that is given on the m= line.

Table 6.1: SDP parameters for AMR-NB or AMR-WB, when the MTSI client in terminal offers the bandwidth-efficient payload format

| Parameter | Usage |
| --- | --- |
| octet-align | Shall not be included |
| mode-set | Shall not be included |
| mode-change-period | Shall not be included |
| mode-change-capability | Shall be set to 2 |
| mode-change-neighbor | Shall not be included |
| maxptime | Shall be set to 240, see also Table 7.1 |
| crc | Shall not be included |
| robust-sorting | Shall not be included |
| interleaving | Shall not be included |
| ptime | Shall be set according to Table 7.1 |
| channels | Shall either be set to 1 or be omitted |
| max-red | Shall be included and shall be set to 220 or less |
| ecn-capable-rtp: leap ect=0 | Shall be included if offering to use ECN and if the session setup allows for bit-rate adaptation |

Table 6.2: SDP parameters for AMR-NB or AMR-WB, when the MTSI client in terminal offers the octet-aligned payload format

| Parameter | Usage |
| --- | --- |
| octet-align | Shall be set to 1 |
| mode-set | Shall not be included |
| mode-change-period | Shall not be included |
| mode-change-capability | Shall be set to 2 |
| mode-change-neighbor | Shall not be included |
| maxptime | Shall be set to 240, see also Table 7.1 |
| crc | Shall not be included |
| robust-sorting | Shall not be included |
| interleaving | Shall not be included |
| ptime | Shall be set according to Table 7.1 |
| channels | Shall either be set to 1 or be omitted |
| max-red | Shall be included and shall be set to 220 or less |
| ecn-capable-rtp: leap ect=0 | Shall be included if offering to use ECN and if the session setup allows for bit-rate adaptation |

Table 6.2a: SDP parameters for EVS (both Primary and AMR-WB IO modes, when the MTSI client in terminal offers EVS

| Parameter | Usage |
| --- | --- |
| ptime | Shall be set according to Table 7.1 |
| maxptime | Shall be set to 240, see also Table 7.1 |
| evs-mode-switch | MTSI client in terminal shall not include evs-mode-switch in the initial SDP offer. |
| hf-only | The SDP offer-answer considerations in TS 26.445 [125] apply. |
| dtx | MTSI client in terminal shall not include dtx in the initial SDP offer. |
| dtx-recv | MTSI client in terminal shall not include dtx-recv. |
| max-red | Shall be included and shall be set to 220 or less. |
| channels | The SDP offer-answer considerations in TS 26.445 [125] apply. |
| cmr | The SDP offer-answer considerations in TS 26.445 [125] apply. |
| br | An MTSI client in terminal supporting the EVS codec is required to support the entire bit-rate range but may offer a smaller bit-rate range or even a single bit-rate. |
| br-send | The SDP offer-answer considerations in TS 26.445 [125] apply. |
| br-recv | The SDP offer-answer considerations in TS 26.445 [125] apply. |
| bw | The SDP offer-answer considerations in TS 26.445 [125] apply. |
| bw-send | The SDP offer-answer considerations in TS 26.445 [125] apply. |
| bw-recv | The SDP offer-answer considerations in TS 26.445 [125] apply. |
| ch-send | The SDP offer-answer considerations in TS 26.445 [125] apply. |
| ch-recv | The SDP offer-answer considerations in TS 26.445 [125] apply. |
| ch-aw-recv | The SDP offer-answer considerations in TS 26.445 [125] apply. |
| mode-set | Shall not be included |
| mode-change-period | Shall not be included |
| mode-change-capability | The SDP offer-answer considerations in TS 26.445 [125] apply. |
| mode-change-neighbor | Shall not be included |

Table 6.2b: SDP parameters for IVAS (including EVS Primary and EVS AMR-WB IO modes)

| Parameter | Usage |
| --- | --- |
| ptime | Shall be set according to Table 7.1 |
| maxptime | Shall be set to 240, see also Table 7.1 |
| evs-mode-switch | Shall be set according to Table 6.2a |
| hf-only | If present, it shall be set to 1 |
| dtx | Shall be set according to Table 6.2a |
| dtx-send |  |
| dtx-recv | Shall be set according to Table 6.2a |
| max-red | Shall be included and shall be set to 220 or less. |
| channels | Shall not be present |
| cmr | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| br | Shall be set according to Table 6.2a |
| br-send | Shall be set according to Table 6.2a |
| br-recv | Shall be set according to Table 6.2a |
| bw | Shall be set according to Table 6.2a |
| bw-send | Shall be set according to Table 6.2a |
| bw-recv | Shall be set according to Table 6.2a |
| ch-send | Shall not be included |
| ch-recv | Shall not be included |
| ch-aw-recv | Shall be set according to Table 6.2a |
| mode-set | Shall be set according to Table 6.2a |
| mode-change-period | Shall be set according to Table 6.2a |
| mode-change-capability | Shall be set according to Table 6.2a |
| mode-change-neighbor | Shall be set according to Table 6.2a |
| mono-init | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| mono-init-send | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| mono-init-recv | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| ibr | An MTSI client in terminal supporting the IVAS codec is required to support the entire bit-rate range but may offer a smaller bit-rate range or even a single bit-rate. |
| ibr-send | The SDP offer-answer considerations for ibr and in TS 26.253 [188] apply. |
| ibr-recv | The SDP offer-answer considerations for ibr in TS 26.253 [188] apply. |
| ibw | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| ibw-send | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| ibw-recv | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| cf | Shall not be included. |
| cf-send | Shall be included and contain all coded formats the offerer is supporting and prepared to send. |
| cf-recv | Shall contain all formats of Table A.4.1-1 of [188] or not be included. |
| cf-sub-info | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| ivas-icm | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| ivas-icm-send | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| ivas-icm-recv | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| ns-mode-init | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| ns-mode-init-send | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| ns-mode-init-recv | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| pi-types | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| pi-types-send | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| pi-types-recv | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| pi-br | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| pi-br-send | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| pi-br-recv | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| sr-dof | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| sr-tc | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| sr-tc-fr | The SDP offer-answer considerations in TS 26.253 [188] apply. |
| sr-tc-cp | The SDP offer-answer considerations in TS 26.253 [188] apply. |

When the channels parameter is omitted then this means that one channel is being offered.

The mode-set parameter is omitted, allowing maximum freedom for the visited network.

The mode-change-capability parameter is included and set to 2 for AMR-NB and AMR-WB, to support potential interworking with 2G radio access (GERAN). For EVS AMR-WB IO it is not required to include the mode-change-capability parameter.

An example of an SDP offer for AMR-NB is shown in Table A.1.1. An example of an SDP offer for both AMR-NB and AMR-WB is shown in Table A.1.2. An example of SDP offer for AMR-NB, AMR-WB, and EVS is shown in Table A.14.1.

An SDP example for offering and accepting a dual-mono session for EVS is shown in clauses A.14.1 and A.14.3.

SDP examples for IVAS are shown in clause A.19.

An MTSI client in terminal may divide the offer-answer negotiation into several phases and offer different configurations in different SDP offers. If this is done then the first SDP offer in the initial offer-answer negotiation shall include the most preferable configurations. For AMR-NB, this means that the first SDP offer in the initial offer-answer negotiation shall include at least one RTP payload type for AMR-NB with the parameters as defined in Table 6.1. If wideband speech is offered then the first SDP offer in the initial offer-answer negotiation shall include also at least one RTP payload type for AMR-WB with the parameters as defined in Table 6.1. This also means that offers for octet-aligned payload format do not need to be included in the first SDP offer. If super-wideband or fullband speech is offered, the first SDP offer in the initial offer-answer negotiation shall include at least one RTP payload type for EVS with the parameters as defined in [125]. One example of dividing the offer-answer negotiation into two phases, and the corresponding SDP offers, is shown in clause A.1.1.2.2.

NOTE 4:	Dividing the offer-answer negotiation into several phases may lead to never offering the less preferred configurations, if the other end-point accepts to use at least one of the configurations offered in the initial SDP offer.

If the speech media is re-negotiated during the session then the knowledge from earlier offer-answer negotiations should be used in order to shorten the session re-negotiation time. I.e., failed offer-answer transactions shall not be repeated.

Next change

#### 10.2.1.12	IVAS Coded Format Request

RTCP_APP_REQ_ICF: IVAS Coded Format Request

Figure 10.6g: IVAS Coded Format request

Codecs: This request can be used for the IVAS codec when operating in IVAS Immersive mode, EVS Primary mode or EVS AMR-WB IO mode.

The DATA field is a 4-bit field and is encoded as described in the table below:

Table 10.1.c Encoding of the DATA field in the IVAS Coded Format Request.

| Data | Identifier | Coded Format request | Subclause in TS 26.253 [188] |
| --- | --- | --- | --- |
| 0000 | Stereo | Stereo Operation | 4.2.3 |
| 0001 | SBA | Scene-based Audio (SBA, Ambisonics) Operation | 4.2.4 |
| 0010 | MASA | Metadata-Assisted Spatial Audio (MASA) Operation | 4.2.5 |
| 0011 | ISM | Objects (Independent Streams with Metadata, ISM) Operation | 4.2.6 |
| 0100 | MC | Multi-Channel (MC) Operation | 4.2.7 |
| 0101 | OMASA | Combined Objects and MASA (OMASA) Operation | 4.2.9 |
| 0110 | OSBA | Combined Objects and SBA (OSBA) Operation | 4.2.8 |
| 0111 | NO_REQ |  |  |
| 1000 | [LCLD] |  |  |
| 1001 | [LC3plus] |  |  |
| 1010 | Not used |  |  |
| 1011 | Not used |  |  |
| 1100 | Not used |  |  |
| 1101 | Not used |  |  |
| 1110 | Not used |  |  |
| 1111 | Not used |  |  |

The IVAS Coded Format request may be used for switching between different IVAS Immersive coded format and also requesting switching from IVAS EVS Primary mode or IVAS EVS AMR-WB IO mode to IVAS Immersive mode.

The coded format request shall comply with the media type parameters that are negotiated for the session.

A media receiver operating in IVAS EVS Primary mode or IVAS EVS AMR-WB IO mode and sending an IVAS Coded Format request should also include an IVAS Bitrate request.

NOTE: The IVAS Bitrate request may be replaced by an inband CMR.

A media sender operating in IVAS EVS Primary mode or IVAS EVS AMR-WB IO mode and receiving an IVAS Coded Format request without an IVAS Bitrate request (subclause 10.2.1.13) should apply the rules for Initial Codec Mode rules described in subclause 7.5.2.1.10.

Next change

#### 10.2.1.14	IVAS Coded Subformat Request

RTCP_APP_REQ_ICSF: IVAS Coded Subformat Request

Figure 10.6i: IVAS Coded Format request

Codecs: This request can be used for the IVAS codec when operating in IVAS Immersive mode, EVS Primary mode or EVS AMR-WB IO mode.

The subFMT field is a 6-bit field and is encoded as shown in TS 26.253 Table A.3.3.3.3.3.3-2.

The IVAS Coded Subformat request is used to switch to a specific subformat of an IVAS Coded Format. If the switching to the requested subformat would also result in switching to IVAS Immersive mode (i.e., from EVS Primary mode or from EVS AMR-WB IO mode) or to a different IVAS Coded Format then an IVAS Coded Format request shall also be included in the same RTCP APP packet.

The coded subformat request shall comply with the media type parameters that are negotiated for the session.

#### 10.2.1.15	IVAS Processing Information Data

RTCP_APP_REQ_IPID: IVAS Processing Information Data (IVAS PI Data)

Figure 10.6i: IVAS PI Data

Codecs: The IVAS PI Data can be used for the IVAS codec is operating in IVAS Immersive mode. The IVAS PI Data enables sending Processing Information (PI) data in RTCP APP packets as an alternative to sending PI data in the RTP payload, see TS 26.253 Annex A.3.5 [188].

The ‘Length’ field (8 bits) indicates the number of octets used for the PI data block and including the Time Stamp (if included). The ‘Length’ field is an unsigned integer from 0 to 255 and use MSB first encoding. The value 0 is not used since the minimum size of a PI data block is 1 octet, i.e., when the PI data block only includes NO_PI_DATA.

The ‘Time Stamp’ field (32 bits) is optional to include and may be used to synchronize PI data with an audio frame. The Time Stamp is the RTP Time Stamp that would have been included in the the RTP header if the PI data would have been sent in an RTP payload.

The ‘T’ bit is used to indicate if Time Stamp is included or not:

-	T = ‘0’: Time Stamp is not included,

-	T = ‘1’: Time Stamp is included.

The ‘R’ bits are reserved for future use. The ‘R’ bits shall be set to ‘0’ when sending the IVAS PI Data and shall be ignorded when receiving the IVAS PI Data.

The PI data block is a variable length field, contains one or more PI data and use the same encoding and packetization as described for PI data in the RTP payload, see TS 26.253 Annex A.3.5 [188].

The IVAS PI data may be sent in either forward direction (media sender to media receiver) and in reverse direction (media receiver to media sender) depending on the type of PI data being sent, see TS 26.253 Annex A.3.5 [188] for more information on different PI data types.

If the PI data block, including the Time Stamp (if included), exceeds 255 octets, then the PI data shall be distributed over multiple IVAS PI Data.

If different PI data needs synchronization with different audio frames, and thus needs different Time Stamps, then these PI data cannot be sent in the same IVAS PI data.

Several IVAS PI data may be included in the same RTCP APP packet.

The PI data shall comply with the media type parameters that are negotiated for the session.

Next change

### 10.2.3	SDP negotiation for RTCP-APP

RTCP-APP request messages that can be used are negotiated with SDP using the ‘3gpp_mtsi_app_adapt’ attribute. The syntax for the 3GPP MTSI RTCP-APP adaptation attribute is:

a=3gpp_mtsi_app_adapt:<reqNames>

where:

<reqNames> is a comma-separated list identifying the different request messages (see below).

The ABNF for the RTCP-APP adaptation messages negotiation attribute is the following:

adaptation attribute	= "a" "=" "3gpp_mtsi_app_adapt" ":" reqName *("," reqName)

reqName			= "RedReq" / "FrameAggReq" / "AmrCmr" / "EvsRateReq" / "EvsBandwidthReq" / "EvsParRedReq" / "EvsIoModeReq" / "EvsPrimaryModeReq" / "IvasCodedFrameReq" / "IvasImmersiveBitRateReq".

The name denotes the RTCP APP packet types the SDP sender supportes to receive. The meaning of the values is as follows:

RedReq: Redundancy Request, clause 10.2.1.3

FrameAggReq: Frame Aggregation Request, clause 10.2.1.4

AmrCmr: Codec Mode Request for AMR and AMR-WB, clause 10.2.1.5

EvsRateReq: EVS Primary Rate Request, clause 10.2.1.7

EvsBandwidthReq: EVS Bandwidth Request, clause 10.2.1.8

EvsParRedReq: EVS Partial Redundancy Request, clause 10.2.1.9

EvsIoModeReq: EVS Primary mode to EVS AMR-WB IO mode Switching Request, clause 10.2.1.10

EvsPrimaryModeReq: EVS AMR-WB IO mode to EVS Primary mode Switching Request, clause 10.2.1.11

IvasCodedFrameReq: IVAS Coded Format Request, clause 10.2.1.12

IvasImmersiveBitRateReq: IVAS Bit Rate Request, clause 10.2.1.13

-	IvasCodedSubformatReq: IVAS Coded Subformat Request, clause 10.2.1.14

-	IvasProcessingInformationData: IVAS Processing Information Data, clause 10.2.1.15

An MTSI client supporting the reception of any RTCP APP packets defined in the present specification shall indicate the supported RTCP APP packet types in an initial SDP offer or answer it sends using the SDP "a=3gpp_mtsi_app_adapt" attribute. If the answerer receives an "a=3gpp_mtsi_app_adapt" attribute in the SDP offer, it may send the indicated RTCP APP packet types towards the offerer. The answerer shall indicate its capabilties with the "a=3gpp_mtsi_app_adapt" attribute irrespective if an "a=3gpp_mtsi_app_adapt" attribute was received and the capabilities within. If the offerer receives an "a=3gpp_mtsi_app_adapt" attribute in the SDP answer, it may send the indicated RTCP APP packet types towards the answerer.

An MTSI client supporting only AMR and AMR-WB therefore may for instance include the following in the SDP offer:

a=3gpp_mtsi_app_adapt: RedReq,FrameAggReq,AmrCmr

An MTSI client supporting only AMR, AMR-WB and EVS may for instance include the following in the SDP offer:

a=3gpp_mtsi_app_adapt: RedReq,FrameAggReq,AmrCmr,EvsRateReq,EvsBandwidthReq,EvsParRedReq,EvsIoModeReq,EvsPrimaryModeReq

The attribute shall only be used on media level.

When interworking with pre-Rel-12 clients or non-MTSI clients, it may happen that they support the RTCP-APP signalling but not the SDP negotiation for AMR and AMR-WB. An MTSI client failing to negotiate RTCP-APP as described may still try to use the RTCP-APP signalling when requesting adaptation, but the MTSI client shall then also monitor the received media in order to determine if some or all of the adaptation requests included in the RTCP-APP were partially or fully followed or not followed at all. If none of the adaptation requests is followed, not even partially, then this is an indication that the remote client does not support the RTCP-APP signalling. The MTSI client should then try to use other means for triggering the adaptation, for example CMR in the AMR/AMR-WB payload or RTCP Sender Reports/Receiver Reports.

Next change

# M.4	3gpp_mtsi_app_adapt

Contact name, email address, and telephone number:

3GPP Specifications Manager

3gppContact@etsi.org

+33 (0)492944200

Attribute Name (as it will appear in SDP)

3gpp_mtsi_app_adapt

Long-form Attribute Name in English:

3GPP MTSI RTCP-APP Adaptation attribute

Type of Attribute

Media level

Is Attribute Value subject to the Charset Attribute?

This Attribute is not dependent on charset.

Purpose of the attribute:

This attribute is used to negotiate which RTCP-APP request messages that can be used in a session.

Appropriate Attribute Values for this Attribute:

The attribute is a value attribute. The defined values are: "RedReq", "FrameAggReq", "AmrCmr", "EvsRateReq", "EvsBandwidthReq", "EvsParRedReq", "EvsIoModeReq", "EvsPrimaryModeReq", "IvasCodedFrameReq", "IvasImmersiveBitRateReq", ‘IvasCodedSubformatReq’, ‘IvasProcessingInformationData’.

MUX Category for this Attribute:

IDENTICAL-PER-PT

End changes