3GPP TSG-SA4 Meeting #134	S4-251860

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.261 | CR | 0001 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding performance requirements for spatial directivity tests of SBA and MASA in send direction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dolby Laboratories Inc., Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATIAS_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently spatial directivity tests of SBA and MASA in send direction have effectively no requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add requirements for SBA and MASA capture performance for single-source tests. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Compliance evaluations of the SBA and MASA capture feature of a UE offering immersive audio support in relation to requirements are impossible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.2, 5.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 26.260 CR 007 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

### 5.5.2	SBA

ϕi-ϕi

(θi-θi

#### 5.5.2.1	Applicability

Based on the specification of the physical test arrangement used for UE testing in sending according to clause 5.4.2 of TS 26.260, a UE may support half-circular planar, full-circular planar or 3D spherical SBA capture. Depending on this capability, either the requirements according to clauses 5.5.2.2, 5.5.2.3 or 5.5.2.4 apply.

Half-circular planar capture is defined herein as providing differentiation of source azimuth directions between -90° and 90° in the 0° elevation plane.

Full-circular planar capture is defined herein as providing differentiation of source azimuth directions between -180° and 180° in the 0° elevation plane.

[3D spherical capture is defined herein as an extension of Full-circular planar capture that additionally provides differentiation of sources a different elevation angles.]

Compliance shall be checked by the relevant tests described in TS 26.260.

#### 5.5.2.2	Half-circular planar capture

For UEs supporting half-circular planar capture, the following set of requirements exist.

Azimuth

For half-circular planar capture, the estimated azimuths ϕi for source directions i=1…L-1, where L=7, as listed in Table 5 of TS 26.260 clause 5.6.4.3.1, shall be monotonically increasing such that ϕi+1>ϕi.

The estimated azimuth for source direction 4, the 0° source direction, shall be well within the front half of the listening plane, such that -60°<ϕ4<60°

The estimated azimuths of source directions 1 and 7, the ±90° source directions, shall be well within the right and left halves of the listening plane, respectively, such that -150°<ϕ1<-30°and 30°<ϕ7<150°.

#### NOTE: The measured angle ϕ1 shall be unwrapped to ensure continuity in the neighbourhood of its corresponding source direction  ϕ1.

[Elevation

The estimated source elevations for the source directions listed in Table 5 of TS 26.260 clause 5.6.4.3.1 shall have an absolute value of θi≤30°.]

#### 5.5.2.3	Full-circular planar capture

For UEs supporting full-circular planar capture, the requirements according to clause 5.5.2.2 apply with the source directions i=1…L-1, where L=12, as listed in Tables 5 and 6 of TS 26.260 clauses 5.6.4.3.1 and 5.6.4.3.2. In addition, the requirement  ϕ1>ϕ12-360° applies.

Furthermore, the estimated azimuth of source direction i=10 shall be well within the rear half of the listening plane, such that 120°< ϕ10<240°.

#### NOTE: The measured angles ϕ1 and ϕ12 shall be unwrapped to ensure continuity in the neighbourhood of their corresponding source directions  ϕ1 and ϕ12, respectively.

#### 5.5.2.4	3D spherical capture

Compliance shall be checked by the relevant tests described in TS 26.260.

UEs supporting full SBA capture shall meet the requirements for full-circular planar capture according to clause 5.5.2.3.

In addition, the following requirements apply:

Azimuth

The requirements according to clause 5.5.2.2 apply with the source directions i=13…L-1, where L=20, as listed in Table 7 of TS 26.260 clauses 5.6.4.3.3. In addition, the requirement  ϕ13>ϕ20-360° applies.

The estimated azimuths of non-zero elevation source directions i=13,17, i.e. the ±90° azimuth directions, shall be within the right and left halves of the listening plane, respectively, such that -180°<ϕ13<0°and 0°<ϕ17<180°.

The estimated azimuths of non-zero elevation source directions i=15, 19, i.e. the 0° and 180° azimuth directions, shall be within the front and rear halves of the listening plane, respectively, such that -90°<ϕ15<90°and 90°<ϕ19<270°.

#### NOTE: The measured angles ϕ13 and ϕ20 shall be unwrapped to ensure continuity in the neighbourhood of their corresponding source directions  ϕ13 and ϕ20, respectively.

[Elevation

Source directions with equal non-zero elevations, i.e. i=13…20, shall have estimated elevations within a range of ±30° from their mean estimated elevation, such that θ -θi≤30°, where  θ=1Ni=1320θi.

The following objective pertains to source directions beyond the sets described in Tables 5-7 in TS 26.260, clause 5.6.4.3. It is included to test a UE capable of 3D spherical capture more fully, if additional source directions were to be included.

- Sets of source directions with the same azimuth should have monotonically increasing estimated elevation with respect to their actual elevation, such that for each pair of source directions i and j with equal actual azimuth ϕi=ϕj, if θj>θi then  θj>θi.]

==============Next change==============

### 5.5.3 MASA

ϕi-ϕi

(θi-θi

#### 5.5.3.1	Applicability

Based on the specification of the physical test arrangement used for UE testing in sending according to clause 5.4.2 of TS 26.260, a UE may support half-circular planar, full-circular planar or 3D spherical MASA capture. Depending on this capability, either the requirements according to clauses 5.5.2.2, 5.5.2.3 or 5.5.2.4 apply.

Half-circular planar capture is defined herein as providing differentiation of source azimuth directions between -90° and 90° in the 0° elevation plane.

Full-circular planar capture is defined herein as providing differentiation of source azimuth directions between -180° and 180° in the 0° elevation plane.

[3D spherical capture is defined herein as an extension of Full-circular planar capture that additionally provides differentiation of sources a different elevation angles.]

Compliance shall be checked by the relevant tests described in TS 26.260.

==============End of change==============