3GPP TSG-SA WG4 Meeting #134	S4-25 1868

Dallas, US, 17 – 21 November 2025

Source:	Fraunhofer IIS

Title:	Performance evaluation of ISM capturing solutions

Agenda item:	7.6

Document for:	Discussion and agreement

# 1. Introduction

While the evaluation of ISM capturing solutions is covered by the single-source test methods and recording scenario included in the current DaCAS-2 permanent document [1] and in TS 26.260 [2], multi-source evaluation suitable for ISM capturing example solutions has not been addressed so far. Initial discussion on multi-source evaluation has been triggered in [3] and [4], targeting SBA capturing solutions specifically. In this document, we propose agreement for additional recording scenarios and objective performance test methods for ISM capturing example solutions, focusing on multi-source scenarios.

# 2. Recording scenarios and procedure

To enable multi-source evaluation of ISM capturing solutions, we propose two additional recording scenarios to be included in the database, which are detailed in Table 1. The proposed recording scenarios target smartphone-type devices and should therefore be made for all available smartphone-type target devices.

- Teleconferencing scenario (X-1, Figure 1): Smartphone lying flat on a table, 2 speakers positioned along the long axis of the smartphone within a certain opening angle

- Video call scenario (X-2, Figure 2): Smartphone in hand-held mode, landscape orientation, 2 speakers facing the screen/front camera

The proposed recording scenarios are also attached as recording_scenarios.xlsx.

To ensure the availability of suitable reference signals for the evaluation of ISM capturing solutions, we propose that the following recording procedure is followed for multi-source recording scenarios targeting ISM capturing solutions:

- Record sound sources individually. These individual sound source recordings correspond to the reference signals used for the evaluation of ISM capturing solutions.

- Sum up the individual sound source recordings to obtain final input signals to example solutions

Table 1 Recording scenarios for multi-source ISM evaluation

| No | X-1 | X-2 |
| --- | --- | --- |
| Sound source | According to “Sound source” specified in Table 1 in section 4.1.2.1 of [1] | According to “Sound source” specified in Table 1 in section 4.1.2.1 of [1] |
| Source signal | British English single talk sequence according to the ITU-T P.501 | British English single talk sequence according to the ITU-T P.501 |
| Source signal characteristics | Language: English Gender: Male, Female Range: 20Hz-20kHz Length: 35.4 s Level: -27 dB RMS Sample Rate: 48 kHz Bit depth: 16 bit | Language: English Gender: Male, Female Range: 20Hz-20kHz Length: 35.4 s Level: -27 dB RMS Sample Rate: 48 kHz Bit depth: 16 bit |
| Sound source calibration | According to “Sound source calibration” specified in Table 1 in section 4.1.2.1 of [1] | According to “Sound source calibration” specified in Table 1 in section 4.1.2.1 of [1] |
| Acoustic environment | According to “Acoustic environment” specified in Table 1 in section 4.1.2.1 of [1] | According to “Acoustic environment” specified in Table 1 in section 4.1.2.1 of [1] |
| Detailed Positioning | UE positioning/orientation: UE lying flat on a table Source distance: 0.5-1 m Source direction: Loudspeaker diaphragm towards the UE Source height (relative to UE): 0.4 m Source angles: Azimuth angle combinations (2 sources active simultaneously): [-90°, 90°], [-110°, 70°], [-110°, 90°] | UE positioning/orientation: UE in hand-held mode, landscape orientation Source distance: 0.3-0.5 m Source direction: Loudspeaker diaphragm towards the UE Source height (relative to UE): 0 m Source angles: Azimuth angle combinations (2 sources active simultaneously): [-30°, 30°], [-45°, 45°], [-30°, 45°] |
| Reference recording | The reference recordings correspond to the individual sound source recordings obtained by following the outlined recording procedure. | The reference recordings correspond to the individual sound source recordings obtained by following the outlined recording procedure. |
| Description/additional info | Recording procedure - Record sound sources individually - Sum up the individual sound source recordings to obtain final input signals to example solutions Sources at different locations should use different sequences as the source signal. | Recording procedure - Record sound sources individually - Sum up the individual sound source recordings to obtain final input signals to example solutions Sources at different locations should use different sequences as the source signal. |

# Figure 1 Sketch of recording scenario X-1

Figure 2 Sketch of recording scenario X-2

# 3. Objective performance evaluation

Considering ISM capturing example solutions in multi-source scenarios, a number of requirements can be identified (assuming a mapping of n sources to n objects):

- The loudness and frequency response of each object should match that of the corresponding source signal

- No distortion of the extracted objects

- The crosstalk between objects should be below a certain threshold

- Sources should not switch between objects (also if sources are moving or are silent for a long time)

Having these requirements in mind, we propose to extend the frequency response test method defined in [1] section 4.1.4.3 to ISM capturing solutions in multi-source scenarios:

- Calculate one frequency response measure per object

- For each object, calculate the frequency response according to TS 26.260 [2] clause 5.6.3.2, using the corresponding individual sound source recording as reference signal

4. Conclusion

Updates present in Sections 2 and 3 targeting multi-source evaluation for ISM capturing solutions are proposed to be included in the DaCAS-2 permanent document.

5. References

[1]  	S4-251477: “DaCAS-2: Test methodologies and requirements v0.3”

[2]  	3GPP TS 26.260: “Objective test methodologies for the evaluation of immersive 		audio systems”, Release 18

[3]	S4-251330: “Performance Evaluation for SBA solutions & Recording scenarios”

[4]	S4aA250101: “Performance Evaluation for SBA solutions”