3GPP TSG-SA4 Meeting #134		Tdoc S4-251884

# Dallas, US, 19-23 Nov 2025

Source:	Ericsson LM

Title:	IVAS PI data resolution of ISM position and orientation

Agenda item:	7.5

Document for:	Discussion & Agreement

# Introduction

The IVAS RTP payload format allows for transmission of Processing Information (PI) data assisting the rendering of IVAS frames. The endorsed proposal [1] uses a high resolution for signaling the positions and orientations of ISMs. While high resolution may be desirable from a quality perspective, this could cause a significant overhead when transmitting such data for a larger number of ISMs. The issue was discussed in [2] and [3]. In [2], a more compact representation is proposed.

In this contribution, the source suggests enabling this compact representation. Two potential variants of signaling are suggested, to be discussed and agreed on.

# Discussion

In [1], additional PI data types are introduced. Although the representations allow for accurate rendering, they may be having unnecessary high dynamic range or resolution for certain use cases, thereby consuming unnecessary transmission resources. The position and orientation data, e.g. ISM_ORIENTATION and ISM_POSITION, are two examples of such PI data. In [2], more compact representations are proposed for these.

Looking at the ISM_POSITION, which in [1] uses the same representation as the LISTENER_POSITION, we have a range of ±327.68 m, with 0.01 m resolution, utilizing 6 bytes per ISM. In [2], a similar resolution of 0.01 m is represented with a limited range of ~±10 m in X, Y and ± 5 m in Z direction resulting in 4 bytes per object. It is likely that the more compact representation is good enough in many use cases.

# Proposal

It is proposed to specify alternative representations of ISM_ORIENTATION and ISM_POSITION PI data in TS 26.253 for different application scenarios. The representations introduced in [1] can be considered for more advanced application scenarios, while representations like the ones introduced in [2] can be considered for higher efficiency. Two options for signaling the compact representation could be

- adding the compact representation orientations as new PI types, or

- signaling the compact representation in the PI header by reducing the PI size parameter by one bit.

Option 1) could be realized by using the reserved slots in the PI type table, e.g. by using the 4 entries 11011-11110 in Table A.3.5.5-2, and introducing new entries summarized in Table A* below, to be added to the tables for the forward and reverse direction PI data respectively.

Table A.3.5.5-2: Additional PI types in an IVAS session.

| Type bits | PI type | Description | SDP indication | Size (bytes) |
| --- | --- | --- | --- | --- |
| 01111, 11011-11110 | Reserved | - | - | - |

Table A*: New PI types to be added to Table A.3.5.5-1 and Table A.3.5.5-1A for forward and reverse direction PI data respectively.

| 11011 | ISM_ORIENTATION_COMPACT | Describes an orientation for each ISM, compact representation. | fisoc | Number of ISMs x 4 | TBD |
| --- | --- | --- | --- | --- | --- |
| 11100 | ISM_POSITION_COMPACT | Describes a position of each ISM, compact representation. | fipoc | Number of ISMs x 4 | TBD |
| 11000 | R_ISM_ORIENTATION_COMPACT | An editing request for orientation for each ISM, compact representation. | risoc | Number of ISMs x 4 | TBD |
| 11001 | R_ISM_POSITION_COMPACT | An editing request for position for each ISM, compact representation. | ripoc | Number of ISMs x 4 | TBD |

Option 2) would be to reduce the PI size field to 7 bits and use 1 bit in the PI header to indicate resolution.

0 1 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 +--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+ ¦PF¦ PM ¦ PI type ¦R ¦ PI size ¦ +--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+--+

The R bit could indicate 0 for high resolution and 1 for compact representation. The PI size would change according to Table A.3.5.2-3 below:

Table A.3.5.2-3: Size bits for PI header.

| PI Size bits | Indicated size in bytes (or other indication) |
| --- | --- |
| 000 0000 | 0 |
| 000 0001 | 1 |
| … | … |
| 111 1110 | 126 |
| 111 1111 | Another size word follows |

It is proposed to consider these options for signalling the compact orientation to allow efficient operation of ISM PI data.

References

Tdoc S4-251366, Further corrections to Annex A

Tdoc S4-250916, CR 26.253, RTP Header Extensions for IVAS

Tdoc S4-251005, IVAS PI data representations