3GPP TSG-SA WG4 Meeting #134	S4-251891

# Dallas, US, 17 – 21 November 2025

Source:	Nokia

Title:	Requirements for raw and compensated microphone signals

Agenda item:	7.6

Document for:	Discussion & Agreement

# Introduction

During the previous SA4 meetings, the requirements for raw and compensated microphone signal requirements have been formulated and documented in the DaCAS-2 permanent document [1]. Currently, the requirements comprise only recommendations for the raw microphone signals. In addition, certain requirements and recommendations for compensated signals have been proposed. Further requirements for the raw microphone signals are proposed in this contribution, to cover minimum performance requirements for resonance frequencies, SNR, ADC and acoustical overload point (AOP).

# Discussion

Currently, the proposed raw microphone signal requirements in DaCAS-2 comprise only recommendations on the raw microphone signal performance. These recommends are considered to give a guideline for implementors to achieve sufficient quality when implementing, e.g., DaCAS example solution to a device. In general, these recommendations are seen as a guideline for implementors to achieve sufficient immersive capture quality, even when using any other solution than design under DaCAS work to obtain immersive capture signal in IVAS input formats.

## 2.1 Raw microphone signal requirements

While the general send quality of devices is governed under other specifications and is typically targeted on the signals seen by the network/user, it is seen valuable to discuss and potentially set requirements for the capture front-end. We propose following requirements for the raw microphone signals: Resonance behaviour for raw integrated microphone signals, SNR performance for the raw microphone signals, requirements on ADC bit resolution, and requirements for AOP.

Resonance frequency: It is seen that implementors should try to achieve as high resonance frequencies as possible for the integrated microphones. Typically, achieved resonant behaviour have high impact on the obtained spatial capture quality, i.e., obtained resonance frequencies may limit the applicable frequency range for spatial audio capture. Furthermore, typically the frequencies above resonance frequencies are more unpredictable in terms of sensitivity and phase properties, potentially leading to a sub-optimal spatial audio capture quality at the frequencies above the resonances.

SNR: Signal-to-noise -ratio is seen as the key influencer on the obtained device performance. Also for devices capable on spatial capture, this is no different. It is considered that the higher the SNR of the integrated microphones is, the higher quality is achievable. Thus, it is seen that indicating sufficient raw microphone signal requirements in terms of SNR would benefit the implementors and the market in general.

ADC: It is understood that the current resolution of ADCs used in modern devices is at least 16 bits. To achieve sufficient SNR and low enough noise level, the devices using DaCAS example solutions should not use lower resolution than 16 bits.

AOP: Acoustic overload point defines the maximum sound pressure level (SPL) that the microphone can handle below certain distortion limit. To ensure proper performance in high-SPL situation, it is seen beneficial to set certain lower limit on the AOP performance when considering raw microphone signals.

%%%% START CHANGE %%%%

  - 3.1 Raw microphone signals

  - 3.1.1 Requirements

[

Editor’s note: Decision needed whether to have normative or informative requirements/recommendations.

[Raw microphone input for DaCAS example solutions shall comply with the requirements specified in Table 1.]

Table 1 Raw microphone signal requirements

| Feature | Requirement | Recommendation |
| --- | --- | --- |
| Raw frequency response (excluding resonances) | TBD | Minimum captured frequency (-3dB point) should be below 100 Hz Frequency response above the minimum frequency and below the resonances should be as flat as possible. |
| Resonances | Shall be above 6 kHz | Should be above 8 kHz. |
| SNR | Shall be at least 54 dB | Should be at least 60 dB |
| Sensitivity | TBD | [Level should be below -25 dBFS (1 dB) with 1 kHz sine signal @ 94 dB SPL] |
| Directivity | TBD | Recommended to have omnidirectional characteristics. Other directivity patterns are not generally excluded. Microphone directivity characteristics shall be documented clearly. |
| ADC bit depth | The ADC bit depth shall be at least 16 bits | The ADC bit depth should be 24 bits. |
| Acoustical Overload Point | Shall be at least 120 dB SPL @ 5% THD | Should be at least 130 dB SPL @ 5% THD |

NOTE: It is expected that all the raw integrated microphones comply with the above requirements. In addition, it is favorable to have as similar characteristics as possible for all raw integrated microphones.

]

%%%% END CHANGE %%%%

## 2.2 Compensated microphone signal requirements

On the current requirements for compensated microphone signal requirements, following (largely editorial) changes are proposed:

%%%% START CHANGE %%%%

  - 3.2 Compensation on microphone signals

Any raw microphone compensation, including the approaches outlined in the section 3.2.2, are considered as optional in the development of example solutions. Example solution developers may selectively adopt the solutions outlined in the section 3.2.2 or any other solution based on specific application requirements. If other solutions are adopted, these shall be documented.

    - 3.2.1 Requirements

[

Editor’s note: Decision needed whether to have normative or informative requirements/recommendations.

Compensated microphone input for DaCAS example solutions shall comply with the requirements specified in Table 2.

Table 2 Compensated microphone signal requirements

| Feature | Requirement | Recommendation |
| --- | --- | --- |
| Compensated frequency response | When compensation processing is applied to the same signal as used for designing the compensation filters: - Compensated frequency response shall be within ‘Required’ mask defined in Table 3 - Differences between compensated frequency responses shall be within ‘Required’ mask defined in Table 4 | Compensated frequency response should be considered in the context of isotropic equalization target. When compensation processing is applied to the same signal as used for designing the compensation filters: - Compensated frequency response shall be within ‘Recommended’ mask defined in Table 3 - Differences between compensated frequency responses shall be within ‘Recommended’ mask defined in Table 4 |
| Phase properties | TBD | Compensation processing should compensate sound source direction independent phase differences between integrated microphone responses. |

Table 3 Compensated microphone sensitivity/frequency mask

|  | Required |  | Recommended |  |
| --- | --- | --- | --- | --- |
| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) | Upper limit (dB) | Lower limit (dB) |
| 100 | 4 |  | 2 |  |
| 200 | 4 | -4 | 2 | -2 |
| 5000 | 4 | -4 | 2 | -2 |
| 12500 | 4 | -6 | 2 | -3 |
| 16000 | 4 |  | 2 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |  |  |

Figure 1 Compensated microphone response masks

Table 4 Compensated microphone sensitivity/frequency difference mask

|  | Required |  | Recommended |  |
| --- | --- | --- | --- | --- |
| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) | Upper limit (dB) | Lower limit (dB) |
| 100 | 2 |  | 1 | -1 |
| 200 | 2 | -2 | 1 | -1 |
| 5000 | 2 | -2 | 1 | -1 |
| 12500 | 2 | -4 | 1 | -2 |
| 16000 | 2 |  | 1 |  |
| NOTE: All sensitivity values are expressed in dB on an arbitrary scale. |  |  |  |  |

Figure 2 Compensated microphone response difference masks

]

%%%% END CHANGE %%%%

# Conclusion

In this contribution further requirements for raw microphone signals are discussed and proposed. In addition, editorial updates to the compensated microphone signal requirements are proposed. Proposal is to incorporate the changes to the DaCAS-2 permanent document [1].

# References

[1] S4-251477: “DaCAS-2: Test methodologies and requirements v0.3”