3GPP SA4 Audio SWG 134 																				S4-251904

Dallas, USA, 17th -25th Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.253 | CR | 0028 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Playout configuration status PI data for IVAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Panasonic Holdings Corporation, Nokia, Philips International B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IVAS_Codec |  |  |  |  |  | Date: |  |  | 2025-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Practical feature (context adaptation) is missing for which Annex A does not specify the RTP handling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RTP handling is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of context adaptation may lead to suboptimal immersive quality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depends on CR26253-0016 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE 1

### A.3.5.5	Supported PI data types

Supported PI types are listed in tables A.3.5.5-1 and A.3.5.5-2 and described in the following subsections. Table A.3.5.5-1 lists PI types for forward direction signalling. Table A.3.5.5-2 lists additional PI types.

Table A.3.5.5-1 : Supported forward direction PI types in an IVAS session.

| Type bits | Forward direction PI type | Description | SDP indication | Size (bytes) | Described in clause |
| --- | --- | --- | --- | --- | --- |
| 00000 | SCENE_ORIENTATION | Describes the orientation of a spatial audio scene in unit quaternions. | fsco | 8 | A.3.5.6.1.2 |
| 00001 | DEVICE_ORIENTATION_COMPENSATED | Describes the orientation of a device in unit quaternions. The orientation is compensated in the transmitted audio. | fdoc | 8 | A.3.5.6.1.3 |
| 00010 | DEVICE_ORIENTATION_UNCOMPENSATED | Describes the orientation of a device in unit quaternions. The orientation is not compensated in the transmitted audio. | fdou | 8 | A.3.5.6.1.3 |
| 00011 | ACOUSTIC_ENVIRONMENT | Selects and optionally describes the acoustic environment. | face | 1,5 or 8 | A.3.5.6.2 |

Table A.3.5.5-1A: Supported reverse direction PI types in an IVAS session.

| Type bits | Reverse direction PI type | Description | SDP indication | Size (bytes) | Described in clause |
| --- | --- | --- | --- | --- | --- |
| 11011 | PLAYOUT_CONFIGURATION_STATUS | Playout configuration status. Indicates the current status. | rpcs | 1 | A.3.5.7.1 |

Table A.3.5.5-2: Additional PI types in an IVAS session.

| Type bits | PI type | Description | SDP indication | Size (bytes) |
| --- | --- | --- | --- | --- |
| 11100-11110 | Reserved | - | - | - |
| 11111 | NO_PI_DATA | Indicates an empty PI data frame. | nopi | 0 |

NO_PI_DATA PI data type can be used to indicate empty PI data sections. The PM marker bits for a NO_PI_DATA PI data type shall be set as PM=10, see table A.3.5.2-2. For example, if an IVAS RTP payload includes multiple audio frames, and some of the audio frames do not have associated PI data, NO_PI_DATA PI type can be used.

CHANGE 2

A.3.5.7.8	Playout Configuration Status

The PLAYOUT_CONFIGURATION_STATUS PI data describes the media receiver playout configuration status. It provides a feedback mechanism towards the media sender enabling the media sender to adjust, optimize, or otherwise modify the audio capture or input format generation configuration or transmission of other PI data relating to specific playout configuration(s). In other words, the PCS PI data provides information about the media receiver’s playout context to aid audio capture or input format generation or PI data transmission, i.e., adapting the bitstream from media sender according to playout configuration of media receiver, and the PCS PI data therefore does not constitute a request for changing codec format, described in clause A.3.3.3.3.3.3. By signalling the playout configuration to the media sender performing the immersive audio capture, context-aware capture optimization and fine-tuning at the sender side may be achieved—leading to an enhanced overall audio experience with optimized use of the available resources.

The PCS PI data conveys the media receiver’s present playout context—by signalling whether the playout configuration is loudspeaker without height channels (mono, stereo, or any planar multi-channel layout including custom layouts), loudspeaker with height channels (any multi-channel layout with 3D playback capability including custom layouts), binaural (with or without room effect and with or without head tracking), or a default configuration with unspecified playout indication. Based on this information, the media sender may respond with a bitstream adapted to the media receiver's updated playout configuration. The size of the PCS PI data is 1 byte, with the PCS value accounting for 3 bits and the remaining 5 bits are zero padded to ensure byte alignment. The structure of the PCS PI data frame is illustrated in Figure A.3.5.7.8-1, and the PCS bit field definitions are defined in Table A.3.5.7.8-1.

If the PCS PI data indicates a loudspeaker configuration (PCS=001, PCS=010) the media sender may adjust the capture settings (e.g., capture front end) and, e.g., stop applying signal processing techniques for pre-binauralizing the audio or switch to any other suitable configuration. Additionally, the media sender should stop sending data related to binaural rendering (e.g., scene or device orientation PI data).

The PCS PI data can also include information indicating whether head tracking is used in binaural rendering (PCS=011-110) at the media receiver performing immersive audio reproduction. When the data indicates that head tracking is used (PCS=011, PCS=100) the media sender may, e.g., configure the processing of captured microphone signals to encode a spatial audio signal in a non-binauralized format (e.g., MASA or SBA). The signal can then be decoded/rendered at the media receiver for binaural reproduction using a head tracked decoding process. When the data indicates head tracking is not used (PCS=101, PCS=110), the media sender may configure processing of captured microphone signals to encode a spatial audio signal in binaural format (i.e., pre-binauralize the spatial audio signal). The signal can be decoded/rendered at the media receiver using a non-head tracked decoding process. In former case, the media sender may also send data related to binaural rendering (e.g., scene or device orientation PI data).

The media sender may adjust audio capture settings, or input format generation, based on current coded format and PCS PI data. If PCS PI data indicates a planar loudspeaker configuration (PCS=001), audio capture settings or input format with height information can be adjusted by removing height content (e.g., adjusting audio capture to ignore height information), switching to a subformat with no height information (e.g., from FOA to FOA planar), or removing (or inserting zero data to) height channels of a multi-channel input format. If PCS PI data indicates a loudspeaker configuration with height (PCS=010) or a binaural configuration (PCS=011-110), audio capture settings or input format generation can be adjusted by adding height content (e.g., adjusting audio capture to find/retain height information, switching to a subformat with height information (e.g., from planar FOA to FOA or from MC 7.1 to 7.1.4, e.g., by otherwise adding zero-data height information or channels). While a PCS PI data may result in a media sender switching, e.g., a subformat, the PCS PI data is not understood as a request to do so.

Additionally, if the media sender is transmitting AE PI data, described in clause A.3.5.6.2, to the media receiver and the PCS PI data indicates a binaural without room effect configuration (PCS=011, PCS=101) or a loudspeaker configuration (PCS=001, PCS=010), the media sender should stop transmitting AE PI data to the media receiver. In case the configuration is then changed back to binaural with room effect (PCS=100, PCS=110), the media sender may resume to transmit AE PI data.

PCS=000 may be used to indicate the playout configuration of the media receiver is un-specified. When PCS PI data is negotiated for the session, the playout configuration for the media receiver is expected to be un-specified (PCS=000) at the beginning of the session.

0 1 2 3 4 5 6 7
+-+-+-+-+-+-+-+-+
| PCS |0 0 0 0 0|
+-+-+-+-+-+-+-+-+

Figure A.3.5.7.8-1: PCS PI data frame

Table A.3.5.7.8-1: PCS bit field definitions

| PCS | Definition |
| --- | --- |
| 000 | Unspecified |
| 001 | Loudspeaker (without height) |
| 010 | Loudspeaker (with height) |
| 011 | Binaural (without room effect and with head tracking) |
| 100 | Binaural (with room effect and with head tracking) |
| 101 | Binaural (without room effect and without head tracking) |
| 110 | Binaural (with room effect and without head tracking) |
| 111 | Reserved |

END OF CHANGES