3GPP TSG-SA WG4 Meeting #134	S4-251905

# Dallas, US, 17 – 21 November 2025

Source:	Nokia, Dolby Laboratories Inc.

Title:	On bundling periods and ULBC usage

Agenda item:	7.9 – FS_ULBC

Document for:	DISCUSSION

# 1. Introduction

FS_ULBC time plan [1] indicates target to finalize identification of relevant design constraints for ULBC at present meeting, including:

- Frame length,

and to progress identifying other relevant design constraints, including:

- Algorithmic delay, and

- Speech quality.

Frame length and algorithmic delay are related to bundling periods, and then further related to link-level simulations for which the parameters have been debated at several meetings. These aspects furthermore touch speech quality, especially in terms of the end-to-end delay and resulting user experience.

This document continues discussion on the bundling periods with focus on expected ULBC user experience (or use cases).

# 2. Discussion

Voice frame length, codec algorithmic delay, and voice bundling period can all be seen to contribute towards the end-to-end delay as experienced by users. In the context of GEO channel discussion, bundling period is of particular interest. On high level, a smaller bundling period can be understood to enable lower end-to-end delay, while a larger bundling period, e.g., reduces overhead. For both codec design and user experience, various trade-offs can be expected. The exact relationship between the bunding period and frame length is open, and the specific codec and RTP payload designs (frame bundling) further impact, e.g., frame loss behaviour.

FS_ULBC discussions have focused on voice bundling periods of 80 ms, 160 ms, and 320 ms. Better understanding of the user experience expectation may be necessary.

One proposal [2] considers setting the maximum bundling time as no longer than 160 ms. In [2], the justification is based on significant loss of a voice segment under a packet loss scenario. For example, 320 ms of speech is lost for each lost packet in case of 320-ms bundling. Such behaviour could significantly impact user experience. On the other hand, a packet loss concealment example is provided in [3]. According to the example of [3], a longer bundling time, e.g., 320 ms as discussed here, can be used by a codec with a smaller frame size. Thus, the effect of packet loss can be quite different than in case of [2]. In both cases, the latency caused by the bundling period is however significant with impact on user experience.

Draft TR 26.960 [4] considers the end-to-end delay through inclusion of estimates for mouth-to-ear delay for relevant FS_ULBC scenarios: UEs connected via GEO and TN and UEs connected via GEO access only, see Table 5.1.3-1 in [4].

While all bundling periods (80, 160, 320 ms) in FS_ULBC result in significant mouth-to-ear delay estimates, each step introduces more delay: the additional delay is 80 ms and 160 ms (for a total of 240 ms extra delay), when going from 80 ms to 160 ms and from 160 ms to 320 ms, respectively.

Based on the two scenarios, the three bundling periods, and other evaluation assumptions, lower bound estimate for mouth-to-ear delay is about 0.5 s and upper bound estimate is about 1.5 s (see Table 5.1.3-1). The sources note that while the expected user experience can be characterized as regular voice call (although with some level of user dissatisfaction) towards the lower bound, it is better characterized as push-to-talk type experience towards the upper bound. Where exactly this differentiation should be made may be left for further discussion.

# 3. Summary

Assuming current voice bundling periods (80 ms, 160 ms, 320 ms) are all relevant for FS_ULBC (and the related link-level simulations), the user experience for ULBC calls can appear very “laggy” and can often be best characterized as push-to-talk type experience. This is true at least towards the upper range of current mouth-to-ear estimates.

In view of the sources, it will be necessary to clarify what is expected in each application scenario in terms of the user experience and further take these aspects into account in the design constraints and performance requirements as needed. Specifically, it should be clarified whether ULBC is expected to operate in both regular voice call operation and used in push-to-talk fashion, or whether it is only one of these experiences that is targeted. Accordingly, requirements relating to mouth-to-ear delay may require further definition and the relevant bundling periods updated as needed. For example, it could potentially be concluded that 320 ms bundling period is not suitable for ULBC, as argued for packet loss reasons previously in [2].

# References

[1]	S4-250751: “Timeplan for FS_ULBC Study Item”, China Mobile, April 2025

[2]	S4aA250226: “[FS_ULBC] On bundling time limit for ULBC”, Huawei Technologies Co., Ltd., September 2025

[3]	S4aA250268: “[FS_ULBC] Packet Loss Concealment with existing AI based codec DAC”, Dolby Laboratories Inc., Ericsson LM, Nokia, Novamint, September 2025

[4]	Draft TR 26.960, v0.2.0, May 2025