Source:	Orange

Title:	On design constraints for ULBC

## Document for:	Agreement

## Agenda Item:	7.9

- Introduction

The present Tdoc is a revision of [1] (with minor improvements) for consideration in discussions and potential editing of ULBC design constraints.

- Proposed design constraints

| Parameter | Design Constraint | Note |
| --- | --- | --- |
| Sample rate and audio bandwidth | Support of [8, 16, 32, 48] kHz / [NB, WB, SWB, FB] required Editor’s Notes: - Support of 8 kHz justified for interoperability. Would NB be just tested and supported “externally” based on some external resampling? - Support of 48 kHz may be considered at higher bitrate operation - Support at least a single model (e.g. SWB)? | Many neural codecs operate at 24 kHz, this specific sampling rate should be discussed There may be complexity considerations associated to this box; joint decisions may be needed |
| Frame length | [Multiple of 20 ms] | Dependency: RAN simulations and system considerations |
| Packet loss concealment (PLC) | Support required Editor’s Notes: - Typical loss profiles/characteristics to be clarified - Support of redundancy to be clarified | Support of JBM to be added as extra box |
| Potential use of noise suppression as part of the codec | If noise suppression is supported inside ULBC, it is expected that there is a mechanism to disable noise suppression in the codec Editor’s Notes: Clarifications needed: - Need to support noise suppression in UBLC? (typically vendor specific, defined outside the codec) - Impacts on test methodology, DTX operation/performance | Motivations: - Disabling noise suppression is required to test feature apart - Avoid tandeming in real operation - IMS voice communication is defined in TS 22.228, GEO satellite access has no specific requirement on noise handling |
| Discontinuous transmission including voice activity detection and comfort noise | Support required Editor’s Note: Typical radio characteristics and optimizations (SPS, DRX, bitrate) to be clarified |  |

## Reference

- 3GPP Tdoc S4aA250240, On design constraints for ULBC, Source: Orange