3GPP TSG-SA WG4 Meeting #134	S4-251979

Dallas, US, 17 – 21 November 2025

Revision of S4-251868

Source:	Fraunhofer IIS

Title:	Performance evaluation of ISM capturing solutions

Agenda item:	7.6

Document for:	Discussion and agreement

# 1. Introduction

While the evaluation of ISM capturing solutions is covered by the single-source test methods and recording scenario included in the current DaCAS-2 permanent document [1] and in TS 26.260 [2], multi-source evaluation suitable for ISM capturing example solutions has not been addressed so far. Initial discussion on multi-source evaluation has been triggered in [3] and [4], targeting SBA capturing solutions specifically. In this document, we propose agreement for additional recording scenarios and objective performance test methods for ISM capturing example solutions, focusing on multi-source scenarios.

# 2. Recording scenarios and procedure

To enable multi-source evaluation of ISM capturing solutions, we propose two additional recording scenarios to be included in the database, which are detailed in Table 1. The proposed recording scenarios target smartphone-type devices and should therefore be made for all available smartphone-type target devices.

- Teleconferencing scenario (X-1, Figure 1): Smartphone lying flat on a table, screen facing up, 2 speakers positioned along the long axis of the smartphone within a certain opening angle

- Video call scenario (X-2, Figure 2): Smartphone in hand-held mode, landscape orientation, 2 speakers facing the screen/front camera

The proposed recording scenarios are also attached as recording_scenarios.xlsx.

To ensure the availability of suitable reference signals for the evaluation of ISM capturing solutions, we propose that the following recording procedure is followed for multi-source recording scenarios targeting ISM capturing solutions:

- Record sound sources individually. These individual sound source recordings correspond to the reference signals used for the evaluation of ISM capturing solutions.

- Sum up the individual sound source recordings to obtain final input signals to example solutions. This is feasible since it is foreseen that the raw microphone signals must be recorded.

Table 1 Recording scenarios for multi-source ISM evaluation

| No | X-1 | X-2 |
| --- | --- | --- |
| Sound source | According to “Sound source” specified in Table 1 in section 4.1.2.1 of [1] | According to “Sound source” specified in Table 1 in section 4.1.2.1 of [1] |
| Source signal | British English single talk sequence according to the ITU-T P.501 | British English single talk sequence according to the ITU-T P.501 |
| Source signal characteristics | Language: English Gender: Male, Female Range: 20Hz-20kHz Length: 35.4 s Level: -27 dB RMS Sample Rate: 48 kHz Bit depth: 16 bit | Language: English Gender: Male, Female Range: 20Hz-20kHz Length: 35.4 s Level: -27 dB RMS Sample Rate: 48 kHz Bit depth: 16 bit |
| Sound source calibration | According to “Sound source calibration” specified in Table 1 in section 4.1.2.1 of [1] | According to “Sound source calibration” specified in Table 1 in section 4.1.2.1 of [1] |
| Acoustic environment | According to “Acoustic environment” specified in Table 1 in section 4.1.2.1 of [1] | According to “Acoustic environment” specified in Table 1 in section 4.1.2.1 of [1] |
| Detailed Positioning | UE positioning/orientation: UE lying flat on a table, screen facing up Source distance: 0.5-1 m, equal distance of both sources to UE Source direction: Loudspeaker diaphragm towards the center of the UE Source height (relative to UE): 0.4 m Source angles: Azimuth angle combinations (2 sources active simultaneously): [-90°, 90°], [-110°, 70°], [-110°, 90°] | UE positioning/orientation: UE in hand-held mode, landscape orientation, screen oriented towards sources Source distance: 0.3-0.5 m, equal distance of both sources to UE Source direction: Loudspeaker diaphragm towards the center of the UE Source height (relative to UE): 0 m Source angles: Azimuth angle combinations (2 sources active simultaneously): [-30°, 30°], [-45°, 45°], [-30°, 45°] |
| Reference recording | The reference recordings correspond to the individual sound source recordings obtained by following the outlined recording procedure. | The reference recordings correspond to the individual sound source recordings obtained by following the outlined recording procedure. |
| Description/additional info | Recording procedure - Record sound sources individually - Sum up the individual sound source recordings to obtain final input signals to example solutions. This is feasible since it is foreseen that the raw microphone signals must be recorded. Sources at different locations should use different sequences as the source signal. The following overlap pattern of the two sources should be implemented: only source 1 active (25%) -> only source 2 active (25%) -> source 1 + source 2 active (50%) Device types: This scenario is applicable only for smartphone-type devices | Recording procedure - Record sound sources individually - Sum up the individual sound source recordings to obtain final input signals to example solutions. This is feasible since it is foreseen that the raw microphone signals must be recorded. Sources at different locations should use different sequences as the source signal. The following overlap pattern of the two sources should be implemented: only source 1 active (25%) -> only source 2 active (25%) -> source 1 + source 2 active (50%) Device types: This scenario is applicable only for smartphone-type devices |

# Figure 1 Sketch of recording scenario X-1

Figure 2 Sketch of recording scenario X-2

# 3. Objective performance evaluation

Considering ISM capturing example solutions in multi-source scenarios, a number of requirements can be identified (assuming a mapping of n sources to n objects):

- The loudness and frequency response of each object should be sufficiently close to that of the corresponding source signal

- Distortion of the extracted objects should be below an acceptable threshold

- The crosstalk between objects should be below an acceptable threshold

- Sources should not switch between objects (also if sources are moving or are silent for a long time)

Having these requirements in mind, we propose to extend the frequency response test method defined in [1] section 4.1.4.3 to ISM capturing solutions in multi-source scenarios:

- Calculate one frequency response measure per object

- For each object, calculate the frequency response according to TS 26.260 [2] clause 5.6.3.2, using the corresponding individual sound source recording as reference signal

4. Conclusion

Updates present in Sections 2 and 3 targeting multi-source evaluation for ISM capturing solutions are proposed to be included in the DaCAS-2 permanent document.

5. References

[1]  	S4-251477: “DaCAS-2: Test methodologies and requirements v0.3”

[2]  	3GPP TS 26.260: “Objective test methodologies for the evaluation of immersive 		audio systems”, Release 18

[3]	S4-251330: “Performance Evaluation for SBA solutions & Recording scenarios”

[4]	S4aA250101: “Performance Evaluation for SBA solutions”