3GPP TSG-SA4 Meeting #134	S4-251972

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.261 | CR | 0002 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New performance requirements for immersive UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | HEAD acoustics GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATIAS_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Performance requirements and objectives are required to ensure sufficient quality of immersive UEs supporting IVAS codec |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Updated performance requirements are not available for the evaluation of immersive UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 26.260 CR 008 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

# Introduction

TS 26.261 specifies minimum performance requirements for the electro-acoustic characteristics of LTE, NR and WLAN terminals when used to provide immersive services. The performance requirements are specified in the main body of the text; the test methods and test considerations are described in TS 26.260.

# 1	Scope

The present document specifies minimum performance requirements for the electro-acoustic characteristics of LTE, NR and WLAN terminals that support the 3GPP Immersive Voice and Audio Service (IVAS). It is applicable to any terminal capable of supporting wideband, super-wideband or fullband immersive services.

The set of minimum performance requirements enables a guaranteed level of speech quality while taking possible physical limits of the terminal design into account. Some performance objectives are also defined if such design limits can be overcome.

The present document covers both conversational services based on MTSI / telepresence and non-conversational services.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.260: "Objective test methodologies for the evaluation of immersive audio systems"

[3]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[4]	3GPP TS 26.131: "Terminal Acoustic Characteristics for Telephony; Requirements".

[5]	ETSI TS 103 739 (2021-10) V1.4.1: " Transmission requirements for wideband mobile wireless terminals (handset and headset) from a QoS perspective as perceived by the user".

[6]	ETSI TS 103 740 (2021-10) V1.4.1: "Transmission requirements for wideband mobile wireless terminals (hands-free) from a QoS perspective as perceived by the user".

[7]	3GPP TS 26 250: "Codec for Immersive Voice and Audio Services (IVAS); General overview".

[8]	3GPP TS 26 251: "Codec for Immersive Voice and Audio Services (IVAS); C code (fixed-point)".

[9]	3GPP TS 26 253: "Codec for Immersive Voice and Audio Services (IVAS); Detailed Algorithmic Description including RTP payload format and SDP parameter definitions".

[10]	3GPP TS 26 258: "Codec for Immersive Voice and Audio Services (IVAS); C code (floating-point)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document the terms wideband, super-wideband and fullband refer to test signals associated with the corresponding operating codec modes in TS 26.260.

The overload point (maximum load capacity) is for the purposes of this document defined as the RMS level of a digital representation of a full-scale pure tone at the input of the IVAS encoder. The overload point is defined in TS 26.260.

For the purposes of the present document, the term electrical interface is defined as an analogue or digital access to a UE, which allows injecting and capturing signals electrically instead of through an acoustical interface. The interface can be either analogue (wired) or digital (wired or wireless). The purpose of this interface is to connect a separate device (typically a headset), which provides a receiver and transmitter.

Stereo panorama: The spatial image of a stereo signal, in which the sound source directions lie in a range from -100% (left) to 100% (right).

## 3.2	Symbols

dB	Decibel

Hz	Unit of frequency (Hertz)

ϕ	Azimuth angle (phi)

θ	Elevation angle (theta)

TR	UE delay in receiving direction

TS	UE delay in sending direction

ζϕ	Estimate for a single source direction in the stereo panorama that is physically positioned in direction ϕ relative to the capturing device

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

DTX	Discontinuous Transmission

EVS	Enhanced Voice Services

HATS	Head and Torso Simulator

ILD	Interaural Level Difference

ISM	Independent Stream with Metadata

ITD	Interaural Time Difference

IVAS	Immersive Voice and Audio Services

MASA	Metadata-Assisted Spatial Audio

OMASA	Objects (ISM) with Metadata-Assisted Spatial Audio

OSBA	Objects (ISM) with Scene-Based Audio

POI	Point of Interconnection (with PSTN)

RLR	Receive Loudness Rating

SBA	Scene-Based Audio

SLR	Send Loudness Rating

WLAN	Wireless Local Area Network

# 4	Interfaces

## 4.1	General

The interfaces required to define immersive terminal electro-acoustic characteristics are shown in TS 26.260. These are the air interface and the point of interconnect (POI). The interfaces are shown for different types of immersive formats.

Measurements can be made using the system simulator (SS) described in TS 26.260. For conversational services, MTSI aspects are specified by TS 26.114 [3].

## 4.2	Air interfaces

The same air interfaces as in clause 4.2 of TS 26.131 [4] apply.

## 4.3	Acoustical interfaces

The acoustical interfaces are described in clause 5.4.2 of TS 26.260 [2].

## 4.4	Electrical interfaces

The electrical interface UE is considered in this specification and details on standardized analogue (wired) and digital (wired and wireless) electrical interfaces can be found in TS 26.260. For the electrical interface, the POI in sending / receiving direction is respectively defined as the input / output of the reference coder of the system simulator.

Any of the UE types mentioned in clause 4.3 providing an electrical interface can be considered as Electrical Interface UE.

## 4.5	UE configuration

4.5.1	Noise suppression mode configuration

UEs may support multiple noise suppression modes in UE sending direction as specified in Annex A of TS 26.253 [29].

# 5	Performance in sending

## 5.1	Applicability

The performance requirements in this clause shall apply when UE is used to provide immersive audio capture.

## 5.2	Delay

The UE delay TS in send direction shall be less or equal to the delay requirements in Table 5.2.

Table 5.2: Requirement on sending UE delay

| Audio format | Subformat | UE type(s) | Maximum delay TS (ms) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Performance Requirement | Performance Objective |
| Stereo | Stereo | All | TBD | TBD |
|  | Binaural | All | TBD | TBD |
| ISM | ISM1-ISM4 | All | TBD | TBD |
| SBA | FOA | All | TBD | TBD |
|  | HOA2 | All | TBD | TBD |
|  | HOA3 | All | TBD | TBD |
| MASA | 1 TC | All | TBD | TBD |
|  | 2 TC | All | TBD | TBD |
| OSBA | (ISM1-ISM4) x (FOA, HOA2, HOA3) | All | TBD | TBD |
| OMASA | (ISM1-ISM4) x MASA | All | TBD | TBD |
| Multichannel | 5.1 | All | TBD | TBD |
|  | 7.1 | All | TBD | TBD |
|  | 5.1.2 | All | TBD | TBD |
|  | 5.1.4 | All | TBD | TBD |
|  | 7.1.4 | All | TBD | TBD |

NOTE: This requirement only applies for one-way (sending only) cases where round-delay does not apply.

Compliance shall be checked by the relevant tests described in TS 26.260.

## 5.3	Loudness

### 5.3.1	Loudness for single source

The nominal values of SLR to the POI shall be:

SLR = 13 ± 3 dB

NOTE:	The value of 13 dB is motivated by the corresponding SLR requirement for desktop hands-free and handheld hands-free in TS 26.131 [4].

Compliance shall be checked by the relevant tests described in TS 26.260.

### 5.3.2	Loudness for ambient sound fields

For all SND-UE-types providing an acoustical interface, the ambient sound field loudness in send direction at the POI shall be according to the ranges in Table T6.

Table 5.3.2-1: Ambient sound fields used for acoustical interface UE

| Ambient Sound Field | Ambient Sound Field Loudness (in LKFS) |  |  |  |
| --- | --- | --- | --- | --- |
|  | ns-disabled | ns-min | ns-default | ns-max |
| [Rock_Concert.wav] | TBD ± TBD | TBD ± TBD | TBD ± TBD | ≤ TBD |
| [Nature_Scene.wav] | TBD ± TBD | TBD ± TBD | TBD ± TBD | ≤ TBD |
| [Kindergarden.wav] | TBD ± TBD | TBD ± TBD | TBD ± TBD | ≤ TBD |
| [Downtown_Scene.wav] | TBD ± TBD | TBD ± TBD | TBD ± TBD | ≤ TBD |
| NOTE: Wave filenames correspond to the database provided in clause 8 of ETSI TS 103 224. The table entries TBDs are placeholders and need to be confirmed. |  |  |  |  |

For all SND-UE-types providing an electrical interface with binaural input format, the ambient sound field loudness in send direction at the POI shall be according to the ranges Table T7.

Table 5.3.2-2: Ambient sound fields used for electrical interface UE and binaural input format

| Ambient Sound Field | Ambient Sound Field Loudness (in LKFS) |  |  |  |
| --- | --- | --- | --- | --- |
|  | ns-disabled | ns-min | ns-default | ns-max |
| [Rock_Concert.wav] | TBD ± TBD | TBD ± TBD | TBD ± TBD | ≤ TBD |
| [Nature_Scene.wav] | TBD ± TBD | TBD ± TBD | TBD ± TBD | ≤ TBD |
| [Kindergarden.wav] | TBD ± TBD | TBD ± TBD | TBD ± TBD | ≤ TBD |
| [Downtown_Scene.wav] | TBD ± TBD | TBD ± TBD | TBD ± TBD | ≤ TBD |
| NOTE: Wave filenames correspond to the database provided in clause 8 of ETSI TS 103 224. The table entries TBDs are placeholders and need to be confirmed. |  |  |  |  |

Compliance shall be checked by the relevant tests described in TS 26.260.

## 5.4	Frequency response

### 5.4.1	Frequency response for single source

#### 5.4.1.1	General

The sensitivity/frequency characteristics shall be as follows.

The sending sensitivity frequency response, measured from the source direction under test to the SS audio output (digital output of the reference speech decoder of the SS), shall be within a mask, which can be drawn between the points given in the respective requirements table. The mask is drawn with straight lines between the breaking points in the requirements table on a logarithmic (frequency) - linear (dB sensitivity) scale.

#### 5.4.1.2	Acoustical interface

The requirements for acoustical interface testing are given in Table 5.4.1.2.

Table 5.4.1.2: Sending sensitivity/frequency requirements table for acoustical interface

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | TBD |  |
| 200 | TBD | TBD |
| 300 | TBD | TBD |
| 4000 | TBD | TBD |
| 8000 | TBD | TBD |
| 12000 | TBD |  |

Compliance shall be checked by the relevant tests described in TS 26.260.

#### 5.4.1.3	Electrical interface

The requirements for electrical interface testing are given in Table 5.4.1.3-1.

Table 5.4.1.3-1: Sending sensitivity/frequency requirements table for electrical interface

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | TBD |  |
| 200 | TBD | TBD |
| 300 | TBD | TBD |
| 6000 | TBD | TBD |
| 8000 | TBD | TBD |
| 12000 | TBD |  |

Compliance shall be checked by the relevant tests described in TS 26.260.

## 5.5	Directional information

### 5.5.1	Stereo

The estimated source directions in the stereo panorama ζϕi shall be monotonically increasing between -60° and +60°, i.e., ζϕi-ζϕi-1≥10% for i=3,…,6,  where i is the source direction index according to Table 5 in TS 26.260. This requirement verifies that the estimated stereo panorama is consistent for the relevant source directions ϕi.

The estimated source directions in the stereo panorama shall provide a minimum and symmetric width between ϕ2 and ϕ6, i.e., ζϕ2 and ζ(ϕ6) shall be larger than or equal to 60%.

The estimated source directions in the stereo panorama shall be consistent at the edges, i.e., ζϕ1≤ ζϕ2 and ζϕ7≥ζϕ6.

For frontal incidence position (ϕ4=0°), the estimated sound source direction shall be in the center of the stereo panorama, i.e., ζϕ4<3%.

For the first L = 7 source directions of Table 5 in TS 26.260, the absolute value of ICTD shall be less than 1.5 ms.

Compliance shall be checked by the relevant tests described in TS 26.260.

### 5.5.2	SBA

#### 5.5.2.1	Applicability

Based on the specification of the physical test arrangement used for UE testing in sending according to clause 5.4.2 of TS 26.260, a UE may support half-circular planar, full-circular planar or 3D spherical SBA capture. Depending on this capability, either the requirements according to clauses 5.5.2.2, 5.5.2.3 or 5.5.2.4 apply.

If the type of SBA capture capability is not specified by the manufacturer, at least the requirements for half-circular planar capture shall be met. In this case, it is recommended to additionally evaluate requirements for full-circular planar and 3D spherical capture capabilities. This allows to determine and report the UE’s supported capture capability.

ϕi-ϕi

(θi-θi

Compliance shall be checked by the relevant tests described in TS 26.260.

#### 5.5.2.2	Half-circular planar capture

For UEs supporting half-circular planar capture, the following set of requirements apply:

Azimuth

For half-circular planar capture, the estimated azimuths ϕi for source directions i=1…L-1, where L=7, as listed in Table 5 of TS 26.260, shall be monotonically increasing such that ϕi+1>ϕi.

The estimated azimuth for source direction 4, the 0° source direction, shall be well within the front half of the listening plane, such that -60°<ϕ4<60°

The estimated azimuths of source directions 1 and 7, the ±90° source directions, shall be well within the right and left halves of the listening plane, respectively, such that -150°<ϕ1<-30°and 30°<ϕ7<150°.

NOTE:	The measured angle ϕ1 shall be unwrapped to ensure continuity in the neighbourhood of its corresponding source direction  ϕ1.

Elevation

The estimated source elevations for the source directions listed in Table 5 of TS 26.260 [shall have an absolute value of θi≤30°].

#### 5.5.2.3	Full-circular planar capture

For UEs supporting full-circular planar capture, the requirements according to clause 5.5.2.2 apply with the source directions i=1…L-1, where L=12, as listed in Table 5 of TS 26.260. In addition, the requirement ϕ1>ϕ12-360° applies.

Furthermore, the estimated azimuth of source direction i=10 shall be well within the rear half of the listening plane, such that 120°< ϕ10<240°.

NOTE:	The measured angles ϕ1 and ϕ12 shall be unwrapped to ensure continuity in the neighbourhood of their corresponding source directions  ϕ1 and ϕ12, respectively.

#### 5.5.2.4	3D spherical capture

UEs supporting full SBA capture should meet the requirements for full-circular planar capture according to clause 5.5.2.3.

In addition, the following requirements apply:

Azimuth

The requirements according to clause 5.5.2.2 apply with the source directions i=13…L-1, where L=20, as listed in Table 5 of TS 26.260. In addition, the requirement  ϕ13>ϕ20-360° applies.

The estimated azimuths of non-zero elevation source directions i=13,17, i.e. the ±90° azimuth directions, shall be within the right and left halves of the listening plane, respectively, such that -180°<ϕ13<0°and 0°<ϕ17<180°.

The estimated azimuths of non-zero elevation source directions i=15, 19, i.e. the 0° and 180° azimuth directions, shall be within the front and rear halves of the listening plane, respectively, such that -90°<ϕ15<90°and 90°<ϕ19<270°.

NOTE:	The measured angles ϕ13 and ϕ20 shall be unwrapped to ensure continuity in the neighbourhood of their corresponding source directions  ϕ13 and ϕ20, respectively.

Elevation

[Source directions with equal non-zero elevations, i.e. i=13…20, shall have estimated elevations within a range of ±30° from their mean estimated elevation, such that θ -θi≤30°, where  θ=1Ni=1320θi.

The following objective pertains to source directions beyond the sets described in Table 5 in TS 26.260. It is included to test a UE capable of 3D spherical capture more fully, if additional source directions were to be included.

Sets of source directions with the same azimuth should have monotonically increasing estimated elevation with respect to their actual elevation, such that for each pair of source directions i and j with equal actual azimuth ϕi=ϕj, if θj>θi then θj>θi].

### 5.5.3	MASA

Based on the specification of the physical test arrangement used for UE testing in sending according to clause 5.4.2 of TS 26.260, a UE may support half-circular planar, full-circular planar or 3D spherical MASA capture. Depending on this capability, either the requirements according to clauses 5.5.2.2, 5.5.2.3 or 5.5.2.4 apply.

If the type of MASA capture capability is not specified by the manufacturer, at least the requirements for half-circular planar capture shall be met. In this case, it is recommended to additionally evaluate requirements for full-circular planar and 3D spherical capture capabilities. This allows to determine and report the UE’s supported capture capability.

ϕi-ϕi

(θi-θi

Compliance shall be checked by the relevant tests described in TS 26.260.

## 5.6	Acoustic echo control

Requirements and performance objectives for TCL only apply for UEs for certain combinations of SND-UE-type and RCV-UE-type as specified according to clause 5.4.2 of TS 26.260. For nominal setting of the volume control, requirements for TCL are provided in Table 5.6-1 for each SND-/RCV-UE-type combination.

For any SND-/RCV-UE-type combination and any setting of the volume control, TCL shall be ≥ [40 dB].

Table 5.6-1: Requirements on TCL (in dB) for nominal volume control setting

| UE-Type |  | RCV |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Headset UE | Handheld Hands-free UE | Table-mounted UE | Loudspeaker UE |
| SND | Headset UE | [46] | [46] | [46] | [46] |
|  | Handheld Hands-free UE | [46] | [46] | [46] | [46] |
|  | Table-mounted UE | [46] | [46] | [46] | [46] |

NOTE:	Values in brackets are provisional.

Compliance with this requirement shall be checked by the relevant test described in TS 26.260.

# 6	Performance in receiving

## 6.1	Applicability

The performance requirements in this clause shall apply when UE is used to provide immersive audio rendering.

## 6.2	Delay

The UE delay TR in receiving direction shall be less or equal to TBD.

NOTE:	This requirement only applies for one-way (receiving only) cases where round-delay does not apply.

Compliance shall be checked by the relevant tests described in TS 26.260.

## 6.3	Loudness

The nominal value of Loudness Level in Receive (LLR) shall be:

-	75 ± 4 phon for headset UE,

-	75 ± 4 phon for electrical interface UE (including binaural rendering and calibration),

-	67 -4/+8 phon for handheld hands-free UE,

-	71 ± 4 phon for table-mounted hands-free UE,

-	75 ± 4 phon for loudspeaker UE.

In case a user controlled receive volume control is provided, for at least one setting of the control the LLR shall meet the nominal value.

When the control is set to maximum, the LLR shall not be louder than 89 phon for all UE types. With the volume control set to the minimum position the LLR shall not be quieter than 52 phon and should not be quieter than 58 phon for all UE types.

NOTE:	The loudness level requirements are motivated by corresponding requirements in ETSI TS 103 739 [5] for desktop hands-free and handheld hands-free and in ETSI TS 103 740 [6] for headset, respectively.

Performance requirements and objectives apply for source position of azimuth 0° and elevation 0° in the test signal. They should also be evaluated for the source positions listed in Table 6.3, and results should be included in the test report.

Table 6.3: Additional source positions for loudness

| Azimuth [°] | Elevation [°] |
| --- | --- |
| +90 | 0 |
| -90 | 0 |
| 180 | 0 |
| 0 | +90 |

Compliance shall be checked by the relevant tests described in TS 26.260.

## 6.4	Frequency response

### 6.4.1	Frequency response for single source

#### 6.4.1.1	General

The sensitivity/frequency characteristics shall be as follows.

The receiving sensitivity frequency response, measured either from the digital interface to the DRP with diffuse-field correction or from the SS audio input (analogue or digital input of the reference speech encoder of the SS) to the DRP with diffuse-field correction shall be within a mask, which can be drawn with straight lines between the breaking points in table 2 on a logarithmic (frequency) - linear (dB sensitivity) scale.

#### 6.4.1.2	Acoustical interface

The requirements for acoustical interface testing are given in Table 6.4.1.2-1.

Table 6.4.1.2-1: Sending sensitivity/frequency requirements table for acoustical interface

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | TBD |  |
| 200 | TBD | TBD |
| 300 | TBD | TBD |
| 4000 | TBD | TBD |
| 8000 | TBD | TBD |
| 12000 | TBD |  |

Compliance shall be checked by the relevant tests described in TS 26.260.

#### 6.4.1.3	Electrical interface

The requirements for electrical interface testing are given in Table 6.4.1.3-1.

Table 6.4.1.3-1: Sending sensitivity/frequency requirements table for electrical interface

| Frequency (Hz) | Upper limit (dB) | Lower limit (dB) |
| --- | --- | --- |
| 100 | TBD |  |
| 200 | TBD | TBD |
| 300 | TBD | TBD |
| 6000 | TBD | TBD |
| 8000 | TBD | TBD |
| 12000 | TBD |  |

Compliance shall be checked by the relevant tests described in TS 26.260.

## 6.5	Binaural rendering

### 6.5.1	ILD

For headset UE or electrical interface UE (if intended for headset usage), the UE shall pass ILD requirements defined in Table 6.5.1-1.

If the UE supports headtracking, the same requirements in Table 6.5.1 shall be met for each HATS orientation φj∈{0°, -30°, +30°}.

NOTE:	If the UE supports headtracking and the requirements are not met, the test operator should verify if the failure might be caused by an automatic reset of the reference orientation.

Table 6.5.1-1: ILD requirements

| i | Source azimuth (°) | Source elevation (°) | 500 Hz |  | 1000 Hz |  | 2000 Hz |  | 4000 Hz |  | 8000 Hz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Lower limit (dB) | Upper limit (dB) | Lower limit (dB) | Upper limit (dB) | Lower limit (dB) | Upper limit (dB) | Lower limit (dB) | Upper limit (dB) | Lower limit (dB) | Upper limit (dB) |
| 1 | 0 | 0 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 2 | 180 | 0 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 3 | 0 | 90 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 4 | 90 | 0 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |
| 5 | -90 (270) | 0 | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |

Compliance shall be checked by the relevant tests described in TS 26.260.

### 6.5.2	ITD

For headset UE or electrical interface UE (if intended for headset usage), the UE shall pass ITD requirements defined in Table 6.5.2.

If the UE supports headtracking, the same requirements in Table 6.5.2-1 shall be met for each HATS orientation.

NOTE:	If the UE supports headtracking and the requirements are not met, the test operator should verify if the failure might be caused by an automatic reset of the reference orientation.

Table 6.5.2-1: ITD requirements

| i | Source azimuth (°) | Source elevation (°) | Lower limit (ms) | Upper limit (ms) |
| --- | --- | --- | --- | --- |
| 1 | 0 | 0 | TBD | TBD |
| 2 | 180 | 0 | TBD | TBD |
| 3 | 0 | 90 | TBD | TBD |
| 4 | 90 | 0 | TBD | TBD |
| 5 | -90 (270) | 0 | TBD | TBD |

Compliance shall be checked by the relevant tests described in TS 26.260.

### 6.5.3	Motion-to-Sound Latency

The M2S latency measured with the test method from clause 5.7.5 from TS°26.260 should not exceed [80 ms] and shall not exceed [100 ms].

The absolute difference between the initial ILD calibration (ILD0°) and the verification ILD (ILD0°,v) shall not exceed 0.5 dB.

Compliance shall be checked by the relevant tests described in TS 26.260.

# 7	Performance in sending+receiving

## 7.1	Applicability

The performance requirements in this clause shall apply when UE is used to provide end to end immersive audio, including both capture and rendering.

## 7.2	Delay

The UE roundtrip delay (TS + TR) in send and receive direction shall be less or equal to TBD.

NOTE:	This requirement does not apply for one-way scenarios, delay corresponds here to complete terminal delay.

==============End of change==============