3GPP TSG SA WG 04 Meeting #135                                                                     S4-260300

Goa, India, Feb 9-14, 2026                                                               revision of S4-260290

# Title:	Analysis and recommended handling of the reply liaisons from other working groups to SA4 on ULBC

Source:	Qualcomm Inc.

Agenda item:	7.8 FS_ULBC

# Document for:	Agreement

# 1. Introduction

SA4 received the following reply LS’s on ULBC. S4aA250139[1] analyzed and recommended actions for the remaining LS’s that S4-251953 [2] has not addressed, but S4aA250139 [1] was noted without presentation at the Dec 2nd Audio Ad Hoc meeting. In addition, there was a new LS from SA1 S4-260018 [4]. This paper analyzes all remaining LS’s (which have been renamed) from S4-251953 [2], taking into account recent developments in other Working Groups since S4-251953 [2], along with the new SA1 LS, and recommends actions that are needed. For clarity, the parts that were discussed at SA4 #134 were crossed out.

| 3 | S4-260011 | Reply LS on the RAN simulation assumptions for ULBC | RAN1 | To: SA4 |
| --- | --- | --- | --- | --- |
| 4 | S4-260012 | Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC | RAN1 | CC: SA4 |
| 5 | S4-260015 | Reply LS on the RAN simulation assumptions, bundling period and SPS for ULBC | RAN2 | To: SA4 |
| 6 | S4-260014 | Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC | RAN2 | CC: SA4 |
| 7 | S4-260017 | Response LS on the RAN simulation assumptions for ULBC | RAN4 | To: SA4 |
| 8 | S4-260020 | Reply LS on the RAN simulation assumptions for ULBC | SA2 | To: SA4 |
| 9 | S4-260021 | LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC | SA2 | To: SA4 and others |
| 10 | S4-260023 | Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC | SA3 | CC: SA4 |
| 11 | S4-260018 | Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC | SA1 | CC: SA4 |

# 2. Analysis of the reply LS’s

# For clarity, the analysis and proposals are in red.

# 2.3 #3 S4-260011, RAN1 To SA4

# This one was analyzed in [1].

# Recommended handling: noted.

# The RAN1 reply LS is as follows, and the analysis is in red.

Q1: SA4 kindly asks RAN1 to confirm the evaluation assumptions in the attachment, and provide feedback, if any.

RAN1 reply to Q1:

On the evaluation assumptions, RAN1 generally agrees with the overall set of parameters selected by SA4, with the following comments:

- If SA4 wants to evaluate 40ms bundling, RAN1 specifications may support this case by assuming 15kHz SCS (single and multi-tone) in the uplink. It is up to SA4 whether to consider this case in their evaluations.

- Although the example Figure 5.2.2.3-1 is supportable by RAN1 specifications in most scenarios, it may not be supportable in the case where the cell is very large (e.g. >3000km), when the UE does not support TA report and the network does not support UE-specific K-offset. The example Figure 5.2.2.3-1 itself also requires the UE to be configured with two HARQ processes and with HARQ feedback disabled.

A proposed pCR S4-260215 for the PD was submitted to this meeting, and the handling of this feedback can be based on the decision of S4-260215.

where  is receive antenna gain,  is noise figure,  is ambient temperature,  is antenna temperature, and  is the received antenna gain.

# Recommended handling: noted

# Proposal 1: consider

# correcting the sentence on DL CNR in PD v0.5.0 as proposed in S4-260146_FS_ULBC_P-doc_v0.5.0_QC.docx

2.4. S4-260012 RAN1 CC: SA4

RAN1 has discussed the following question from SA2:

Question 2 (To RAN1): Can RAN1 provide any data regarding the probability that such number of consecutive packets e.g. 16 or 64 can be lost or erroneously decompressed? How often such event can occur?

RAN1 reply:

Assuming line-of-sight condition for the duration of the call, no change in large scale parameters (e.g. fixed shadowing), and using an NTN TDL-C channel model (which is a channel model previously used by RAN1 and SA4 evaluations), the probability of having 16 or 64 packets consecutively lost is negligible under typical operating conditions (e.g. up to 10% packet error rate)

The probability of 16 or 64 consecutive packet losses is negligible under typical operating conditions.

It doesn’t say whether the probability of fewer than 16 or 64 consecutive packet losses is also negligible. In fact, as shown in [3], the probability of fewer consecutive packet losses may not be negligible.

For the case of line-of-sight not being guaranteed for the duration of the call, an intermittent blockage (e.g. according to the land mobile satellite channel model in TR 38.811) may still occur in practice, in which case the probability of having 16 or 64 packets consecutively lost may not be negligible. RAN1 has not reached consensus on whether to consider this case.

# Impact on SA4: depends on SA2 design.

# Recommended handling: noted.

# 2.5 S4-260015 RAN2 To: SA4

RAN2 would like to thank SA4 for the LS. RAN2 would like to provide the following response.

Question 1: the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option.

RAN2 Answer:

For transmitting voice packets via the UP solution RAN2 expects 1 byte for PDCP + 1 byte for RLC UM + the MAC header size.

For transmitting voice packets via the CP solution RAN2 expects 2 bytes for RRC + 2 bytes for RLC AM (the only mode currently specified for CP) or 1 byte for RLC UM (if RLC UM will be introduced for SRB in case a CP based solution will be selected) + the MAC header size.

The CP solution has been dropped by SA2 and this is now irrelevant. There was a proposal S4-260150 submitted to this meeting to address this.

RAN2 may provide further feedback on the expected average RoHC header size later.

Question 2: specifically, whether a packet overhead of 1 byte of MAC header is realistic.

RAN2 answer:

RAN2 assumes the MAC header size may be 1 to 3 bytes. However, in most common scenario, the total MAC header size is likely to be 3 bytes.

MAC header size may be 1 to 3 bytes. SA4 can ask under what condition 1 byte can be used.

Regarding question on whether the 120ms and/or 240ms bundling periods would be valid for the SPS operation in NB-IoT NTN, RAN2 has not started work on SPS for voice but thinks that SPS periodicities like 120ms and 240ms periodicities that do not divide 10240 would require additional specification work in RAN2 to resolve the issues.

SA4 cannot assume 120ms or 240ms bundling periods.

# Impact on SA4: the decisions affect the packet overhead calculation and bundling periods.

# Proposal 2:

- SA4 asks RAN2 under what conditions a 2-bytes or even 1-byte MAC header can be used respectively.

- SA4 will discuss whether supporting 120ms SPS periodicity is essential, and if so SA4 can ask RAN2 if it is feasible to do the specification work for supporting 120ms.

# 2.6 S4-260014 CC: SA4

RAN2 thanks SA2 for the LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC. RAN2 has discussed the relevant issues, and would like to provide the following replies:

Question 1 (To RAN2): Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC?

RAN2 reply to Question 1: RAN2 is not able to make any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC, and thinks the implementation of ROHC functions (e.g., compression and decompression) falls outside of RAN2 scope.

Question 3 (To RAN2): Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically?

RAN2 reply to Question 3: Scheduling methods may handle the voice packets of different sizes (e.g., via semi-persistent scheduling and dynamic scheduling, including over-provisioning of resources in UL), but RAN2 will further study and discuss the details of the scheduling mechanism to avoid the potential issues.

Question 6 (To RAN2): Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20?

RAN2 reply to Question 6: RAN2 understands that it is technically feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20.

Question 9 (To RAN2): Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media?

Question 10 (To RAN2): If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example: in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media?

RAN2 reply to Question 9 and 10: RAN2 understands it is technically feasible to introduce SRB with RLC UM for voice packet.

RAN2 also discussed the CP and UP solution for the support of IMS voice over NB-IoT NTN. Majority of companies in RAN2 prefer the UP over CP solution (because of the higher impacts on RAN2 specs of the CP solution). RAN2 also understands that the final decision will be taken at the checkpoint in December plenary.

# Impact on SA4: the reply has become outdated after SA2’s decision of not using CP to transport voice packets.

# Recommended handling: noted.

# 2.7 S4-260017 RAN4 To: SA4

Question from SA4: What are the supported power classes for UE and HPUE for NB-IoT NTN in the current release, and if any updates are expected from future releases, and in which band(s)?

Answer from RAN4:

From Rel-18 onwards, PC3 (23dBm) and PC5 (20 dBm) are supported for NB-IoT NTN UE on bands 256, 255, 254 and 253, and from Rel-19 onwards on band 252.

In Rel-19, RAN4 are introducing new power classes for NB-IoT NTN UE including PC1 (31dBm) and PC2 (26dBm) for bands 256, and 255.  The target device types include both hand-held and non-handheld device types.

In Rel-20, RAN4 is expected to discuss the feasibility of introducing new power class(es) for NB-IoT NTN operation.  RAN plenary has already approved a work item for Rel-20 (RP-251867), where one of the objectives is to study, and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37 dBm) for NB-IoT NTN. The feasibility and specific power would be under further study and this work has not been started yet. There are also no designated bands yet for this higher than PC1 study.

The current values 23dBm, 26dBm, and 31dBm in the PD are confirmed.

Impact: this affects the UL CNR calculation.

Recommended handling: noted.

Proposal 3: Update the PD with a note that “Higher maximum UE transmission power (e.g., up to 37dBm) is FFS pending the outcome of the RAN4 study RP-251867 expected to be completed by March 2027.”

# 2.8 S4-260020 SA2 To: SA4

SA2 would like to thank SA4 for the “LS on the RAN simulation assumptions for ULBC”.

Please find answers to the questions related to SA2:

SA4 kindly asks SA2 and RAN2 to comment on

- the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and

[SA2 answer]: SA2 has documented a number of alternative solutions in TR 23.700-19 for “Key Issue #1: Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC” that cover various options related to a) using User Plane or Control Plane CIoT EPS Optimisation and b) IP or non-IP PDN type. SA2 has not yet concluded on the solutions for Key Issue #1. The final protocol overhead depends on the selected solutions.

The conclusion on all these issues and the estimated overhead is expected to be provided later. The completion date of the FS_5GSAT_Ph4_ARC SID is December 2025 (SA#110).

- specifically, whether a packet overhead of 1 byte of MAC header is realistic.

[SA2 answer]: SA2 believes RAN2 should answer this question and also provide feedback on the PDCP, RLC and MAC overhead expected.

Impact: no action for SA4 now.

# Recommended handling: noted

# Proposal 4: Check paper S4-260150 and confirm its accuracy in capturing SA2’s decision on the schemes for transporting IMS voice packets IMS over NB-IoT NTN connected to EPC.

# 2.9 S4-260021 SA2 To: SA4 and others

# 4. Analysis of SA2 reply LS

Question 1 (To RAN2): Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC?

Question 2 (To RAN1): Can RAN1 provide any data regarding the probability that such number of consecutive packets e.g. 16 or 64 can be lost or erroneously decompressed? How often such event can occur?

Voice packets of different sizes can be observed over the NB-IoT NTN link e.g. due to talk/silence transitions, or occasionally RoHC state change, etc. The size difference of voice packets during talk/silent transition is described in S2-2506371/S2-2507021/S2-2507102 (neither endorsed nor agreed).

More information on the RoHC related issues is described in S2-2506371, S2-2507021 and S2-2507102 that are neither endorsed nor agreed by SA2.

Question 3 (To RAN2): Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically?

In the context of using "Non-IP" for voice media traffic there is no  header compression. To reduce the header overhead, it is necessary to understand which parts of the RTP header, e.g., sequence number, timestamp, SSRC (Synchronization Source ID), Payload Type (PT) is essential to the receiver support IMS voice over NB-IOT NTN.

Question 4 (To SA4): What are the essential RTP header fields for the minimum information that needs to be provided in the RTP header for IMS voice over NB-IoT NTN?

SA4 needs to discuss this and respond.

Question 5 (To SA4): Does SA4 have any views on how to reduce the RTP header overhead for IMS voice over NB-IoT NTN?

SA4 needs to discuss this and respond.

SA2 has observed that RAN2 has enforced the limitation that the capability of a NB-IoT UE supports a maximum 2 DRBs. In order for SA2 to reach a conclusion to use the user plane and DRBs to support IMS voice, SA2 would like to understand whether NB-IoT Access Stratum protocols can support more than 2 DRBs (e.g. 3 DRBs) in Rel-20, then the UE can support simultaneous voice and other services even during the call.

Question 6 (To RAN2): Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20?

In addition to user-plane based solutions to carry voice media, several other proposed solutions under Key Issue #1 (Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC) rely on control-plane-based mechanisms, where Control Plane CIoT EPS optimisation and SRBs are used to transport SIP signaling and/or voice media.

For this category of “control plane solutions”, SA2 understands that CT WG1 in rel.19 under the WI NORDAT_CP specified a new NAS message for data transfer over NAS with a reduced NAS layer overhead of 2 bytes, NAS layer security overhead of 4 bytes for MAC (integrity protection) and 1 byte for SN (Total = 7 bytes). SA2 is evaluating whether it is possible to reduce the overhead further. Specifically SA2 has the following questions:

Question 7 (To SA3): Considering that in the context of alternative solutions documented in TR 23.700-19 for the support of IMS voice over NB-IoT NTN connected to EPC, a specific SRB (i.e. via a dedicated EPS bearer for Data over NAS) will be used for transfer of voice media packets only, is there a concern to eliminate the 5 bytes of NAS layer security overhead?

Question 8 (To CT1): In addition to question 7 related to security, is there any further possibility to reduce the NAS overhead further than what the rel.19 NORDAT_CP WI provided in order to transport voice packets over NB-IoT NTN?

Currently, SRBs are only supported over RLC Acknowledged Mode (AM) (TS 36.331), and the possible impacts of this are discussed in S2-2507107, which is not endorsed or agreed by SA2.

In order for SA2 to be able to evaluate between user-plane and control-plane solutions to carry voice media, the use of RLC Unacknowledged Mode (UM) for SRBs, SA2 has the following question:

Question 9 (To RAN2): Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media?

Question 10 (To RAN2): If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example: in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media?

According to TS 22.228, it is possible for the UE to support more than one IMS voice calls e.g. simultaneously using the communication HOLD service that enables a user to suspend media within a session and resume that media at a later time.

In addition, SA2 observed that Dual Tone Multi-Frequency (DTMF) signalling defined in TS 23.014 and TS 26.114 Annex G would also require DTMF events as telephone events within the same RTP media stream as the speech, i.e. "inband", using the same IP address and UDP port as the RTP for speech, but using a different RTP Payload Type number.

These requirementswill have an effect on the overall protocol stack overhead for a non-IP based solution to support IMS voice over NB-IoT NTN.

SA2 assumes that requirements for multiple calls and DTMF still apply to support of IMS voice over NB-IoT NTN bus since these are service requirement, SA2 would like verification from SA1.

Question 11 (To SA1): Can SA1 confirm, whether support for more than one IMS voice calls and support for DTMF is required for IMS voice over NB-IoT NTN?

# Impact on SA4: SA4 needs to answer Questions 4 and 5.

# Proposal 5:

# SA4 sends an LS to SA2 and notifies SA2 that SA4 will study the essential fields in the RTP header and how to reduce the RTP header size.

# 2.10 S4-260023 SA3 CC: SA4

SA3 thanks SA2 for the LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC (S3-253118/S2-2507636).

SA3 has discussed the following question:

Question 7 (To SA3): Considering that in the context of alternative solutions documented in TR 23.700-19 for the support of IMS voice over NB-IoT NTN connected to EPC, a specific SRB (i.e. via a dedicated EPS bearer for Data over NAS) will be used for transfer of voice media packets only, is there a concern to eliminate the 5 bytes of NAS layer security overhead?

Answer: The majority of companies in SA3 were not in favour of allowing non-integrity protected voice packets to be sent over NAS to the MME. Hence there was no consensus in SA3 to eliminate the 5 bytes of NAS layer security overhead.

# Impact on SA4: this has become irrelevant given SA2’s decision of sending voice packets in the UP and no action is needed from SA4 now.

# Recommended handling: noted.

# 2.11 S4-260018 SA1 CC: SA4

SA1 thanks sa2 for their LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC, and the question addressed to SA1:

Question 11 (To SA1): Can SA1 confirm, whether support for more than one IMS voice calls and support for DTMF is required for IMS voice over NB-IoT NTN?

SA1’s view is that

- support for more than one (simultaneous) IMS voice call is not required in this Release.

- support for Dual-Tone Multi-Frequency (DTMF) is required.

The above is clarified in the attached CR.

# Impact on SA4: SA4 needs to consider DTMF in addition to voice packets. There were a three proposals from S4-260217 submitted to this meeting.

# Proposal 6: review the proposals in S4-260217 and decide what to do to support DTMF in SA4.

# Recommended handling: noted.

# 3. Proposals

# Proposal 1:

# consider correcting the sentence on DL CNR in PD v0.5.0 as proposed in S4-260146_FS_ULBC_P-doc_v0.5.0_QC.docx

# Proposal 2:

- SA4 asks RAN2 under what conditions 2-bytes or even 1-byte MAC header can be used respectively.

- SA4 will discuss whether supporting 120ms SPS periodicity is essential, and if so SA4 can ask RAN2 if it is feasible to do the specification work for supporting 120ms.

Proposal 3: Update the PD with a note that “Higher maximum UE transmission power (e.g., up to 37dBm) is supported pending the outcome of the RAN4 study RP-251867 expected to be completed by March 2027.”

Proposal 4: Check paper S4-260150 and confirm its accuracy in capturing SA2’s decision on the schemes for transporting IMS voice packets IMS over NB-IoT NTN connected to EPC.

# Proposal 5: SA4 sends an LS to SA2 and notifies SA2 that SA4 will study the essential fields in the RTP header and how to reduce the RTP header size.

# Proposal 6: review the proposals in S4-260217 and decide what to do to support DTMF in SA4.

# References

# [1] S4aA250139, “Analysis and recommended handling of the reply liaisons from other working groups to SA4 on ULBC,” Qualcomm, 3GPP SA4 Audio Ad Hoc meeting, online, Dec 2, 2025.

# [2] S4-251953, “Analysis of the reply liaisons from other working groups to SA4 on ULBC”, Qualcomm, 3GPP SA4  #134 Meeting, Dallas, Nov 17-21, 2025.

# [3] R1-2507687, “IMS voice over NB-IoT NTN,” Qualcomm, 3GPP TSG RAN WG1 #122, Prague, Czech Republic, October 13th – 17th, 2025.

# [4] S4-260018, “Reply LS on issues related to support of IMS voice over NB-IoT NTN

# connected to EPC”, 3GPP TSG SA WG 1 Meeting #112, Nov, 2025.