Source:	Editor*

Title:	DaCAS-1: Target devices and databases v0.4

## Document for:	Discussion & Agreement

## Agenda Item:	7.6

Revision history:

| Date | Meeting | Subject/Comment | Old | New |
| --- | --- | --- | --- | --- |
| 2025-05-22 | SA4#132 | Initial version including S4-251057 | N/A | 0.1 |
| 2025-06-16 | Audio SWG AH Meetings Post SA4 #132 Telco on DaCAS | Addition of 3 mic target device template excel sheet to Annex A | 0.1 | 0.2 |
| 2025-07-22 | SA4#133-e | Incorporating proposals from S4-251328, S4-251367 S4-251371, S4-251374, S4-251381 and editorial work | 0.2 | 0.3 |
| 2025-11-19 | SA4#134 | Add ISM recordings setups and scenarios | 0.3 | 0.4 |

- 1 Introduction

The DaCAS work item [1] aims to design example immersive audio capture solutions for target UE devices, enabling the development of end-to-end solutions for IVAS audio applications. To address the diversity of user equipment (UE) designs and acoustic requirements, proponents have proposed some target device types in [1] and [2]. The agreed potential target devices can be divided into two categories:

- Category 1: Prototype configurations for theoretical validation and possible future device deployment.

  - Device A serves as an idealized accessory device with free-field approximation capabilities, enabling algorithm development without device-specific tuning.

  - Device B represents a practical smartphone design with a non-planar microphone array, highlighting the need for tuning in complex geometries.

- Category 2: Commercial smartphone-based designs for possible legacy device deployment. Complementing the prototypes, proposal of mainstream smartphone configurations to ensure example solutions align with current market available devices.

This permanent document proposes a framework for describing the target devices which are used to collect corresponding example solutions.

- 2 Content

[

## 4.2	Device Types

see S4-251027-TS 26.533-Ver0.1.0

Note: The coordination system needs optimization.

## 4.3	Database

### 4.3.1	Principle of scenarios and database design

The following should be considered when designing the databases:

#### 4.3.1.1	Recording scenario types

There are three recording scenario types proposed as follows:

Device characterization

- Recordings targeted to identify electro-acoustical characteristics of proposed target devices. For example, to identify device frequency response, etc.

Development recordings

- Recordings targeted to facilitate  algorithm development for converting raw/compensated microphone signals into at least one IVAS encoder input format and potential tuning.

Evaluation recordings

- Recordings targeted to evaluate example solutions, including subjective and objective evaluation. For objective evaluation, alignment with TS 26.260 is expected.

Above categories should not be understood as strictly orthogonal scenarios, as it is expected that certain scenarios/recordings may be useful for multiple purposes. Furthermore, while other recording scenario types may be anticipated than what is presented here, it is seen useful to consider whether and how the proposed recording scenarios would fit into the above recording scenario types.

#### 4.3.1.2	Recommendations on defining recording scenarios

Along the above recording scenario types, different recording scenarios for different purposes are anticipated. To ensure definition of meaningful and useful recording scenarios, at least following aspects should be considered when defining new recording scenarios:

- Varying recording scenarios should be represented, from simple single source scenarios to complex scenarios, and from free-field conditions to reverberant conditions.

- For the sake of reproducibility, high quality loudspeakers with well-defined source signals should be favored when possible.

- For device characterization, algorithm development, and objective evaluation purposes, controlled rooms in terms of reverberation and room frequency response should be favored. Referencing existing specifications is recommended, if applicable.

- For subjective evaluation purposes, vast number of varying environments should be represented. Number of proposed recording scenarios in indoor and outdoor environments should be in balance.

- Additional care should be taken when describing and defining the positioning of sound sources and UE.

#### 4.3.1.3	The relationships of the three templates

The relationships of the three templates are illustrated in Figure 4.3.1.3-1 below. Furthermore, cross-referencing between the databases should be favored in general, in order to reduce redundancy.

Target device database

Target device No

Type

Size

Shape

…

Recordings database

Recording item No

Target device No

Rec scenario No

UE orientation

…

Recording scenarios database

Recording scenario No

Sound source

Signal characteristics

Playback level calibration

…

Figure 4.3.1.3-1 – Templates/databases relationship

### 4.3.2	Recording setups and scenarios

The recording databases are described in attached file ‘DaCAS recordings setups and scenarios.xlsx’ file.

### 4.3.3	Recording database

#### 4.3.3.1	Requirement for database

The following should be considered when recording the databases:

- The raw microphone signals must be recorded, i.e., any noise reduction or other audio pre-processing of the DUT must be disabled or by-passed

- The turntable, stand, and DUT mounting should have only minimal acoustic effects (e.g., no microphones should be blocked, minimal acoustic footprint)

- The microphone channel ordering of the DUT, the DUT orientation, the turntable rotation direction, and the 0° position must be documented (if possible, with photos)

- The source signals shall be provided and the characteristics of the source signals shall be documented in the database

#### 4.3.3.2	Mandatory recording scenarios

The following recording scenarios shall be provide for each target device to guarantee the development and evaluation for example solutions:

TBD

#### 4.3.3.3	Database for target devices

All the recording databases are described in attached file ‘DaCAS recordings database.xlsx’ file.

#### 4.3.3.3.1	Database for Four-Microphone Prototype device

Full database is available in[12]:

https://deveo.partner.nokia.com/TECH/projects/nokia_dacas_databases/repositories/webdav/4mic-proto-device-A-and-device-B

Database access: Access to the database requires an account. One account will be provided per company upon request. Please request the account credentials from the database source.

#### 4.3.3.3.2	Database for Three-Microphone Smartphone device:

Full database is available in [16] S4-251328 Link:

https://www.3gpp.org/ftp/tsg_sa/WG4_CODEC/TSGS4_133-e/Docs/S4-251328.zip

Note: These databases are provided by simulation.

#### 4.3.3.3.3	Database for Four-Microphone Smartphone device 1

Full database is available in[13]:

Link:

https://kpan.mioffice.cn/webfolder/ext/zh%24DE%24XejVH%24uVm31GQvyw%40%40?n=0.1314974258223881

password: 0x7o

4.3.3.3.4	Database for Four-microphone Smartphone device 2:

Full database is available in [14] S4-250821 Link:

https://www.3gpp.org/ftp/TSG_SA/WG4_CODEC/TSGS4_132_Fukuoka/Docs/S4-250821.zip

Note: These databases are provided by simulation.

#### 4.3.3.3.5	Database for Three-Microphone XR HMD device:

Full database is available in [15] S4aA250066 Link:

https://www.3gpp.org/ftp/TSG_SA/WG4_CODEC/3GPP_SA4_AHOC_MTGs/SA4_Audio/Docs/S4aA250066.zip

Note: These databases are provided by simulation.

]

- 3 Conclusion

This permanent document is used to track the progress of the target devices , scenarios , database and template for DaCAS.

## References

- S4aA250006: Proposal of a target device

- S4-250745: Proposal of new potential target devices for DaCAS

- S4-251027:TS 26.533-Ver0.1.0

- S4aA250016:[DaCAS] update of Device Definition template

- S4-250823:Exponential sweep database for target device 1

- S4-250654: On DaCAS recording scenarios_r2

- S4-250856:Update DaCAS templates to v2.1

- S4-250962: Proposal on DaCAS Acoustic Properties Measurements

- S4-250567: [DaCAS] target device type proposal

- S4aA250017 [DaCAS] Prototype Device B template upd

- S4-250642: “[DaCAS] Proposed DaCAS target device”, Dolby Laboratories Inc

- S4-250947: Database for Four-microphone prototype device A and B

- S4aA250065: Database of target device 1 for scenario 1 and 2

- S4-250821: Recordings of smartphone-like target device

- S4aA250066: Recordings for XR HMD target device

- S4-251328: Recordings for Three microphone Smartphone Device

- Annex <A>(Attached files)

Template for potential target devices.xlsx[7]

DaCAS recording setups and scenarios.xlsx

DaCAS recordings database.xlsx

DaCAS_Device_Definition Four-microphone prototype device B.xlsx[10]

DaCAS_Device_Definition_supplement_template.xlsx[10]

DaCAS_Device_Definition 3_mic_smartphone.xlsx [2] and [11]

Target devices - Four-mic prototype device A and device B.xlsx[9]