3GPP SA4-MBS SWG #134	S4-251849

# Dallas (TX, US), 17-21 November 2025

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0012 | rev | 0 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Solution for KI4 on Energy-related configuration by the Application Service Provider for media delivery services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics Co., Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of a solution to Key Issue 4 on Energy-related configuration by the Application Service Provider for media delivery services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a clause 7.1x adding a a solution to on Key Issue 4 on Energy-related configuration by the Application Service Provider for media delivery services |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No solution proposed for the KI. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1, 7.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

2nd Change

## 7.1	Mapping of Solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solutions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| #1 |  |  | X |  |  |  |
| #2 |  |  | X |  |  |  |
| #3 |  | X |  |  |  |  |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  |  |  |  |
| #6 | X | X |  |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 | X |  |  |  |  |  |
| #10 |  |  |  | X |  |  |

These solutions are simply candidate solutions. Their inclusion in the following clauses does not imply that they have been agreed upon or endorsed. Any decisions and work to be done for the normative work will be detailed in the conclusions of this Technical Report.

3rd change
(All new text)

## 7.1x	Solution #1x: Energy event driven media service degradation

### 7.1x.1	Key Issue mapping

This Candidate Solution addresses Key Issue #4.

### 7.1x.2	Functional description

#### 7.1x.2.1	Introduction

Whilst there are media streaming features related to QoS, such as network assistance and dynamic policies, these features do not take into account energy considerations related to either that of the network, or that of the media itself.

By using energy-related information about the network obtained via the Energy Information Function (EIF), the 5GMS AS and RTC AS may be able to optimise media sessions by means of energy-driven service degradations. Since fluctuations in network energy consumption and the corresponding energy-related information originates in the EIF, a pragmatic approach to energy-driven service degradations would be driven and triggered by the 5GMS AF or RTC AF through the knowledge of such energy information.

A simple degradation example may be the reporting or notification of the degraded QoS by the 5GMS AF or RTC AF to the UE (similar to that used in dynamic policies), but in this case the UE is unable to distinguish QoS degradation due to server downtime, network connection problems, sudden congestion, or other factors. If a QoS degradation is the result of energy-related aspects leading to a service policy change applied to that specific UE, then this degradation cannot be resolved regardless of how the UE may attempt to use different Application Servers (e.g. multiple DASH servers). By providing additional information to the UE regarding the energy-driven service degradation, the UE may then be able to make a best effort reaction decision to the service degradation.

This Candidate Solution introduces a mechanism for allowing the Media AF to notify the Media Client in the UE of an energy-driven service degradation in the form of energy event information, enabling the Media Client to make informed decisions about adapting media delivery in reaction to the energy-driven QoS degradation event.

#### 7.1x.2.2	Reference architecture for energy event driven media service degradation in the generalised Media Delivery System

Figure 7.1x.2.2-1 depicts a reference architecture that realises this candidate solution in the generalised Media Delivery architecture defined in TS 26.501 [23] and TS 26.506 [59].

Figure 7.11.2.2-1: Reference architecture for energy event driven media service degradation in the generalised Media Delivery System

The core principle of this architecture is to leverage the existing methods for the delivery of energy event information from the Media AF to the Media Client. Energy information collected from the Energy Information Function by the Energy Information AF is used to determine any necessary QoS degradation decisions, the result of which may also trigger the delivery of energy event information to the Media Client.

The solution reuses a function already defined for collection and exposure of energy-related information to the UE:

-	The Energy Information AF instantiated in the Media AF.

The solution reuses the existing reference points defined in the generalised Media Delivery architecture, as well as reference point E12 already defined for the exposure of energy-related information by the EIF to the in the instantiation of solution #5 in clause 7.6.

#### 7.1x.2.3	Energy Event Information

This clause specifies energy event information for energy-driven service degradation. Energy event information is generated by the Energy Information AF based on the network energy status (information from the EIF) as well as the policy information for the corresponding UE, and is delivered to the UE Media Client.

Table 7.1x.2.3: Baseline Energy Event Unformation

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |
| Energy-degraded bit rate | The degraded bit rate value of the current media delivery session due to energy impacts (see NOTE). |
| Original bit rate without degradation | The bit rate value which can be achieved by the media delivery session if the system’s energy situation improves, or if energy restrictions are lifted for the client. |
| DPredicted duration or end time of degradation | Indicates the duration or end time of the QoS degradation currently applied to the client. If the network does not know the end time, the end time may continue to increase in subsequent events, or an unknown duration may be specified. |
| Scope of degradation | Indicates the scope of the QoS degradation. The attribute may specify one or more values indicating the UE, user, cell, AS or network. Depending on the scope specified, the client and/or user may be provided with additional information regarding the nature of the energy QoS degradation. For example: - When specified as UE or user, the temporary network performance degradation applies only to the current device or the user’s subscription plan, allowing the user to use other companion devices or understand why neighbours are not experiencing the same performance degradation. - When specified as cell, the performance degradation may resolve when moving to an area served by a different cell. - When specified as service location, the performance degradation applies to the current service location and the Media Client may switch to a difference service location. - When specified as network, the device user can understand that the policy-based performance degradation applies to all mobile subscribers. |
| Degradation cause | Indicates whether the QoS degradation is due to network-to-device transmission or the processing consumed by the network server (e.g. Media AS). |
| NOTE: The degraded bit rate value may or may not be the same as the recommended bit rate made known to the Media Session Handler at reference point M5 via Network Assistance notifications, which takes into account other factors known to the Media AF, such as network congestion. |  |

### 7.1x.3	Procedures

Figure 7.1x.3-1 below details the different steps for energy event driven media service degradation based on the functional description and architecture outlined in clause 7.1x.2.

Figure 7.1x.3-1: Procedures for client-driven selection of stream variants or delivery paths based on energy characteristics in the generalised Media Delivery System

1.	The Service Announcement procedures between the UE and network includes the exchange of Service Access Information.

2.	Media session handling is set up at reference point M5 between the Media Session Handler and the Media AF.

3.	During the setup in step 2, the UE may request and subscribe to energy event information (EEI) events related to energy-based media service degradation. Other features such as Network Assistance and Dynamic Policy may also be requested.

4.	Network-related energy information is collected by the Energy Information AF and the Energy Information Function.

5.	Policy information specific to the UE (in particular related to energy) may also be collected by the the Energy Information AF.

6.	Energy information related to the processing of media contents (such as encoding) may also be collected from the Media AS by the Energy Information AF.

7.	Using the energy information collected in steps 4 to 6, the Energy Information AF instantiated in the Media AF generates Energy Event Information (EEI).

8.	The latest EEI is prepared by the Energy Information AF.

9.	The decision of whether to send and when to send the EEI to the client UE is determined by the Energy Information AF. This decision may depend on multiple factors including any change in the degraded bit rate, or other mechanisms such as periodic notification.

10.	EEI is delivered between the Media AF and the Media Session Handler.

11.	Upon receiving the EEI, the Media Session Handler may use the information to decide on a QoS degradation response.

12.	The response decision and corresponding media instruction response is made known to the Media Access Function.

13.	The Media Access Function initiates the media level response to the QoS degradation with the Media AS.

### 7.1x.5	Summary

This candidate solution proposes a mechanism to enable energy driven media service degradation, in particular:

1.	The collection of network energy information and media-related information in the network by the Energy Information AF. This information is used to generate energy event information for describing any energy related service degradations for a particular client device.

2.	Targeted energy event information is shared with the client device in order to aid the client in making any media-session related responses to service degradation.

This approach compliments the network driven nature of network energy fluctuations, whilst also enabling the consideration of specific user device energy-related policies as well as energy aspects related to media related processes.

The solution aims to provide such information for the client to differentiate service degradations due to energy impacts as opposed to other existing factors such as traffic congestion, whilst at the same time encouraging the use of already existing reference points and features to complexity.

End of changes