3GPP TSG|WG-SA4 Meeting #134 	S4-251734

Dallas, TX, US, 17-21 November 2025	(revision of xx-yyxxxx)

# Source:	SA4

# Title:	New WID on IMS DC Enhancements

# Document for:	Approval

# Agenda Item:	xxx

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: DC Enhancements

Acronym:	DCEnh-MED

Unique identifier:

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
| X | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification:

# 3	Justification

Release 19 introduced several stage-2 architecture enhancements for the IMS Data Channel (DC) in TS 23.228 Annex AC, defining the roles of the DCSF, MF, DC Application Server, and associated procedures such as bootstrap and application DC setup, standalone DC sessions, and multiplexing across SCTP associations.

However, several stage-3 features in that architecture remain incomplete in TS 26.114, leading to recurring interoperability questions and liaison requests from SA2, SA3, SA3-LI, and GSMA. In particular, SA2 Rel-19 procedures for standalone DC establishment (AC.10) and multiplexing (AC.7.10) assume explicit SDP conventions for signaling application availability, partial acceptance, and de-multiplexing fallback, yet TS 26.114 lacks comprehensive support and examples for these features. This results in non-aligned implementations for scenarios such as “application download pending” on the terminating UE, partial acceptance of multiplexed m-lines, and re-negotiation.

At the same time, Rel-19 introduced new functional models, MF as HTTP Proxy, UDP Proxy, and DC Application Proxy (AC.6) and the interworking hooks with MTSI (AC.7.9), that depend on clear runtime mapping between DC streams and MF-anchored RTP streams. SA4 stage-3 text still lacks normative conventions on how a DC application declares such mapping in SDP or how direction attributes (a=inactive/sendrecv) are applied for DC hold/resume in interworking cases. Similarly, the Rel-19 architecture allows HTTP-based application retrieval and potential interoperation with external content sources, but TS 26.114 leaves the treatment of external URLs, origin policies, and network provided whitelists undefined. Without a generic framework for policy signaling at the SDP and runtime levels, networks cannot enforce consistent behavior for HTTP subprotocols.

Consequently, this work item aims to ensure full stage-2 to stage-3 alignment for the IMS Data Channel feature set introduced in Rel-19. Furthermore, it will address any unclarities and feature enhancements identified by GSMA and other groups. These enhancements will close the open Rel-19 gaps in TS 26.114, improve interoperability across vendors and networks, and create a stable baseline for subsequent service-specific extensions (e.g. AI/ML, AR and Avatar communication) that rely on DC mechanisms.

# 4	Objective

The Work Item will enhance TS 26.114 as follows:

- Define answering behavior for Standalone DC when the app is not yet available

- Complete multiplexing edge-cases (per AC.7.10)

  - Clarify multi-application multiplexing when an m-line combines several a=3gpp-req-app with mixed endpoint types (…-UE and …-Server) and heterogeneous QoS hints

  - Add examples aligning with stage-2 procedures

- Interworking facilitation with MTSI

  - Provide SDP conventions (reusing existing attributes) for associating a DC application stream to MF-anchored RTP legs when interworking is invoked per AC.7.9

  - Include informative examples consistent with AC.7.9 flows

- Minimal, generic runtime policy signaling for HTTP on DC

  - Introduce a generic signaling mechanism to carry network-provided origin/host policy hints that a DC application may follow -

  - Provide normative text guiding DC application behavior for the HTTP subprotocol

- Consolidate Rel-19 additions from 23.228 and address other identified ambiguities and features from other groups, such as GSMA

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 26.114 | CR to address identified enhancements | SA#112 |  |

# 6	Work item Rapporteur(s)

Bouazizi, Imed, Qualcomm Inc., bouazizi@qti.qualcomm.com

# 7	Work item leadership

SA4

# 8	Aspects that involve other WGs

None.

# 9	Supporting Individual Members

Supporting IM name

Qualcomm Inc.

Ericsson LM