3GPP TSG-SA4 Meeting #134	S4-251731

# Dallas, TX, USA, 17 – 21 November 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.264 | CR | 0007 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | OpenAPI specification for Base Avatar Management Interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AvCall-MED |  |  |  |  |  | Date: |  |  | 2025-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Avatar Call feature has defined a BAR interface for the management of base avatars that are hosted by the BAR. The interface is currently missing the normative Open API specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR defines the normative Open API specification for the different APIs of the BAR interface. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The nomrative Open API specification of the BAR APIs will be missing and compliance will not achievable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, Annex C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[3]	3GPP TS 26.119: "Media Capabilities for Augmented Reality".

[4]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[5]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[6]	3GPP TS 26.565: "Split Rendering Media Service Enabler".

[7]	ISO/IEC 23090-14 AMD 2, Information technology — Coded representation of immersive media — Part 14: Scene description — Amendment 2: Support for haptics, augmented reality, avatars, Interactivity, MPEG-I audio, and lighting

[8]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[9]	3GPP TR 26.813: Avatar Representation and Communication.

[10]		ISO/IEC DIS 23090-39, Information technology — Coded representation of immersive media — Part 39: Avatar Representation Format.

[11]	IETF RFC 9110: "HTTP Semantics".

[12]	IETF RFC 9112: "HTTP/1.1".

[13]	OpenAPI, "Open API specification", Online: https://spec.openapis.org/oas/v3.0.0.

[14]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition".

[15]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[16]	IETF RFC 3611, "RTP Control Protocol Extended Reports (RTCP XR)".

[17]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[18]	IETF RFC 9113: "HTTP/2".

[19]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[20]	IETF RFC 8259, "The JavaScript Object Notation (JSON) Data Interchange Format".

==============Next change==============

### B.1.7.3	Data model

#### B.1.7.3.1	Associated information resource

Table B.1-9: Definition of Associated Information resource

| Property name |  |  | Data type |  | Cardinality |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| associatedInfo |  |  | Object |  | 1..1 |  | A list of assets associated with the Base Avatar. |
|  | avatarId |  | ResourceId | 1..1 |  | A unique identifier assigned to a Base Avatar by the BAR on creation. |  |
|  | avatarMetadata |  | Object | 1..1 |  | Metadata related to the Avatar, |  |
|  | assets |  | array(Object) | 1..1 |  | A list of assets associated with the Base Avatar. |  |
|  |  | assetId | ResourceId | 1..1 |  | The identifier of the Asset resource. |  |
|  |  | assetLoDs | array(Object) | 1..1 |  | A list of available LoDs for the corresponding asset. The resulting size in bytes shall be associated with each LoD. NOTE 1: LoDs descriptions/labels and their associated complexity is FFS. NOTE 2: Clarification about how LoD would be described and what that would mean in terms of complexity is FFS. |  |
|  | selectionInfo |  | Object | 0..1 |  | Provides information that the user can use to select this avatar. This may contain a name, a nickname of the asset, usage context e.g. casual, work, and images of renditions of the asset. |  |
|  | supportedAnimations |  | array(string) | 1..1 |  | A list of the URNs that identify the supported animation frameworks by this base avatar. |  |
|  | infoUpdatedAt |  | number | 1..1 |  | A timestamp (in wall clock time) describing the time of the last update to the associated information for the corresponding Base Avatar. This field is updated whenever an asset is modified by the owner of the Base Avatar or BAR. Users may utilize the infoUpdatedAt field to verify the latest validity of previously downloaded Base Avatar data. Comparing the infoUpdatedAt value with the downloaded time allows users to determine if the downloaded data requires updating, |  |

==============Next change==============

Annex C (normative):
OpenAPI representation of HTTP REST APIs

# C.1	General

The normative code specifying the APIs defined in Annex B of the present document, including JSON Schema [20] representations of HTTP message bodies to be used with these APIs, is published on 3GPP Forge according to the OpenAPI 3.0.0 specification [13]. The YAML files corresponding to this version of the present document shall be published to the following location:

https://forge.3gpp.org/rep/all/5G_APIs/-/tags/TSG110-Rel19

Informative copies of these YAML files shall be distributed with the present document for convenience only. Where any discrepancy exists, the version on 3GPP Forge shall be considered definitive.

# C.2	Common Data Types

For the purpose of referencing entities specified in this clause, it shall be assumed that the OpenAPI definitions are contained in a physical file named "TS26264_CommonData.yaml".

# C.3	OpenAPI representation of Mbar_Management APIs

## C.3.1	Mbar_Management_Avatars API

For the purpose of referencing entities specified in this clause, it shall be assumed that the OpenAPI definitions are contained in a physical file named "TS26264_Mbar_Management_Avatars.yaml".

## C.3.2	Mbar_Management_Assets API

For the purpose of referencing entities specified in this clause, it shall be assumed that the OpenAPI definitions are contained in a physical file named "TS26264_Mbar_Management_Assets.yaml".

## C.3.3	Mbar_Management_AssociatedInformation API

For the purpose of referencing entities specified in this clause, it shall be assumed that the OpenAPI definitions are contained in a physical file named "TS26264_Mbar_Management_AssociatedInformation.yaml".

## C.3.4	Mbar_Management_AvatarRepresentations API

For the purpose of referencing entities specified in this clause, it shall be assumed that the OpenAPI definitions are contained in a physical file named "TS26264_Mbar_Management_AvatarRepresentations.yaml".

## C.3.5	Mbar_Management_Sessions API

For the purpose of referencing entities specified in this clause, it shall be assumed that the OpenAPI definitions are contained in a physical file named "TS26264_Mbar_Management_Sessions.yaml".

==============End of change==============