3GPP TSG- SA4 Meeting # 134	S4-251998

17-21 November 2025, Dallas, Texas, USA

Title:	Reply LS on URL for downloading the DC Application list

Response to:	S4-251676 (S6-254794)

Release:	3GPP Rel-20

Work Item:	FS_MMTel_Ph2_APP

Source:	3GPP TSG SA WG4

To:	3GPP TSG SA WG6

Cc:	3GPP TSG SA WG2

Contact person:	Bo Burman

bo <dot> burman <at> ericsson <dot> com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S4-252047 26.114 CR 0597r1

# 1	Overall description

SA4 thanks SA6 for their liaison on URL for downloading DC Application list in S4-251676 (S6-254794), and would like to provide the following answers to the questions in that document, for SA6’s information and further consideration:

Question 1:	Could SA4 clarify whether this URL can be modified to carry query parameters to support SA6's study on DC application list search?

Answer 1:	SA4 would like to emphasize that it is only the very first URL that is restricted to the root (“/”) URL and believes that such well-defined starting point is a desirable feature for security and trust purposes. SA4 further notes that such approach does not need to change to enable more advanced web functionality, for example searching.

Such restricted and well-defined starting point:

1)	does not require downloading a full list of applications, i.e., the returned content is not standardized and is not even in any way restricted to be an "application list" or “menu”

2)	is a web page with all the user interface, hyperlinking properties, and functional possibilities of a modern web page

3)	in current specification does not mandate that only a single URL is ever used for managing and selecting applications, but there may be multiple, subsequent HTTP requests/responses using other URLs (that are not restricted) sent over the same bootstrap data channel

4)	does not in any way prevent the returned "application list" content from being, e.g., a categorized, multi-level and/or search page that contains user interface constructs such as edit boxes, buttons, etc., that could trigger HTTP requests with other URLs, e.g., with query parameters to facilitate any desired search functionality

Question 2:	If the answer to Question 1 is No, then could SA4 update the above clause to support the use case of DC Application list search?

Answer 2:	SA4 believes that no functional update to current specification is necessary to enable the desired functionality, and it should already be possible to support the use case of DC Application list search, as described above. SA4 has updated TS 26.114 to make the above clearer and attaches the agreed CR.

# 2	Actions

To SA6:

ACTION:	SA4 kindly asks SA6 to take the above answers into account, which should address the questions in the LS, in our opinion. If not, SA4 requests SA6 to clarify their use cases further.

# 3	Dates of next TSG SA WG 4 meetings

SA4#135		9 – 13 February 2026 		Goa, India

SA4#135bis-e		13 – 17 April 2026		Online

SA4#136		11 – 15 May 2026		Montreal, Canada