3GPP TSG-S4 #135 Meeting	S4-260119

Goa, India, 9 - 13 February 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.958 | CR |  | rev |  | Current version: | 0.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR on glTF for GS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S4 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | YYYY-MM-DD |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | Adding glTF format description |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x1]		Khronos Group, “KHR_gaussian_splatting”, glTF Extension, Review Draft, 2025.

[x2]		Khronos Group, “glTF 2.0 Specification”.

==============Next change==============

## 4.4		Existing Representation Formats

### 4.4.1		glTF-based Representation Formats

The Khronos Group published a review draft of the KHR_gaussian_splatting extension [2] for glTF 2.0 [3] in August 2025, together with the SPZ binary container format. The extension defines Gaussian splats as POINTS primitives within standard glTF, using the following attribute semantics:

POSITION (VEC3, required): Splat center position, using the standard glTF base attribute.

ROTATION (VEC4, required): Quaternion (x,y,z,w) defining the local axes orientation of each splat.

SCALE (VEC3, required): Per-axis scale in log-space.

OPACITY (SCALAR, required): Opacity in the range [0,1].

SH_DEGREE_l_COEF_n (VEC3, conditional): Spherical harmonics coefficients organized by degree (0 through 3) and coefficient index, enabling view-dependent lighting.

COLOR_0 (VEC3/VEC4, recommended): Baseline color for fallback point-cloud rendering on clients that do not support Gaussian splatting.

KHR_gaussian_splatting is explicitly designed to be extended by other extensions using a nested extensions mechanism inside the KHR_gaussian_splatting object. This is intended for extensions that add compression, alternative coefficient encodings, or other processing without duplicating semantics. This design makes the Khronos extension a strong anchor for MPEG and 3GPP work targeting interoperable distribution and streaming.

Any client that does not recognize the KHR_gaussian_splatting extension can still render the data as a standard point cloud using POSITION and COLOR_0, providing a built-in graceful degradation path.

Within MPEG-I Scene Description, a proposal has been agreed to rebase the MPEG Gaussian Splatting extension on KHR_gaussian_splatting [x1]. Rather than duplicating the core splat semantics, the MPEG extension (MPEG_gaussian_splatting_transport) is carried as a nested extension inside KHR_gaussian_splatting.extensions and adds only transport-level features:

Alternative SH layouts: Two MPEG-specific SH coefficient storage modes are defined alongside the Khronos default. The “mpegProgressive” layout groups coefficients by SH degree (degree 1, degree 2, degree 3 as separate SCALAR accessors), which is efficient for progressive refinement because a receiver can render with only SH degree 0 data and incrementally fetch higher degrees. The “mpegPerChannel” layout separates coefficients by colour channel (R, G, B), which can be more efficient for certain compression schemes. In both MPEG layouts, the DC (degree 0) term is not stored in the MPEG accessors but is either reconstructed from COLOR_0.rgb or carried explicitly via the KHR SH_DEGREE_0_COEF_0 attribute.

Mesh primitive support: The “mpegStitching” layout permits to attach a Gaussian Splat to a 3D triangular mesh primitive which is efficient for using traditional deformation techniques to Gaussian Splats, and provide accurate geometry support . When present, the 3D mesh and its primitive are explicitly signaled in the stitching object. The stitching object is compatible with all modes.

Progressive download: An optional progressive ordering is signalled by listing accessor indices in progressive.stages, ordered from lower to higher fidelity. A receiver may initially fetch only the first stage and render using whatever attributes are available, then progressively refine the rendering as subsequent stages arrive without re-decoding previously available data.

Timed delivery for 4D splats: Dynamic 4D Gaussian splat sequences are supported using the existing MPEG timed media mechanisms. An accessor is treated as time-varying if and only if it carries the MPEG_accessor_timed extension. Timed accessors are backed by circular buffers as defined by MPEG-I Scene Description.

==============End of change==============