3GPP TSG-S4 #135 Meeting	S4-260355

Goa, India, 9 - 13 February 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.813 | CR |  | rev |  | Current version: |  |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR on evaluation framework |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S4 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | YYYY-MM-DD |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | Adding evaluation framework description for Avatars. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

# 11	Evaluation Framework

## 11.1	Design Principles

The evaluation framework follows four principles:

- Black-box evaluation: Metrics are computed from rendered output video, not from internal system states. This ensures cross-vendor comparability.

- Reproducibility: Fixed test content, deterministic rendering conditions, and standardized capture workflows guarantee consistent results across evaluation runs and over time.

- Automation: All metrics can be computed without human intervention, enabling large-scale testing.

## 11.2	Testbed Architecture

The testbed drives the avatar system with standardized stimuli, captures output, and computes metrics.

Key components include:

- Stimulus player that feeds the avatar system with animation streams: blendshape weights, landmarks, and joints poses.

- Render configuration that locks camera intrinsics, lighting, background, and resolution to eliminate uncontrolled variability.

- Capture module that records rendered frames using lossless or visually lossless compression with frame-accurate timestamps.

- Network emulator that applies controlled latency, jitter, bandwidth limits, and packet loss for transport condition testing.

- Metrics engine that computes frame-level and clip-level objective metrics from captured assets.

## 11.3	Objective Metrics for Avatar Evaluation

The following metrics address the three quality dimensions critical to Avatar communication: visual fidelity, animation accuracy, and temporal performance.

NOTE: The list of metrics is not exclusive and more metrics may be added.

### 11.3.1	Visual Quality Metrics

Visual metrics compare rendered output against a reference video on a per-frame basis.

| Metric | Unit | Description |
| --- | --- | --- |
| PSNR | dB | Peak signal-to-noise ratio between reference and test frames. |
| SSIM | 0-1 | Structural similarity index. |

### 11.3.2	Animation Quality Metrics

Animation metrics measure how accurately the Avatar reproduces input motion. Video-based computation that extracts landmarks and skeletons from rendered output is used for fair comparison.

| Metric | Unit | Description |
| --- | --- | --- |
| Lip Vertex Error (LVE) | pixels/mm | RMS error of mouth landmarks between reference and test. Critical for lip sync evaluation. |
| Facial Distance Deviation (FDD) | pixels/mm | Deviation of expression-related landmark distances. Measures facial expression accuracy. |
| Motion Vertex Error (MVE) | pixels/mm | RMS error of body joint positions. Evaluates full-body animation fidelity. |

### 11.3.3	Temporal and Synchronization Metrics

Temporal metrics assess latency, frame rate stability, and audio-visual synchronization. The following metrics should be considered in a second phase of the evaluation due to their complexity.

| Metric | Unit | Description |
| --- | --- | --- |
| Rendering Frame Rate | FPS | Computed from frame timestamp deltas. |
| Dropped Frame Ratio | % | Percentage of missing or repeated frame indices. |
| Motion-to-Photon Latency | ms | Time from input motion event to visible response in rendered frames. |
| End-to-End Latency | ms | Total delay from sender capture to receiver presentation. |
| Audio-Visual Sync Offset | ms | Offset between mouth motion and corresponding audio, measured via cross-correlation. |

## 11.4	Test Content

Standardized animation streams should cover the range of behaviors encountered in real Avatar communication:

- Neutral speech with clear visemes and steady head motion for baseline lip sync assessment.

- Expressive speech with emotions (happiness, surprise, concern) to test facial expression fidelity.

- Conversational turn-taking with gaze shifts, nods, and backchannel gestures.

- Non-verbal body motion including pointing, waving, posture changes, and idle animation.

Each test set should contain the reference audio, reference animation streams, and a reference rendered video from a high-quality reference pipeline and from a capture at the source.

==============End of change==============