3GPP TSG-SA WG4 Meeting #134	S4-252085

# Dallas, Texas, USA, 17 – 21 November 2025	   is revision of S4-251631

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0007 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Solution for KI5 Media Application Server Energy management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2025-11-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addtion of a solution to Key Issue 5 on Media Application Server Energy management. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a clause 7.12 adding a solution to Key Issue 5 on Media Application Server Energy management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No solution proposed for the KI. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.1, 7.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1ST Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[87]	Accubattery: https://play.google.com/store/apps/details?id=com.digibites.accubattery

[88]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[89]	ETSI TS 103 998: "Content Steering for DASH".

2nd Change

## 7.1	Mapping of Solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solutions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| #1 |  |  | X |  |  |  |
| #2 |  |  | X |  |  |  |
| #3 |  | X |  |  |  |  |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  |  |  |  |
| #6 | X | X |  |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 | X |  |  |  |  |  |
| #10 |  |  |  |  |  | X |
| #11 |  |  |  |  | X |  |

These solutions are simply candidate solutions. Their inclusion in the following clauses does not imply that they have been agreed upon or endorsed. Any decisions and work to be done for the normative work will be detailed in the conclusions of this Technical Report.

3rd change
(All new text)

## 7.12	Solution #11: Steering server driven selection of delivery paths based on energy characteristics

### 7.12.1	Key Issue mapping

This solution candidate addresses Key Issue #5 (Media Application Server Energy management) described in clause 6.5.

### 7.12.2	Functional description

#### 7.12.2.1	Introduction

The main media streaming formats such as MPEG-DASH and HLS have been updated to include additional information in their manifests to specify a server (called a "steering server") capable of dynamically selecting the delivery path to use. Currently, this selection is primarily based on Quality of Service (QoS) criteria.

From Release 19, this feature is already supported in the 5G Media Streaming System whereby the Media Player supports content steering for downlink media streaming as described in clause 10.3A.4 of TS 26.512 [88].

In parallel, networks have collectors of energy-related characteristics, including information that characterizes the network's power supply in terms of energy consumption, energy source, carbon footprint, capacity, and availability. These collectors provide insights into environmental impact at different granularities (per user, per service, etc.). This is notably the case with the Energy Information Function (EIF) defined in the 5G architecture.

However, there is currently no method for using these network energy characteristics to select the video streaming delivery path with the lowest environmental impact.

The proposed solution consists of defining a process that allows steering servers to make this choice based on environmental impact, through the sharing of network energy-related characteristics. Since content steering is exclusively defined for downlink media streaming, as specified in clause 10.3A.4 of TS 26.512 [88], this solution is applicable solely within this context.

This solution leverages existing interfaces enabling communication between application providers and 5G networks, particularly for exchanging Quality of Experience data. These interfaces are enhanced with network energy-related characteristics, allowing the application provider to use this information on its steering servers.

### 7.12.2	Reference architecture for steering server driven selection of delivery paths based on energy characteristics

Figure 7.12.2.2-11 depicts a reference architecture that realises this candidate solution in the 5GMS Media Delivery architecture defined in TS 26.501 [23] and TS 26.506 [59].

NOTE:	The Energy Information Collector and reference point E5 are not instantiated in this reference architecture because they are not required by the Candidate Solution.

Figure 7.12.2.2-1: Reference architecture for steering server driven selection of delivery paths based on energy characteristics

The core principle of this architecture revolves around a component called the Energy Information AF, which is instantiated within the 5GMSd AF. Its responsibilities vary depending on its current provisioning state, as determined by the 5GMSd AF (which includes the Energy Information Exposure Specification provisioned by 5GMSd Application Providers – see clause 7.8). These responsibilities may include:

-	Subscribing to and consuming NF Energy Information from the Energy Information Function as defined in TS 23.501 [72]) with required granularities (UE, PDU session and/or QoS flow) in accordance with the parameters of the Energy Information Exposure Specifications.

-	Subscribing to and consuming AS Energy Information from the 5GMSd AS in accordance with the parameters of the Energy Information Exposure Specification.

Unlike other solutions described in this document that utilise the Energy Information AF (e.g. Solution #5 or #10), this approach does not require an Energy Information Collector within the device. The primary goal is to enable selection of the delivery path based on energy characteristics that align with user preferences or service provider criteria. Regardless of the chosen delivery path, the energy characteristics are assumed to remain unchanged on the UE. Therefore, the path selection relies solely on the energy information provided by the network through the Energy Information AF. A second difference is the use of reference point M4, already defined in TS 26.501 [23], to provide steering information to the 5GMSd Client.

### 7.12.3	Procedures

Figure 7.12.3-1 below illustrates the various steps involved in the steering server-driven selection of delivery paths, based on energy characteristics outlined in clause 7.12.2. This process is grounded in content steering as specified in clause 10.3A.4 of TS 26.512 [88] and incorporates energy-related information sharing as defined in Solution #5 in clause 7.6.

Figure 7.12.3-1: Procedures for steering server driven selection of delivery paths based on energy characteristics

The procedure is based on the steps outlined in Solution #5 in clause 7.6, except for those involving the Energy Information Collector, which is not used in the context of this solution. Instead, it integrates the typical content steering procedure, such as the one described for DASH as specified in ETSI TS 103 998 [89]. A steering server is instantiated as part of the (logical) 5GMSd AS to enable to decide the delivery path based on energy characteristics of each path. The details of content steering have been highlighted in boldface within the steps below.

Steps:

1. 	The 5GMSd Application Provider provisions the Energy Information AF with an Energy Information Exposure Specification via reference point M1 (or E1). The Energy Information exposure configuration includes a request for using optimization based on content steering.

NOTE:	The choice between E1 and M1 for provisioning is not addressed in this solution. The advantages and disadvantages of each interface will be analysed in a separate solution.

2.	The Energy Information AF subscribes to receive Network Energy Information reporting from the Energy Information Function via reference point E12, if relevant, in accordance with the set of parameters in the Energy Information Exposure Specification.

3. 	The 5GMSd AF provides the base URLs to the provisioned 5GMSd AS service locations as well as to the 5GMSd AS content steering service.

Then, time passes.

4.	The Energy Information Function may submit a Network Energy Information report to the Energy Information AF via reference point E12.

5.	The Energy Information AF processes the energy-related information report it has received and provide this information to the 5GMSd AF.

6.	The 5GMSd AF provides steering configuration to the 5GMSd AS acting as steering server via reference point E3.

Then, time passes.

7.	The 5GMSd-aware Application triggers the 5GMSd Client to initiate a media delivery session via reference point M6d.

8.	The Media Session Handler interacts with the 5GMSd AF to acquire the whole Service Access Information via reference point M5d. The Service Access Information may include Media Player Entry URLs and Energy Information AF endpoint.

9.	The Media Session Handler provide the Media Player Entries to the 5GMSd Client.

10.	The Media Player is invoked with the selected Media Entry Point to start media access and playback.

11.	Thanks to the information provided in the Media Player Entry, the Media Player requests steering instruction from the content steering service instantiated in the 5GMSd AS via reference point M4d.

12.	The Media Player establishes the transport session for acquiring the Media Player Entry with the Service Location #1.

13.	The Media Player requests the media to AS1 to the Service Location #1.

Steps 14 to 16 are identical to steps 4 to 6. They allow energy-related information about the network to be updated during the media delivery session. If it detects a change in this information, the Energy Information AF may decide to update steering instructions.

17.	The 5GMSd Service Provider ingests the media in the different 5GMSd AS.

Then, steps 18 to 20, or 21 to 23 are identical to steps 11 to 13. They allow to retrieve the media from the Service Location with the higher priority according to the steering instructions. No specific energy information is described in the steering instructions. The processing done by the Energy Information AF allows to identify the appropriate service location. This service location is then configured in the steering instructions with the higher priority to ensure its selection by the Media Player.

### 7.11.4	Summary

This candidate solution proposes a mechanism allowing application providers to select the video streaming delivery path with the lowest environmental impact, based on:

1.	The collection of network energy-related characteristics by the mobile network operator. This information is provided by the Energy Information Function to a new component called Energy Information AF, included in the 5GMSd AF.

2.	Standardised interfaces between the 5GMSd AF and the 5GMSd Application Provider allowing the 5GMSd Application Provider to provision the use of steering mechanism and to  and provision steering information with the goal of reducing its environmental impact.

3.	The use of this information by a 5GMSd AS acting as steering server to dynamically direct users toward the delivery paths with the desired energy characteristics.

This approach optimizes the energy efficiency of multimedia content delivery while maintaining acceptable service quality, thus contributing to reducing the environmental footprint of video streaming services.

The solution applies to any streaming service provider delivering videos via mobile networks and integrates naturally into existing adaptive streaming architectures such as MPEG-DASH and HLS.

The solution has no impact on the UE as steering mechanisms are part of adaptive streaming protocols such as MPEG-DASH and HLS. The selection of the service location with the higher efficiency is done by giving a higher priority to this service location in the instructions provided by the content steering service.

End of changes