3GPP TSG-SA5 Meeting #164	S5-255396

Dallas, USA, 17 - 21 November 2025

Source:	Nokia

Title:	Pseudo-CR on Add use case solution on access control for data

Document for:	Approval

Agenda item:	6.20.8

Spec:	3GPP TR 28.887

Version:	0.2.0

Work Item:	FS_MADCOL_Ph3

Comments

<Proposals, reason for change, abstract, comments if necessary (optional)>

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[Y1]	3GPP TS 28.319: "Access control for management services".

[Y2]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[Y3]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[Y4]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

* * * Next Change * * * *

## 4.3 	Enhancement of Management Services Access Control (MSAC)

#### 4.1.A.4      Potential Solution

In this proposed solution, the following enhancement to the AccessRule class (defined in clause 7.3.3 of TS 28.319[Y1]) is proposed:

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| allowedManagementData | List of the management data that the MnS consumer is allowed to collect for a given scope (as identified by the dataNoteSelector attribute). allowedValues: Includes performance measurements defined in TS 28.552[Y2], key performance indicators (KPIs) defined in TS 28.554[Y3] and trace metrics defined in TS 32.422[Y4]. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

The proposed solution adds a new attribute called “allowedManagementData” to the AccessRule class. With this attribute, the management data that an MnS consumer can collect on a given scope (specified by the dataNodeSelector attribute) is defined.

When an MnS producer receives a request for management data collection from the MnS consumer, it checks if the “requested management data” on a managed object is part of the configured “allowedManagementData” for the MnS consumer for the managed object (represented by the dataNodeSelector attribute) in the AccessRule class. If this is the case, the MnS producer will authorize the management data collection request from the MnS consumer and begin the data collection and reporting as per the configured intervals.

* * * End of Changes * * * *