3GPP TSG-SA5 Meeting #165	S5-260071

Goa, INDIA 9 - 13 February 2026

Source:	Huawei, China Unicom

Title:	Pseudo-CR on TR 28.881 Update Use Case#10 Radio network delivering in transient overload scenario

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	1.0.0

Work Item:	FS_IDMS_MN_Ph4

Comments

It proposes to update the use case, requirements and solution for Use Case#10: Radio network delivering in transient overload scenario.

Proposed Changes

* * * First Change * * *

4.10	Use Case#10: Radio network delivering in transient overload scenario

4.10.1	Description

This use case describes a scenario where a MnS consumer express intent containing an expectation for ensuring a radio network in transient overload scenarios (e.g., high-speed rail or subway systems) to a MnS producer. In transient overload scenarios, the high load caused by massive user access and Burstyn traffic typically lasts only a few seconds specifically when trains pass through a cell, while traffic volume remains extremely low during other periods.

In 3GPP TS 28.312 [1], the existing use case and requirements for intent containing an expectation on radio network performance to be assured is described in clause 5.1.5. The RadioNetworkExpectation is defined to represent MnS consumer's expectations for radio network delivering and assurance in the specified area for the specified scheduling time. The current definition for RadioNetworkExpectation cannot be used to specify the expected radio network performance for the transient overload scenario only. This scenario can help MnS producer identify the actual high load information when these shot-duration, high-impact events occur thereby supporting network expansion decision-making.

4.10.2	Potential requirements

REQ-IDMS_RadioNetworkIntent-CON-1: The intent driven MnS producer for radio network should have capabilities enabling the MnS consumer to express performance assurance expectations for the scenario with specific load condition (e.g. transient overload scenarios).

4.10.3	Potential solution

This solution proposes to reuse and enhance the existing RadioNetworkExpectation defined in 3GPP TS 28.312 [1].

Enhancement Aspect1: Add following attributes as the ExpectationContexts for the RadioNetworkExpectation to support transient overload scenarios:

-	PrbHighLoadRatioContext, it represents the load condition for the radio network that the intent expectation is applied. The contextAttribute is PrbHighLoadRatio, contextCondition is " IS_GREATER_THAN" and contextValueRange is integer with allowed value [0,100]%. The detailed PrbHighLoadRatio see PrbHighLoadRatio defined in TS 28.554 [8].

4.10.4	Evaluation of potential solutions

Only one potential solution has been identified, which is feasible.

* * * Next Change * * * *

5.10	Use Case#10: Radio network delivering in transient overload scenario

The use case of radio network delivering in transient overload scenario in clause 4.10 and following management capabilities are identified:

-	The capability enabling the MnS consumer to express performance assurance requirement for the scenario with specific load condition (e.g. transient overload scenarios)

It is recommended to enhance the RadiNetworkExpectation defined in 3GPP TS 28.312 [1] to support above identified management capabilities:

- Add " PrbHighLoadRatioContext" as the ExpectationContexts for the RadioNetworkExpectation to represent the load condition for the radio network that the intent expectation is applied.

The detailed solution in clause 4.10.3 is used as baseline for normative work.

* * * End of Changes * * * *