3GPP TSG-SA5 Meeting #165	S5-260124

Goa, India, 9th Feb 2026 - 13th Feb 2026

Source:	Ericsson Hungary

Title:	pCR TR 28.884 Historical alarm list solution

Document for:	Approval

Agenda item:	6.20.4

Spec:	3GPP TR 28.884

Version:	V0.2.0

Work Item:	FS_SBMA_Ph4

Comments

We propose some, mostly editorial updates to the requirements and possible solutions.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[3]	3GPP TS 28.532: " Management and orchestration; Generic management services".

[4]	3GPP TS 28.537: " Management and orchestration; Management capabilities".

[5]	3GPP TS 28.552: " Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: " Management and orchestration; 5G end to end Key Performance Indicators (KPIs)".

[7]	3GPP TS 32.423: " Telecommunication management; Subscriber and equipment trace: Trace data definition and management".

[8]	https://datatracker.ietf.org/doc/html/rfc6455

[9]	https://websocket.org/guides/websocket-protocol/

[10]	3GPP TS 28.111: "Management and orchestration; Fault management (FM)".

[11]	3GPP TS 32.531: "Telecommunication management; Software management (SwM); Concepts and Integration Reference Point (IRP) Requirements".

[12]	3GPP TS 32.532: "Telecommunication management; Software management (SwM); Integration Reference Point (IRP); Information Service (IS)".

[13]	3GPP TS 32.533: "Telecommunication management; Software management (SwM); Integration Reference Point (IRP); Common Object Request Broker Architecture (CORBA) Solution Set (SS)".

[14]	3GPP TS 28.631: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements".

[15]	3GPP TS 28.632: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[16]	3GPP TS 28.633: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[17]	SP-250863: Study on SBMA enhancement phase 4

[18]	3GPP TS 32.158: " Design rules for REpresentational State Transfer (REST) Solution Sets (SS)"

[19]	RFC 6241 Network Configuration Protocol (NETCONF)

[20]	3GPP TS 32.101: "Telecommunication management;Principles and high level requirements".

[a]	3GPP TS 28.622: " Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”

* * * Next Change * * * *

### 5.3.2	Potential requirements

REQ-FM-HIST-1: The MnS producer shall support the capability allowing a MnS consumer to retrieve alarms that were active in the past (historical alarms).

REQ-FM-HIST-2: The MnS producer shall support the capability allowing a MnS consumer to  retrieve alarms that were active at a certain point in time in the past.

REQ-FM-HIST-3: The MnS producer shall support the capability allowing a MnS consumer to retrieve alarms that were active during a certain time window in the past.

REQ-FM-HIST-4: The MnS producer shall support the capability allowing a MnS consumer to retrieve the dates and times when the perceived severity of an alarm changed

REQ-FM-HIST-5 The MnS producer shall support the capability allowing a MnS consumer to retrieve the periods when the retrieved historical alarms may not fully represent the real situation in the network at that point in time.

REQ-FM-HIST-6 The MnS producer shall support the capability allowing a MnS consumer to retrieve historical alarms independent of whether notifications about the alarm changes were sent out or not.

* * * Next Change * * * *

### 5.3.3	Potential solutions

#### 5.3.3.1	Overview

The functionality includes:

- storing alarms and alarm changes even after they become inactive.

- the producer shall provide an interface for retrieving historical alarms.

- it shall be possible to filter the retrieval request to get a subset of all historical alarm records

- access control shall be provided for retrieving historical alarms

#### 5.3.3.2	Storing alarm history

The historical alarm list shall contain one or more records for all raised alarms. The records shall include some metadata like the alarmId, timestamp and the JSON body of the alarm change notification. This format is already used in TS 28.622[a] clause 4.3.82 NotificationEntry.

The definition of the alarmId shall be updated in TS 28.111. As alarm history records may be stored corresponding to multiple alarmlists and for weeks or even many months, the alarmId shall be unique within a longer period of time at least within a Subnetwork.

The first record shall be stored when the alarm is raised.

A subsequent record shall be stored when a relevant change occurs. A change is relevant for an alarm if one of the following properties change:

- perceivedSeverity. This includes setting the perceivedSeverity to cleared.

- ackState

Implementers may select other properties, the change of which results in creation of additional records.

Initially, alarms will be available both via the AlarmList MOI and the historical alarm list. Once an alarm becomes inactive it will only be available in the historical alarms list.

The size of the historical alarm list is implementation specific. When the historical alarm list is full the oldest records shall be discarded. The alarmId and the timestamp of the oldest historical alarm should be readable by the consumer.

If the fault management system detects that it may not fully represent the alarm situation it shall store a record containing the notifyPotentialFaultyAlarmList and notifyAlarmListRebuilt notifications (even if there is no subscription for these notifications).

- Note: "Storing" the historical alarm list does not imply anything about implementation. It only specifies that the information shall be available for retrieval.

#### 5.3.3.3	Retrieving alarm history

##### 5.3.3.3.1	Dedicated AlarmHistoryUploadJob

A new IOC could be defined: AlarmHistoryUploadJob under AlamList. This IOC shall be used to order the transfer of historical alarms from the producer to an outside location.

Attributes

- alarmScope a list of DNs as root of subtrees that should include the alarm's source

- TimeWindow (see TS 28.622[a] clause 5.4.1)

- ReportingCtrl, but without the streaming possibility (see TS 28.622[a] clause 4.3.33)

##### 5.3.3.3.2	Using ManagementDataCollection

The ManagementDataCollection could be used to arrange the transfer of historical alarms.

Define alarm history as a new data category.

Instead of targetNodeFilter a simple list of DNs shall be used like the NodeFilter.objectInstances. Maybe areaOfInterest can also be used.

reportingCtrl should be restricted as streaming is not useful for alarm history.

dataScope is not useful for alarm history, it should not be used

condition, processMonitor, consolidateOutput, jobid is not needed for a simple file upload job.

##### 5.3.3.3.3	Modeling alarm history in the NRM

NRM could be extended with a new IOC to model the historical alarms. Two variants can be considered:

An IOC like AlarmList can be defined where a multivalued historicalAlarmRecord attribute has a separate value for each alarm change. The attribute can use the current data type AlarmRecord in TS 28.111 clause 7.3.1.

IOC HistoricalAlarmRecord (Not preferred.): A separate MOI for each historic record. The attributes would be the same as those defined for the data type AlarmRecord in TS 28.111 clause 7.3.1.

##### 5.3.3.3.4	New dedicated operation

A new retrieval operation can be introduced with the time dimension in the operation input parameters. The easiest case is to add a "time" parameter to the operation input parameters that specifies a time in the past for which the active alarms (as represented by their alarm records) shall be retrieved. Filtering capabilities of the retrieval operation may be used to scope the response to specific managed object instances. The target object of the retrieval operation is the already specified "AlarmList" object. Variations of this approach are possible, for example a time window may be specified in the request (for some managed objects), the response then returns the alarm records for all alarms that were active in this time window.

#### 5.3.3.4	Filtering retrieval

The consumer should be able to retrieve a subset of all stored historical alarms. It shall be possible to specify the following retrieval criteria (filter):

- Alarms active at a point of time – the time must not be in the future

- Alarms active in a timewindow – the timewindow must not be in the future

- Alarms where the source of the alarm (objectInstance parameter) is in a subtree of the NRM. Filtering similarly as the AlarmList.unreliableAlarmScope attribute

More advanced filtering e.g. by using more parameters and/or JEX or Xpath would also be possible but is not proposed at this time.

#### 5.3.3.5	Access control

Accessing alarm history provides access to all information that any alarm can carry, including potentially sensitive, secret or personal/private information.

Providing one general access control right, that provides full access to the complete alarm history is relatively simple and allows definition of an FM superuser. Providing detailed access control rules based on the source and the content of the individual alarms is seen as overly complicated and thus is not proposed.

Retrieval solutions based on NRM data nodes are already covered by the current access control solution. For retrieval with a dedicated operation access control shall be implemented.

### 5.3.4	Evaluation of potential solutions

It is proposed to provide historical alarm records in the JSON format of the notification body that would report the alarm change using the datatype NotificationEntry (see TS28.622[a]). This will make parsing historical records simple as the same code can be used that is utilized for active alarms.

It is proposed to define a dedicated AlarmHistoryUploadJob according to clause 5.3.3.3.1 The other solutions are not proposed because:

- Many of the attributes of ManagementDataCollection are not relevant for the historical alarm list, thus it is a bad fit. It is also currently only used on the Subnetwork level. Clause 5.3.3.3.2 is not proposed.

- Historical alarms are retrieved relatively infrequently so exposing this data in the NRM seems unneeded. Exposing big amounts of unneeded data in the NRM is also resource intensive. Clause 5.3.3.3.3 is not proposed.

- SBMA generally tries to avoid dedicated operations. Clause 5.3.3.3.4 is not proposed.

It is proposed to implement only the relatively simple filtering solution described in clause 5.3.3.4. If more sophisticated filtering is needed, it can be added later.

It is proposed to implement only a general access control providing full access.

* * * Next Change * * * *

## 6.x	Use case #3: Historical alarms

The requirements for historical alarms are listed in clause 5.3.2.

All requirements are recommended for normative work based on the potential solution described in clauses 5.3.3 according to the conclusions documented in clause 5.3.4.

* * * End of Changes * * * *