3GPP TSG SA5 Meeting #165	S5-260127

Goa, India, 9-13 February 2026

Source:	Huawei, China Mobile

Title:	Pseudo-CR on TR 32.801-01 Add use case on individual mobile service delivery and assurance for specific events

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 32.801-01

Version:	0.0.0

Work Item:	FS_6G_OAM

Comments

The contribution proposes a use case of individual mobile service delivery and assurance for specific events (e.g. music festival), which reveals critical gaps in current network operations. 6G OAM system needs to provide multiple management capabilities (e.g. network pre-evaluation, network resource adjustment, network performance monitoring and network performance optimization) and coordinate all the management capabilities to deliver individual mobile services on time and ensure the user experience for the users participating the specific events.

Proposed Changes

* * * First Change * * * *

6	6G Management Scenarios

6.1	Management Scenario#1: Individual mobile service delivery and assurance for specific events

6.X.1	Description

This management scenario focus on the scenarios for 6G OAM systems supporting individual mobile service delivery and assurance for specific events. Following uses the Large-scale music festival scenario as an example, in which an opening ceremony and subsequent live performances are supported across multiple geographically distributed venues. During the opening ceremony, AIGC-based virtual hosts and dynamically generated virtual scenes are presented and enable real-time interaction with audiences, which require the mobile network to provide high requirements for latency on both uplink and downlink transmissions. During the performance period, multiple fixed and mobile video capture devices are deployed to support live streaming, which requires the mobile network to provide high requirements on both latency and bandwidth for uplink transmission under mobility conditions. Due to the coexistence of diverse service types, differentiated traffic patterns with non-static spatial and temporal characteristics, it is important for the 6G OAM systems to provide and coordinate multiple management capabilities (e.g. network pre-evaluation, network resource adjustment, network performance monitoring and network performance optimization) by utilizing intelligent mechanism (e.g. Agent, intent driven management, network digital twins) to deliver mobile service on time,  achieve optimal user experience and highly autonomous networks.

Following illustrates the end to end service flows (including pre-event evaluation and planning phase, event assurance phase and post-event evaluation phase) for the individual mobile service delivery and assurance for specific events.

Pre-event evaluation and planning phase:

1. The music festival organizer provides express the individual mobile service deliver and assurance intent to the operator, which includes the overall event plan (e.g. event times, venue names) and corresponding service requirements (e.g. service type, service targets) to the operator to offer corresponding mobile service packages. This interaction can be guided by natural language.

2. The operator express the intent to corresponding 6G OAM systems to generate a detailed service delivery and assurance plan and pre-evaluation of service capacity for specific services (e.g. live streaming service). Multi-round negotiations may be happened between operator and 6G OAM systems to finalize the service delivery and assurance plan. Regarding the event covering multiple areas in which mobile network managed by different 6G OAM systems, operator needs to send the request to different 6G OAM systems.

3. The 6G OAM systems predicts the traffic load and network performance, evaluates the service capacity (i.e. number of user possible to subscribe and access the mobile network with expected service targets) and performs network performance optimization tasks (e.g. load balance optimization among different cells, energy saving optimization) for the specified areas to release more network resources for the requested services. If the existing mobile network cannot provide enough resources, 6G OAM systems may recommend operator to provide the mobile communication equipment (e.g. emergency communication vehicles, mobile wireless access base stations) for specific areas to expand mobile network capacity for the requested mobile services.

4. Operator confirms the detailed service delivery and assurance plan and execute the user subscription policy based on the service capacity evaluation result.

Event assurance phase

5. The operator express the intent to the 6G OAM systems to monitor and assure the network and service performance for the specific areas and user groups covered by the specific event.

6. 6G OAM systems collects the network and service performance metrics from the 6G network, analyzing the network and service performance to identify performance issues (e.g. Poor-performing cell, low UL/RL UE throughput) and predict the performance deterioration risk for corresponding areas and user groups. The network optimization and adjustment actions need to be taken to resolve the identified issues and predicted risks to minimize the impact on user experience. 6G OAM systems also needs to provide the proactive fault intervention and recovery to avoid the service outage. 6G OAM systems may interact with operator to finalize the network optimization and fault recovery actions.

7. The 6G OAM systems also provides network and service performance metrics for operator for the specific areas and user groups to monitor the network and service.

Post-event evaluation phase

8. After completion of the event, operator request the 6G OAM systems to provide the post-event analysis and evaluation report.

9. 6G OAM systems performs post-event evaluation by analysis corresponding network and service performance for specific areas or user groups and generate the evaluation report for the operator. The evaluation report also includes the presentation of dynamic network resource adjustments and the fulfillment of service performance targets for the users, thereby providing evidence of the effectiveness of automated and intelligent service assurance capabilities of 6G OAM systems.

10. Based on the post-event evaluation report from the 6G OAM systems, operator generate the fulfilment report (include the service delivery and assurance successful ratio) for the individual mobile service delivery and assurance for specific events to the music festival organizer.

6.X.2 Potential Requirements

[PR.6.x.2-1]	The 6G OAM systems shall be able to generate a detailed service delivery and assurance plan for specific events based on intent expressed by operators and multi-round negotiations with operators.

[PR.6.x.2-2]	The 6G OAM systems shall be able to predicts the traffic load and network performance for different areas.

[PR.6.x.2-3]	The 6G OAM systems shall be able to evaluates the service capacity (i.e. number of users can be supported by the mobile network) for specific services.

[PR.6.x.2-4]	The 6G OAM systems shall be able to perform and coordinate multiple network performance optimization tasks to generate a detailed service delivery and assurance plan for specific events.

[PR.6.x.2-5]	The 6G OAM systems shall be able to recommend the operator to provide the mobile communication equipment (e.g. mobile wireless access base stations) for specific areas to expand mobile network capacity for specific events.

[PR.6.x.2-6]	The 6G OAM systems shall be able to monitor network and service performance for the specific areas and user groups.

[PR.6.x.2-7]	The 6G OAM systems shall be able to assure the network and service performance for specific event.

[PR.6.x.2-8]	The 6G OAM systems shall be able to identify performance issues (e.g. Poor-performing cell, low UL/RL UE throughput) and predict the performance deterioration risk for corresponding areas and user groups.

[PR.6.x.2-9]	The 6G OAM systems shall be able to perform post-event evaluation by analysing corresponding network and service performance for specific areas or user groups and generate the evaluation report for the operator.

* * * End of Changes * * * *