3GPP TSG-SA5 Meeting #164	S5-260133

Dallas, USA, 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.570 | CR | - | rev | - | Current version: | 0.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 DraftCR TS 28.570 Security event delivery and configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SECHAND-OAM |  |  |  |  |  | Date: |  |  | 2025-10-dd |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TS 33.502, SA3 defined new security related events that need to be delivered to the "Security related events collecting entity". Security event delivery also needs configuration as described in TS 33.502 clause 5.3. For security related event delivery, support of the Syslog Protocol (IETF RFC 5424) is current industry practice among both network equipment vendors and SIEM/SOAR vendors (SIEM = Security Information and Event Management, SOAR = Security Orchestration, Automation and Response). Therefore, it is reasonable to use the Syslog Protocol also for the delivery of the security related events specified in TS 33.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define configuration and event delivery mechanism for security related events. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

# 1	Scope

The present document describes the configuration, monitoring and delivery of security related events based on TS 33.502[b].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[a]	3GPP TS 33.502: "Security related Events Handling".

[b]	3GPP TS 28.532: "Management and orchestration; Generic Management services".

[c]	3GPP TS 28.622: " Generic Network Resource Model (NRM)".

==============Next change==============

The Security related Events Handling enables delivering security related events to an event collecting entity. A network function detects a security event, creates a message that reports this event and sends the event reporting message to a collecting entity.

This specification describes the protocol and method these event reporting messages are delivered and specifies how the service shall be configured and monitored.

Detected security events may be reported or handled by other mechanisms too, e.g. security alarms may be generated. For this reason, when, how and which entities detect security events is out of scope for this specification.

# 5	Requirements

REQ-SECEVENT-1: It shall be possible to transport security related events from the producer to Security related events collecting entity.

REQ-SECEVENT-2: It shall be possible to configure the MnS producer with the needed data to enable transporting security related events from the producer to Security related events collecting entity.

See TS 33.502 [a] for details relevant to the requirements.

# 6	Solution description

## 6.1	Solution components

Table 6.1-1: Security Event Handling solution components

| Usage | Operations and notifications | NRM |
| --- | --- | --- |
| Configuring reporting | Syslog protocol for event transfer | SecEventReporting |

## 6.2	Overview

The solution consists of two main components: NRM data to configure and monitor security event handling and transport the event reporting messages on the syslog protocol to deliver events to the event collector (a.k.a. event collecting entity).

Configuration controls event reporting. As event detection is an internal matter to the implementation, not directly visible on the interface, it is not subject to configuration. Whether a provider stops detection when reporting is stopped or continues detection (e.g. as a base of security alarms) without reporting the occurrences as security events, is an implementation decision. There is no expectation, that events that occurred during the period when reporting was switched of, should be stored, queued and reported when the reporting is switched back again.

It shall be possible to configure the producer to limit or stop sending events in case of overload at the security related events collection entity.

Editor's note: According to 33.502 " It shall be possible to configure the NF to stop sending events in case of overload at the Security related events collection entity." How does the event sender (producer) detect such an overload?

# 7	Model

## 7.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| 3GPP 28.622 [c], IOC, Top | Top |
| 3GPP 28.622 [c], IOC, ManagedElement | ManagedElement |
| 3GPP 28.622 [c], IOC, SubNetwork | SubNetwork |

## 7.2	Class diagrams

### 7.2.1	Relationships

This clause depicts the set of classes (e.g. IOCs) implemented by security event delivery and configuration. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

Figure 7.2.1-1: SecEventReporting NRM fragment

### 7.2.2	Inheritance

This clause depicts the inheritance relationships.

Figure 7.2.2-1: FM control NRM fragment

## 7.3	Class definitions

### 7.3.1	SecurityEventTypesParameters <<dataType>>

#### 7.3.1.1	Definition

Parameters related to each security event type. The attribute elements of this type should be created and deleted by the producer including the securityEventType attribute field. Other attribute fields should be set by the consumer. Any security event type not listed that the system implements shall use the default values of the attribute fields.

#### 7.3.1.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| securityEventType | M | Y | Y | Y | Y |
| isReportingEnabled | M | Y | Y | N | Y |

#### 7.3.1.3	Attribute constraints

None.

#### 7.3.1.4	Notifications

The notification in the notification subclause of the IOC using this datatype shall be available.

### 7.3.1	SyslogParameters <<dataType>>

#### 7.3.1.1	Definition

Parameters needed for the Syslog protocol and for embedding the security event in syslog messages.

TBD.

#### 7.3.1.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |

#### 7.3.1.3	Attribute constraints

None.

#### 7.3.1.4	Notifications

The notification in the notification subclause of the IOC using this datatype shall be available.

### 7.3.2	SecEventReporting

#### 7.3.2.1	Definition

The SecEventReporting represents the capability to configure and monitor security event delivery.

To enable security event reporting and delivery a SecEventReporting MOI needs to be created and its administrativeState needs to be set to UNLOCKED and at least one reporting destination has to be configured. Reports can be sent to multiple destinations.

Once reporting function is enable, it can be set per security event type whether events of that type shall be reported or not.

If the producer is overloaded, it may report every nth occurrence of a specific security event type as specified by the overLoadParameter. In case of severe overload, the system may effectively increase this number. The definition of overload and heavy Overload is implementation specific.

#### 7.3.2.2	Attributes

The SecEventReporting IOC includes attributes inherited from Top IOC and the following attributes:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| administrativeState | M | T | T | F | T |
| operationalState | M | T | F | F | T |
| reportingDestinationList | M | T | T | F | T |
| protocolDetails | M | T | T | F | T |
| overLoadParameter | M | T | T | F | T |
| securityEventTypesList | O | T | T | F | T |

#### 7.3.2.3	Attribute constraints

None.

#### 7.3.2.4	Notifications

The common notifications defined in clause 7.5 are valid for this IOC.

## 7.4	Attribute definitions

### 7.4.1	Attribute properties

The following table defines the properties of attributes specified in the present document.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| administrativeState | Administrative state of a managed object instance. The administrative state describes the permission to use or prohibition against using the object instance. The administrative state is set by the MnS consumer. allowedValues: LOCKED, UNLOCKED. | type: BasicAdministrativeState multiplicity: 1 defaultValue: UNLOCKED isNullable: False |
| operationalState | Operational state of managed object instance. The operational state describes if an object instance is operable ("ENABLED") or inoperable ("DISABLED"). This state is set by the object instance or the MnS producer and is hence READ-ONLY. allowedValues: ENABLED, DISABLED. | type: ENUM multiplicity: 1 defaultValue: DISABLED isNullable: False |
| reportingDestinationList | Destination(s) where the syslog message shall be sent. If no Uri is defined that disables security event reporting. | type: Uri multiplicity: * isNullable: False |
| protocolDetails | Details of the event delivery protocol | type: SyslogParmeters multiplicity: 1 isNullable: False |
| overLoadParameter | If the producer is overloaded, it may report every nth occurrence of a specific security event type as specified by this attribute. In case of severe overload, the system may effectively increase this number. The definition of overload and heavy Overload is implementation specific. | type: integer multiplicity: 10 defaultValue: UNLOCKED isNullable: False |
| securityEventTypesList | Parameters for handling individual security event types. | type: SecurityEventTypesParameters multiplicity: * isNullable: False |
| securityEventType | Name of the security event type. Additional values may be added by vendors or other organizations. Values shall be unique allowedValues: TBD | type: ENUM multiplicity: 1 isNullable: False |
| isReportingEnabled | Specifies whether events of this type should be reported or not. An event is reported if the value is true SecEventReporting. administrativeState=UNLOCKED | type: ENUM multiplicity: 1 defaultValue: true isNullable: False |

### 7.4.2	Constraints

None.

## 7.5	Common notifications

### 7.5.1		Configuration notifications

This clause presents a list of notifications, defined in TS 28.532 [b], that a MnS consumer can receive. The notification header attribute objectClass/objectInstance, captures the DN of an instance of an IOC defined in the present document.

| Name | Notes |
| --- | --- |
| notifyMOICreation |  |
| notifyMOIDeletion |  |
| notifyMOIAttributeValueChanges |  |
| notifyMOIChanges |  |

# 7	Syslog details

TBD

Editor’s note:  There need to be a way to limit the event size. E.g. if a 100 GB message is reported, we probably only want the first section to be reported.

Annex B (informative):
PlantUML for figures

# B.1	Relationships

@startuml Figure 4.2.1-x: SecEventReporting NRM fragment

skinparam monochrome true

skinparam ClassStereotypeFontStyle normal

class ManagedEntity <<ProxyClass>>

class SecEventReporting  <<InformationObjectClass>>

hide empty members

hide circle

ManagedEntity “1” *- “*” SecEventReporting  : <<names>>

note top of ManagedEntity

Represents the following IOCs:

SubNetwork, ManagedElement.

End note

@enduml

Source code for Figure 4.2.1-1 NRM fragment

# B.2	Inheritance

@startuml Figure ?????: SecEventReporting NRM fragment

skinparam monochrome true

abstract class Top <<InformationObjectClass>> {}

class SecEventReporting <<InformationObjectClass>> {}

hide empty members

hide circle

Top  <|-- SecEventReporting

@enduml

Source code for Figure 4.2.2-1: NRM fragment (first part)

==============End of change==============