3GPP TSG-SA5 Meeting #165	S5-260168

Goa, India, 09 - 13 February 2026

Source:	China Mobile

Title:	Pseudo-CR on TR 32.801-01 Add the 6G management architecture design principles

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 32.801-01

Version:	0.0.0

Work Item:	FS_6G_OAM

Comments

This contribution proposes to add the 6G management architecture design principles for TR 32.801-01.

Proposed Changes

* * * First Change * * * *

5	6G Management Architecture Principles

Editor's note: This clause will contain the common 6G management architecture principles identified for the study.

For the 6G management architecture, it is crucial to thoroughly analyse whether and how the architecture of 6G OAM should be restructured or redesigned to support the management of the 6G network, enable new services and features of 6G, and support the new services and features of network management itself. The following is assumed for 6G management architecture:

- The framework of SBMA specified for 5G is assumed as a starting point for discussion.

- The 6G management architecture is assumed to support a unified framework that supports various management capabilities, including specific services/functions, interfaces, and features like plug-and-play and on-demand invocation.

- The 6G management architecture is assumed to be designed as an AI-native, cloud-based management system.

- It should explore the AI technologies to consider integrate disparate automation capabilities introduced in 5G (e.g., AI/ML, IDM, MDA, CCL, NDT) to optimize and resolve operational issues, such as user complaints, alarms, and root cause analysis or support new services.

- It should maintain backward compatibility to seamlessly manage 5G/6G hybrid networks while incorporating intelligent automation to significantly reduce operational complexity.

Based on the above assumption, the 6G management architecture shall:

1.	Support manage 5G/6G networks though a unified management framework.

2.	Support multi-vendor interoperability.

3.	Support the management of multi-access network, e.g. NTN etc.

4.	Support the AI management capabilities, e.g. NDT, Intent, Autonomous agent etc.

5.   Support cloud nativeness principle to design 6G management architecture for 6G resiliency as described in clause 5.6.3 of TR 22.870[X].

* * * End of Changes * * * *