3GPP TSG-SA5 Meeting #165	S5-260176

Goa, India, 09 - 13 February 2026

Source:	Rakuten Mobile Inc.

Title:	pCR on TR 32.801-1 Add use case for data management

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 32.801-1

Version:	0.0.0

Work Item:	FS_6G_OAM

Comments

This contribution proposes to add a use case for data management framework to 6G OAM study in TR 32.801-01.

Proposed Changes

* * * First Change * * * *

### 6.1.1 Management Architecture for 6G

#### 6.1.1.X Management Scenario #<X>:  Data management framework

##### 6.1.1.X.1 Description

In the 5G system, management data is produced and consumed through multiple domain-specific mechanisms, including Configuration Management (CM), Performance Management (PM), Fault Management (FM), Trace, and MDT. Each of these domains defines its own reporting procedures, data models, and transport mechanisms. As a result, the overall data management environment is fragmented, with limited alignment across domains and increased complexity for integration with operators’ OSS/BSS systems and external data processing platforms.

The evolution toward 6G introduces new operational characteristics that significantly increase the volume, velocity, and diversity of data handled by the management system. Emerging use cases, together with the shift toward data-as-a-service and compute-as-a-service models, require efficient access to heterogeneous data sources and support for AI-native functions that depend on continuous, large-scale data ingestion and processing. These trends highlight limitations in the current domain-specific data handling mechanisms. Future management systems are expected to support differentiated QoS requirements for various data types and various deployment scenarios (central or distributed), including constraints on latency, jitter, reliability, and throughput.

To address these developments, data fabric technologies and associated capabilities related to

- Data models

- Data reporting/collection

- Data distribution,

- Data storage

- Data lifecycle management,

- Data security

are expected to play a central role.

In addition, multipoint-to-multipoint data communication within 3GPP management system (between management services) and between 3GPP management system and external data framework, e.g. operators’ OSS/BSS system, core network has become a trend in the industry. Message bus technologies and advancements in the cloud industry can improve efficiency and enable seamless integration with these systems. These are key aspects to be designed in natively in the 6G management system.

##### 6.1.1.X.2 Potential Requirements

MREQ-DMF-1: 3GPP management system should support lifecycle management for management data. The current data models should support standardized meta-data to enable effective data governance and data routing.

MREQ-DMF-2: 3GPP management system should support a unified architecture to collect, report, share and expose management data to internal and external consumers

MREQ-DMF-3: 3GPP management system should multiple QoS requirements for different types of data and deployment scenarios.

Editor’s note: It is FFS if more requirements need to be specified. This is dependent on the progress of the 5GA Rel-20 SBMA_Ph4 SID.

* * * End of Changes * * * *