3GPP TSG SA5 Meeting #165	S5-260191

Goa, India, 9-13 February 2026

Source:	Nokia

Title:	Pseudo-CR on Enhanced Logging for Limited Quota Users

Document for:	Approval

Agenda item:	7.5.2

Spec:	3GPP TR 32.872

Version:	0.2.0

Work Item:	FS_RoamRE_CH

Comments

This pCR proposes a new use case.

Proposed Changes

* * * First Change * * * *

#### 5.3.1.x	Use case #y: Enhanced Logging for Limited Quota Users

In the LBO N40+N47 charging architecture for quota-limited roaming (e.g., 10GB), SMF receives further characteristics indicating limits. Default logging may miss details during CHF failure, leading to inaccurate CDRs. Increased logging captures granular usage, enabling precise post-recovery reconstruction at CHF, avoiding disputes in enterprise/IoT roaming.

The potential charging requirements for this UC is: REQ-3GPPCH-LBHC-x.

* * * Next Change * * * *

### 5.3.2	Potential charging requirements

REQ-3GPPCH-LBHC-1: Charging system shall support failure handling for the Local Breakout charging scenario in which a failure is detected between the CTF and the V-CHF.

REQ-3GPPCH-LBHC-2: Charging system shall support failure handling for the Local Breakout charging scenario in which a failure is detected between the CTF and the H-CHF.

REQ-3GPPCH-LBHC-x: Charging system shall support dynamic switching to increased charging data logging mode at SMF based on limited quota characteristics in LBO roaming.

* * * End of Changes * * * *