3GPP TSG SA5 Meeting #165	S5-260200

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	pCR TR 28.884 Add solution for integration of SBMA with 5GC and 5G Access Network architecture

Document for:	Approval

Agenda item:	6.20.4

Spec:	3GPP TR 28.884

Version:	0.2.0

Work Item:	FS_SBMA_Ph4

Comments

This contribution proposes to update the use case integration of SBMA with 5GC and 5G Access Network architecture, and introduces corresponding solutions.

Proposed Changes

* * * First Change * * * *

## 5.1	Use case #1: Integration of SBMA with 5GC and 5G Access Network architecture

### 5.1.1	Description

In a 5G-Advanced network deployment, a mobile operator adopts an SBMA that leverages a modular reference model as defined in annex A.11 of TS 28.533 [2]. This model abstracts Management Functions (MnFs) as logical entities, independent of their physical implementation, and enables them to interact exclusively through Management Services (MnSs).

Each MnF registers its MnSs with a service registry, enabling dynamic discovery and selection of services across domains and vendors.

For example:

- A fault management function in the RAN management domain autonomously detects faults using AI/ML models and triggers a healing workflow.

- This MnF can then orchestrate actions across both RAN and Core domains, invoking MnSs such as performance analytics, configuration updates, and resource scaling.

- The orchestration logic is implementation-agnostic, which means it does not rely on where or how the functions are deployed, only on the standardized MnS interfaces.

- The SBMA framework ensures that each MnF clearly declares its role as a producer or consumer of MnSs, supporting modularity and reusability.

Issue 1:

In Release-19, a modular reference model has been defined in annex A.11 of TS 28.533 [2]. It needs to continue evolving the architecture reference model for management and orchestration to support new requirements such as AI/ML integration, autonomy, and cross-domain orchestration.

For MnF NDTF, the content of the column "Reference management capability of MnS defined in Annex F" is "FFS". It needs to be updated according to the conclusion of NDT normative work.

Issue 2:

In TS 28.533 [2]: Annex A.11 Table A.11-b defines each MnF in the architecture reference model for management and orchestration along with the corresponding reference specifications that define their functionality. The content of the column "Reference management capability of MnS defined in Annex F" needs to be aligned with the content of corresponding column "Management Capability" of Annex F Table F-1: Overview of management capabilities and corresponding solution sets.

### 5.1.2	Potential requirements

REQ-SBMA-ARM-1:	The 3GPP management architecture should provide a consolidated view of management functions and their relationships.

### 5.1.3	Potential solutions

As a solution for issue 1, for MnF NDTF in Table A.11-b of TS 28.533 [2], it is proposed to update its content of column "Reference management capability of MnS defined in Annex F" from "FFS" to the following:

- NDT Lifecycle Management.

As a solution for issue 2:

- For table A.11-b of TS 28.533 [2], it is proposed to add new MnFs and their reference specifications information in normative work stage. No dedicated new MnFs are identified.

- To fix the gap of issue 2, it is proposed to update the content of the column "Reference management capability of MnS defined in Annex F" of Table A.11-b in TS 28.533 [2] to align with the content of corresponding column "Management Capability" of Annex F Table F-1 in TS 28.533 [2]. This improves clarity and traceability, and improving the usability of TS 28.533 [2].

* * * End of Changes * * * *