3GPP TSG-SA5 Meeting #165	S5-260202

Goa, India, 9 - 13 February 2026

Source:	Huawei

Title:	pCR TR 28.884 Add solution for message bus data request and discovery

Document for:	Approval

Agenda item:	6.20.4

Spec:	3GPP TR 28.884

Version:	0.2.0

Work Item:	FS_SBMA_Ph4

Comments

This contribution proposes to add solution for message bus data request and discovery in SBMA for 5G.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[3]	3GPP TS 28.532: " Management and orchestration; Generic management services".

[4]	3GPP TS 28.537: " Management and orchestration; Management capabilities".

[5]	3GPP TS 28.552: " Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: " Management and orchestration; 5G end to end Key Performance Indicators (KPIs)".

[7]	3GPP TS 32.423: " Telecommunication management; Subscriber and equipment trace: Trace data definition and management".

[8]	https://datatracker.ietf.org/doc/html/rfc6455

[9]	https://websocket.org/guides/websocket-protocol/

[10]	3GPP TS 28.111: "Management and orchestration; Fault management (FM)".

[11]	3GPP TS 32.531: "Telecommunication management; Software management (SwM); Concepts and Integration Reference Point (IRP) Requirements".

[12]	3GPP TS 32.532: "Telecommunication management; Software management (SwM); Integration Reference Point (IRP); Information Service (IS)".

[13]	3GPP TS 32.533: "Telecommunication management; Software management (SwM); Integration Reference Point (IRP); Common Object Request Broker Architecture (CORBA) Solution Set (SS)".

[14]	3GPP TS 28.631: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements".

[15]	3GPP TS 28.632: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[16]	3GPP TS 28.633: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[17]	SP-250863: Study on SBMA enhancement phase 4

[18]	3GPP TS 32.158: " Design rules for REpresentational State Transfer (REST) Solution Sets (SS)"

[19]	RFC 6241 Network Configuration Protocol (NETCONF)

[20]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[XX]	3GPP TS 28.622: "Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

* * * Next Change * * * *

### 5.2.2	Potential requirements

### 5.2.3	Potential solutions

#### 5.2.3.x	Solution #x: Data request for management data streaming based on message bus

When the message bus target (e.g. topic and message broker MnF information) is specified by the MnS consumer, it can be conveyed by the MnS consumer via data request mechanism.

For performance data request by the NRM fragment-based measurement control service (defined in TS 28.622 [XX]), the MnS consumer specifies the streaming target by attribute ManagementDataCollection.reportingCtrl.streamTarget. Therefore, ReportingCtrl <<choice>> can be enhanced to support data streaming based on message bus by adding a new choice attribute mBusTarget with type of MBusTarget <<datatype>>. This attribute mBusTarget shall be supported, when the MnS producer supports message bus-based streaming data reporting. The value of attribute mBusTarget is decided by one of the following ways:

- The value is specified by the consumer in the request of data collection;

- The value is negotiated between the consumer and the producer through a method which is out of the scope of the standardization and then conveyed in the request of data collection from the consumer;

- The value is specified by the producer and then notified to the consumer in the response of data collection request.

The attributes of ReportingCtrl <<choice>> in clause 4.3.33.2 of TS 28.622 [XX] can be enhanced by adding a new CHOICE 5.1 as following:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| CHOICE_1.1 fileReportingPeriod | CM | T | T | F | T |
| CHOICE_2.1 fileReportingPeriod | CM | T | T | F | T |
| CHOICE_2.2 notificationRecipientAddress | CM | T | T | F | T |
| CHOICE_3.1 fileReportingPeriod | CM | T | T | F | T |
| CHOICE_3.2 fileLocation | CM | T | T | F | T |
| CHOICE_4.1 streamTarget | CM | T | T | F | T |
| CHOICE_5.1 mBusTarget | CM | T | T | F | T |

The following attributes are considered for MBusTarget <<datatype>>:

- brokerEndpoint (M): indicates the IP address and port number of the message broking MnF.

- topicName (M): indicates the target message bus topic that the streaming data to be reported to.

### 5.2.3.y	Solution #y: Data discovery for management data streaming based on message bus

When MnS producer supports data streaming based on message bus, it should be discovered by the MnS consumer.

For the performance data discovery mechanism defined in TS 28.622 [XX], the MnS producer registers the data reporting capability by attribute MgmtDataInfo.supportedReportingMethod (FILE or STREAM). Therefore, attribute supportedReportingMethod can be enhanced to support data streaming based on message bus by adding a new allowed value “STREAM_MBUS”. The consumer can discovery that the producer supports message bus as a reporting method, by reading the attribute supportedReportingMethod.

The attribute definition of supportedReportingMethod in 4.4.1 of TS 28.622 [XX] can be enhanced by adding a new allowed value "STREAM_MBUS" as following:

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| supportedReportingMethod | List of supported reporting methods for the associated management data. allowedValues: - FILE - STREAM - STREAM_MBUS | type: ENUM multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |

* * * End of Changes * * * *