3GPP TSG SA5 Meeting #165	S5-260207

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	pCR TR 28.888 Add solution for transformation of MnS information for external MnS consumers

Document for:	Approval

Agenda item:	6.20.9

Spec:	3GPP TR 28.888

Version:	0.2.0

Work Item:	FS_EnExpo

Comments

This contribution proposes to add solution for transformation of MnS information for external MnS consumers.

Proposed Changes

* * * First Change * * * *

### 5.3.3	Potential solutions

Editor's note:	This clause provides one or more solutions. Further (sub-)clause(s) may be added to capture details.

This clause identifies potential solutions to support the transformation of internal slice identifiers (S-NSSAI) into external identifiers (AF-Service-Identifier) when exposing MnS information to external MnS consumers located outside the PLMN trust domain. The objective is to ensure that internal 5G system information is not revealed while still enabling the exposure of network slice related management information.

Reuse of existing configuration management models

TS 28.541 [11] already defines the necessary NRM support for representing AF-Service-Identifier for untrusted AFs, including the unTrustAfInfoList attribute of NefInfo in clause 5.3.164. This existing NRM structure is sufficient to support the encapsulation and transformation of internal identifiers for configuration-related MnS information.

Therefore, the existing configuration management (CM) NRMs already satisfy the encapsulation requirement for external MnS consumers. No additional CM information models or attributes are required to support the transformation of S-NSSAI into AF-Service-Identifier.

Need for new encapsulated performance measurement definitions

While configuration management is already supported, the current performance management (PM) specifications do not provide a mechanism to expose slice related performance measurements using AF-Service-Identifier. TS 28.552 [12] defines performance measurements that can be split into sub-counters per S-NSSAI, but no mapping or transformation mechanism exists for the case of an external MnS consumer or untrusted AF.

To address this gap, the following solution is proposed:

a)	 Define encapsulated PM KPIs based on AF-Service-Identifier

New PM measurement types or measurement groupings may be introduced to represent slice level performance using AF-Service-Identifier instead of S-NSSAI. These encapsulated PM KPIs would:

-	Be derived from existing S-NSSAI-based sub-counters

-	Be aggregated or filtered according to the AF-Service-Identifier mapping

-	Ensure that internal slice identifiers remain hidden from external MnS consumers

-	Support NSaaS related exposure requirements defined in TS 28.530 [13]

(b)	Introduce a transformation/mapping functionality for PM

A transformation/mapping function may be defined to:

- Translate S-NSSAI-indexed PM counters into AF-Service-Identifier–indexed PM counters

- Apply the mapping rules configured in the management system (aligned with clause 6.2.5.0 of TS 23.501 [9] and clause 4.3.6.1 of TS 23.502 [10])

-	Ensure consistent transformation across CM and PM domains

These additions would extend the PM information model without modifying existing S-NSSAI based performance counters.

* * * End of Changes * * * *