3GPP TSG-SA5 Meeting #165	S5-260218

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	Rel-20 pCR TR 28.883 Rapporteur clean up

Document for:	Approval

Agenda item:	6.20.3

Spec:	3GPP TR 28.883

Version:	V0.2.0

Work Item:	FS_NDT_Ph2

Comments

This contribution proposes the clean-up from rapporteur.

Proposed Changes

* * * First Change * * * *

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

NDT	Network Digital Twin

IHF	Intent Handling Function

* * * Next Change * * * *

4	Concepts and background

The concepts and overview described in TS 28.561[3] are applicable in this study. Network Digital Twin (NDT) is used as a replica of a mobile network, to learn how an actual mobile network would behave in certain scenarios, without causing any changes to the actual mobile network. To provide meaningful results, an NDT needs to model the behaviour of the mobile network, so that the result of the operations on the virtual replica is a good approximation to the result of similar operations on the actual network. The created NDT can provide the capability of reporting the topology of the network, and the non-topology aspects of the real network which includes both network elements (e.g., 5GC NFs or gNB) information and infrastructure resource information.

The NDT management capabilities specified in TS 28.561[3] are applicable in this study, which include:

- Control and life cycle management of NDTs

- NDT support for network automation

- NDT support for verification

- NDT support for data generation

- Advanced NDT capabilities

* * * Next Change * * * *

5.8.3	Potential solutions

TS 28.622 defines the ExternalDataType IOC which specifies a type of external management data and the associated meta data. This solution aims to enhance NDTJob IOC and NDTFunction IOC by reusing ExternalDataType IOC to carry the NDT MnS consumer’s external data preference for an NDT job and show the supported external data of NDT Function used for NDT modelling. Details are as shown below:

1. Enhancement for NDTJob IOC

- Add new attribute “externalDataTypeRefList” representing the NDT Job is associated with one or more other ExternalDataType MOIs that are used for NDT modelling.

2. Enhancement for NDTFunction IOC

- Add new attribute “supportedExternalDataList” for NDTFunction IOC which represents the external data supported by the NDTFunction for NDT modelling.

* * * End of Changes * * * *