3GPP TSG SA5 Meeting #165	S5-260221

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	Rel-20 pCR TR 32.801-01 New use case on RAN energy efficiency optimization for green network

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 32.801-01

Version:	V0.0.0

Work Item:	FS_6G_OAM

Comments

This contribution proposes a new use case for 6G OAM on RAN energy efficiency optimization for green network.

Proposed Changes

* * * First Change * * * *

6	6G Management Scenarios

6.1 New 6G management scenarios

6.1.x Management Scenario for Energy Efficiency

6.1.x.1 Management Scenario #<x>: RAN energy efficiency optimization for green network

6.1.x.1.1 Description

Energy efficiency is a key focus for 6G, as networks are expected to support exponentially higher data rates and device densities. For 6G network evolution, the network energy efficiency optimization still plays an important role in operator network management. Operators are aiming at decreasing power consumption in 6G networks to lower their operational expense with energy saving management solutions.

One typical scenario of energy saving is to switch off capacity boosters when the traffic demand is low, and re-activated them on a need basis. TS 28.310 [a] introduces the ES use case for NG-RAN where the capacity booster cell - gNB is overlaid by the candidate cell(s) which can be gNB or eNB. In 6G context, the scenarios will become more complex considering the interworking among 6G RAN, gNB and eNB.

Besides the energy saving management, the attention on usage of renewable energy and reduction of GHG (e.g., carbon dioxide) emission raised in the communication industry for 6G. With the involvement of renewable energy generation, the operator may start the deployment of base station which is powered by renewable energy in multiple ways, such as installing solar PV (i.e., Photovoltaic) attached to base station, or replacing energy source of energy grid from fossil energy source to renewable energy according to the contract between the network operator and its energy supplier. There is the scenario where 6G base stations powered by solar PV and/or Energy Grid both exist and their coverage area may overlap as shown in Figure 6.1.x.1.1-1. The Base Station #A to which the 6G RAN or gNB or eNB candidate cell #A belongs uses fully renewable energy which is generated by its attached solar PV, while its neighbour, Base Station #B to which the 6G RAN or gNB or eNB candidate cell #B, uses the traditional powering method – Energy Grid, which may use fossil energy and emits carbon dioxide.

Figure 6.1.x.1.1-1: Example of 6G RAN energy efficiency optimization for green network

Figure 6.1.x.1.1-1 shows that a 6G RAN capacity booster cell is overlaid by the 6G RAN, gNB or eNB candidate cell(s) which either powered by Energy Grid or renewable energy (e.g., Solar PV). There can be three typical cases of energy efficiency optimization:

-	 Intra-RAT energy saving if the candidate cell is a 6G base station

-	 Inter-RAT energy saving if the candidate cell is an gNB or eNB

-	Carbon emission management control

For the first and second case, 6G OAM system collects the traffic load performance measurements from the 6G RAN capacity booster cell and candidate cells (e.g., 6G RAN or gNB or eNB candidate cell #A and 6G RAN or gNB or eNB candidate cell #B), and may request the capacity booster cell to enter the energy saving mode when its traffic is below certain threshold. The capacity booster cell can be re-activated when additional capacity is needed.

For the third case, 6G OAM system collects the traffic load performance measurements from both 6G Base station #A and #B and may configure a policy to indicate the traffic load transfer from 6G Base Station #B to 6G Base Station #A when 6G Base Station #A traffic is below certain threshold to reduce the carbon emission caused by 6G Base Station #B.

6.1.x.1.2 Potential Requirements

[PR 6.1.x.1.2-1] The 6G OAM system should have the capability to support Intra-RAT energy saving, Inter-RAT energy saving and carbon emission management.

[PR 6.1.x.1.2-2] The 6G OAM system should have the capability allowing its authorized consumer to configure cell overlaid relations, configure energy saving policies, and enable or disable the function for a 6G RAN capacity booster cell to enter energy saving mode.

[PR 6.1.x.1.2-3] The 6G OAM system should have the capability notifying an authorized consumer when the energy saving mode of a 6G RAN capacity booster cell is activated or deactivated.

[PR 6.1.x.1.2-4] The 6G OAM system should have the capability configuring energy efficiency policies based on renewable energy information.

[PR 6.1.x.1.2-5] The 6G OAM system should have the capability collecting traffic load performance measurements and energy efficiency KPIs for the 6G RAN.

* * * End of Changes * * * *