3GPP TSG SA5 Meeting #165	S5-260278

Goa, India, 9-13 February 2026

Source:	Nokia

Title:	Pseudo-CR on Rel-20 TR 32.801-1 6G principles on ES and EE architecture

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR32.801-01

Version:	0.0.0

Work Item:	Study on 6G Management and Orchestration (FS_6G_OAM)

Comments

References

[1]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[2]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

Background

In 5G, there was no specific architecture outlining the different roles and capabilities expected in energy savings and energy efficiency solutions. Several capabilities and actors were added but it is not clear which roles they play and how the capabilities and actors are related to one another, causing confusion to implementations and interoperability. Hence, there is a need to study and define the principles and capabilities in Energy Savings and Energy Efficiency management architecture for 6G.

Observations:

In 5G, specifically, the following actors (see clause 6.2 in TS 28.310 [1]) are mentioned, sometime with their functionalities overlapping:

- Centralized MnS for ES

- NF provisioning MnS

- MnS producer of Domain Centralized SON ES management

- Domain-Centralized ES management

- MnS Producer of Domain-Centralized ES

- MnS Producer of Distributed ES management

- Distributed ES function

- Domain-SON ES functionality

- Domain-centralized SON ES

- Distributed SON ES functionality

- Domain Centralized SON ES management

- CESManagementFunction (IOC defined in TS 28.541 [2])

- DESManagementFunction (IOC defined in TS 28.541 [2])

For Energy Savings and Energy Efficiency management, three scenarios are addressed (see TS 28.310 and TS 28.541).

Scenario 1: A Centralized energy saving scenario, where a centralized ES MnF with an MnS capability for Energy saving management (in the management system) for NG-RAN that collects and analyses traffic load performance measurements, configures the NR cells directly to enter into and out of the energy saving state (e.g., by updating energySaving state attribute) (see Clause 6.2.2 of TS 28.310 [1], specifically Clause 6.2.2.1.1 on Energy saving activation and clause 6.2.2.1.2 on Energy saving deactivation). The attributes of CESManagementFunction IOC (defined in TS 28.541 [2]) in NR NRM is used to control this functionality.

In the similar use case to Switch off edge UPFs during off-peak traffic hours (see clause 5.1.3.4 of TS 28.310 [1]), the Network Operator (NOP) can decide to decommission the edge UPFs for energy saving to:

-	redirect the remaining traffic to and from these edge UPFs to existing central UPFs, and

-	decommission these edge UPFs, or scale them in/down, or any other action enabling to achieve energy saving, depending on e.g. whether these UPFs are virtualized or not. The decommissioning of edge UPFs can be done e.g. by administratively putting them out of service so that they can’t carry any more traffic, either with immediate effect or only when no more users are using these UPFs.

The attributes in UPFFunction (see TS 28.541 [2]) in 5GC NRM is used to control this functionality.

Obeservation #1:

The following are the various actors are mentioned in TS 28.310 [1] related to this scenario, with many functionalites overlapping:

-	MnS producer (where the energy saving decision is made) (see clause 5.1.3.2.2 and clause 5.1.3.2.3 of TS 28.310 [1]).

-	MnS producer (collects the traffic load performance measurements from the NR capacity booster cell and candidate cells, and may request a NR capacity booster cell to enter the energy saving mode when its traffic is below certain threshold) (see clause 5.1.3.2.2 of TS 28.310 [1]).

-	NR capacity booster (see clause 5.1.3.2.2 of TS 28.310 [1]).

-	centralized or distributed ES management (see clause 5.1.3.3 of TS 28.310 [1]).

-	The management service producer responsible for energy saving (see clause 5.2.3.1 of TS 28.310 [1]).

-	Network Operator (NOP) (see clause 5.1.3.4 of TS 28.310 [1]).

-	management service producer responsible for energy saving (see clause 5.2.3.2 of TS 28.310 [1]).

-	Domain-centralized ES (see clause 5.2.3.3 of TS 28.310 [1]).

-	centralized MnS for ES (see clause 6.2.1 of TC 28.310 [1]).

-	NF provisioning MnS consumer requests the producer (see clause 6.2.1 of TC 28.310 [1]).

-	MnS producer of Domain Centralized SON ES management (see clause 6.2.2.1.1 of TS 28.310 [1]).

-	MnS producer for Domain-centralized ES (see clause 6.2.2.1.1 of TS 28.310 [1]).

-	NR capacity booster cell (see clause 6.2.2.1.1 of TS 28.310 [1]).

-	MnS producer of Domain-Centralized ES management (see clause 6.2.2.1.2 of TS 28.310 [1])

-	Provisioning MnS for NF (see clause 6.2.2.2.1 of TS 28.310 [1])

-	Domain-SON ES functionality (see clause 6.2.2.2.2.2 of TS 28.310 [1])

-	Domain-centralized SON ES (see clause 6.2.2.2.3.2 of TS 28.310 [1])

There are several entities or actors mentioned in TS 28.310 [1] related to this scenario, often with overlapping or unclear functionalities.

Conclusion #1:

Based on the scenario described and above observations:

- There should be an MnS capability for Energy saving management defined. The MnS supporting this capability can make Network Elements and/or Network Functions (e.g., NR Cell, UPF) to enter or exit energy saving state by provisioning the attributes in the Network Elements and Network Functions, utilizing the configured control parameters.

- An MnF playing the role of the MnS Producer supporting this MnS capability for Energy saving management would also play the role of MnS Consumer of a Provisioning MnS Producer and a Performance Assurance MnS Producer of the Managed Elements and Managed Functions, e.g., for NG-RAN and 5G Core.

Scenario 2: A Distributed energy saving scenario where, a Distributed ES MnF with an MnS capability for Energy saving management (in the management system) for NG-RAN, configures (with e.g., cell load threshold information, neighbour relations, ES policy, etc) the Network-element Distributed energy saving functionality within the Network Element (e.g., NG-RAN node) on how to control ES in NR cells (see Clause 6.2.3, figure 6.2.3.2.1-1 of TS 28.310 [1]). The Network-element Distributed energy saving functionality within the Network Element, (e.g., within the NG-RAN node) decides how to switch the energy saving states of the cells based on the configured cell load threshold information, neighbour relations, ES policy, etc (see clause 15.4 in TS 38.300 [3]).

Observation #2:

The following are the various actors are mentioned in TS 28.310 [1] related to this scenario, with many functionalites overlapping:

-	NR cells with MnS producer(s) (clause 5.1.3.2.2 of TS 28.310 [1])

-	NR capacity booster cell (clause 5.1.3.2.2 of TS 28.310 [1])

-	candidate cell (clause 5.1.3.2.2 of TS 28.310 [1])

-	gNB providing the capacity booster and MnS producer(s) (see clause 5.1.3.2.3 of TS 28.3102 [1])

-	centralized or distributed ES management (clause 5.1.3.3 of TS 28.310 [1])

-	The distributed ES function (see clause 5.2.3.3 of TS 28.310 [1]).

-	gNB owning the capacity booster cells (see clause 6.2.1 of TC 28.310 [1])

-	Distributed SON ES solution (see clause 6.2.3.0 of TS 28.310 [1]).

-	Provisioning MnS for NF (see clause 6.2.3.1.1 of TS 28.310 [1]).

-	Distributed SON ES functionality (see clause 6.2.3.1.2.2 of TS 28.310 [1]).

-	Distributed SON ES (see clause 6.2.3.1.3.2 of TS 28.310 [1]).

-	MnS producer of Distributed ES management (see clause 6.2.3.2.1 of TS 28.310 [1]).

-	MnS producer for distributed ES management (see clause 6.2.3.1.3.2 of TS 28.310 [1]).

-	distributed ES function (see clause 6.2.3.1.3.2 and clause 6.2.3.2.2 of TS 28.310 [1]).

-	MnS producer for distributed ES management (see clause 6.2.3.1.3.2 of TS 28.310 [1]).

-	distributed ES management (see clause 6.2.3.2.2 of TS 28.310 [1]).

There are several entities or actors mentioned in TS 28.310 [1] related to this scenario, often with overlapping or unclear functionalities.

Obeservation #2:

Based on the scenario described and above observations:

- There should be an MnS capability for Energy saving management. The MnS supporting this capability can provision cell load, neighbour relations, ES policy, etc on to the Network Element. An MnF playing the role of the MnS Producer supporting this MnS capability for Energy saving management would also play the role of MnS Consumer of a Provisioning MnS Producer of the Managed Elements and Managed Functions, e.g., for NG-RAN to configure cell load, neighbour relations, ES policy, etc.

- There should be a Network-element Distributed energy saving functionality within the Network Element (e.g., within the NG-RAN node) that can perform autonomous switch-off and make the Network element/Network Function (e.g., NR cell) to enter or exit energy saving state, utilizing the configurations set by the management (i.e., MnS supporting Energy saving management capability). The definition of this functionality is not in the scope SA5 and is defined by other WGs (e.g., 3GPP RAN WGs for RAN or 3GPP SA2 for Core).

Scenario 3: A Energy Saving and Energy Efficiency optimization scenario, where an MnS capability for Energy Saving and Energy Efficiency optimization (in the management system) optimizes Energy Efficiency across multiple NG-RAN nodes and NR cells by deciding grouping of cells that should be considered together for energy saving and the sequence in which cells should enter or exit energy saving state.

The MnS capability for Energy Saving and Energy Efficiency optimization configures the results onto the other functionalities that control energy savings in individual Network Elements or Network Function (e.g., cells) (see Clause 5.1.3.5).

Observation #3:

The following is the actor mentioned in TS 28.310 [1] related to this scenario:

-	energy saving function (see clause 5.1.3.5 of TS 28.310 [1]).

Conclusion #3:

Based on the scenario described and above observations:

- There should be an MnS capability for Energy Saving and Energy Efficiency optimization. The MnS supporting this capability optimizes energy saving and energy efficiency across multiple Network Elements and Network Functions (e.g., cells) by configuring information the other functionalities that control individual Network Elements and Network Functions (e.g., grouping of cells with the sequence in which cells should enter or exit energy saving state).

The following figure summarizes the entities in these scenarios.

Figure 1: Scenarios for EE management (as derived from in TS28.310).

Proposal

From the scenarios, observations and conclusions described above, there is a need to study and define the MnS capabilities in Energy Savings and Energy Efficiency management scenarios for 6G. This pCR proposes architectural framework outlining the scenarios, principles and entities in Energy Savings and Energy Efficiency management architecture for 6G, while considering the support for all the scenarios in 5G and allowing enhancements in the 6G for further energy saving and energy efficiency scenarios.

Proposed Changes

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[A]	3GPP TS 38.914: "Study on 6G Scenarios and requirements".

* * * Next Change * * * *

# 5	6G Management Architecture Principles

## 5.K	Energy efficiency management architectural framework

The 6G management system should support the following Energy Saving (ES) and Energy Efficiency (EE) management scenarios. This includes the MnS capabilities that need to be supported in 6G to enable the support of the same set of capabilities as in 5G, while allowing extensions for 6G.

1)	A Centralized energy saving scenario, to save energy in the network, that includes:

-	an MnS capability for Energy saving management (in the management system) that can make Network Elements and/or Network Functions (e.g., 6G Cell, 6G NF etc) to enter or exit energy saving state directly based on configured control parameters (e.g., traffic load thresholds). Any MnF playing the role of the MnS Producer supporting this MnS capability for Energy saving management and would also play the role of MnS Consumer of a Provisioning MnS Producer and a Performance Assurance MnS Producer of the Managed Elements and Managed Functions, e.g., for RAN and Core.

2)	A Distributed energy saving scenario, to save energy in the network, that includes:

-	a Network-element distributed energy saving functionality within the Network Element (e.g., within the RAN node) that can perform autonomous switch-off and transition the Network element/Network Function (e.g., cells) to enter or exit energy saving state. The definition of this functionality is not in the scope SA5 and is defined by other WGs (e.g., 3GPP RAN WGs for RAN or 3GPP SA2 WG for Core).

-	an MnS capability for Energy saving management (in the management system) that can configure the Network Element or Network Function (e.g., NG-RAN Node) with load thresholds, ES policy, etc, to be utilized by Network-element distributed energy saving functionality to achieve energy saving. Any MnF playing the role of the MnS Producer supporting this MnS capability for Energy saving management and would also play the role of MnS Consumer of a Provisioning MnS Producer of the Managed Elements and Managed Functions, e.g., for RAN and Core.

3)	An Energy Saving and Energy Efficiency optimization scenario, to optimizes network energy saving and energy efficiency, that includes:

-	an MnS capability for Energy Saving and Energy Efficiency optimization (in the management system) responsible optimizing the energy efficiency and improving energy savings across multiple Network Elements and Network Functions (e.g., RAN nodes, cells, Core Network Functions etc), and supports optimizing the configuration used in the Centralized energy saving scenario and Distributed energy saving scenario (e.g., by collecting and analysing the performance of the Network Elements and/or Network Functions such as traffic loads, identifying a group of cells with coverage overlaps for energy saving, and identifying the sequence in which these Network Elements or Network Function should enter or exit energy saving state etc). Any MnF playing the role of the MnS Producer offering this MnS capability for Energy saving and energy efficiency optimization and would also play the role of MnS Consumer of an MnS Producer supporting MnS capability for Energy saving management, a Provisioning MnS Producer and a Performance Assurance MnS Producer of the Managed Elements and Managed Functions, e.g., for RAN and Core.

Based on the scenarios described, the following principles are applicable and to be supported for 6G Energy Saving and Energy Efficiency:

Principles K-1: The 6G system should support a Distributed Energy Saving functionality (local to the Network Element) that can make the Network Elements or Network Functions (e.g., cells at its local RAN Node or NFs in the core) to enter or exit energy saving state. This functionality is specified by 3GPP RAN WGs for RAN nodes and 3GPP SA2 WGs for Core NFs. It has the scope of only one Network Element (e.g, a single RAN Node) and hence it cannot be assumed to account for impacts across multiple Network Elements (e.g., across multiple RAN Nodes in the network). It can be configured with inputs by the OAM to enable the triggering of ES (e.g., cell load thresholds, ES trigger time thresholds, etc) which it utilizes to make ES decisions for the Network Element or Network Function (e.g., cells).

NOTE:	Clause 5.2 of TR 38.914 [A] captures the 6G requirement in RAN to enable lower energy consumption, hence it can be assumed in the management specifications that the 6G network will continue to support the local Distributed ES management functionality.

Principles K-2:  The 6G system should support an MnS capability for Energy Saving management that can manage energy saving across several Network Elements and Network Functions by:

1)	provisioning the individual Network Elements or Network Functions (e.g., cells) to enter or exit energy saving state, or

2)	configuring the Network Elements or Network Function (e.g., RAN Nodes) with thresholds, to be utilized by Distributed ES functionality in the Network Element or Network Function (e.g., RAN Nodes) for deciding how to make Network Elements or Network Functions (e.g., cells) to enter or exit energy saving state.

The two energy saving actions described above eventually configure the Network Elements or Network Functions, for example, in 1) trigger thresholds for energy saving states in the RAN Node are configured, and in 2) cells or the Core NFs are configured with different values for energy saving state. Hence, there should be a common MnS capability that can be deployed as needed.

This MnS capability for ES management can be specific to a single scope (e.g., for RAN or Core) or provide combined centralized functionality for multiple scope (e.g., for both RAN or Core, at a PLMN level etc).

Principles K-3:  The 6G system should support an MnS capability for Energy Saving and Energy Efficiency optimization that can provide configurations to the MnS Producer supporting MnS capability for Energy saving management with policies (e.g., grouping of cells for energy saving with the sequence in which cells should enter or exit energy saving state, the thresholds to be used for evaluating energy saving decisions etc) optimizing how ES management is executed.

Any MnF playing the role of the MnS Producer offering this MnS capability for Energy saving and energy efficiency can utilize the automation capabilities to decide (e.g., cell overlaps based on cell coverage, energy saving trigger thresholds based on performance measurements etc.) and enable the Energy Saving functionalities with configurations on how energy saving is executed in the network. This can be specific to a single scope (e.g., for RAN or Core) or provide combined centralized functionality for multiple scopes (e.g., for both RAN or Core, at a PLMN level etc).

The functionalities to be supported for the 6G energy efficiency and energy saving scenarios described in this clause are illustrated in Figure 5.K-1.

Figure 5.K-1: 6G ES and EE management architectural assumptions

* * * End of Changes * * * *