3GPP TSG-SA5 Meeting #165	S5-260287

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	Pseudo-CR on TR 32.801-01 New use case on network issue handling management

Document for:	Approval

Agenda item:	6.20.6

Spec:	3GPP TR 32.801-01

Version:	0.0.0

Work Item:	FS_6G_OAM

Comments

The contribution proposes to add use case for network issue handling management, addressing performance issues (e.g., weak coverage, interference) and faults (e.g., hardware failures) to ensure service continuity. It enables operators to detect anomalies, diagnose root causes, and trigger self-healing actions, improving operational efficiency through intelligent automation.

Proposed Changes

* * * First Change * * * *

6	6G Management Scenarios

6.x	Management scenarios#1: Network issues handling

6.x.1	Description

Network issues, including network performance and fault management are the critical concern for operators. These issues may affect the normal operation of the network and the quality of service. The operators need to ensure that network issues can be automatically identified, and network services can recover automatically, enabling operators' real-time intelligent diagnosis, predictive analytics, and autonomous recovery, ensuring seamless network operations and superior Quality of Service (QoS).

This use case considers intelligent network diagnosis and self-recovery to restore operators 6G service, leveraging machine learning, data analytics, and closed-loop automation across two key scenarios:

- Network performance issues diagnostic analysis and recovery, ensuring that requested network issue can be diagnosed and recovery upon the Operator’s requirements; The network performance can be weak coverage, overlapping coverage, high interference, low uplink/downlink traffic rate, high network load, voice quality problems (low access rate, high call drop rate, low handover success rate, high packet loss rate), etc.

- Fault diagnostic analysis, prediction and recovery, maintaining the uninterrupted availability of essential network services, such as data connectivity service and voice service. Fault including but not limited to radio frequency unit failure, wireless link failure, etc.

Following Figure 6.x.1-1 illustrates the end to end service flows:

Figure 6.x.1-1: Network issue handling management

- The 6G OAM systems detects anomalies (e.g., KPI degradation: high interference, poor coverage), then report the anomalies to Operator.

- Operator reviews the anomalies report and performs issues identification, then requests issue diagnostics and recovery to 6G OAM systems

- The 6G OAM systems perform issue diagnostic analysis and recovery and report the issue diagnostic analysis and recovery result to Operator. For example, Issue ID, Root Cause: Weak Coverage: Building shadowing (RSRP <-115dBm), Overlapping Coverage: 3 cells with PCI mod3 conflict, Actions: Adjusted down tilt (Cells 12/15), Optimized PCI (Cells 8/9/11 → new PCI 42/45/48).

- The monitoring of progress or status for the network issue diagnostic and recovery by the Operator to make sure whether the PMs and KPIs of the network are satisfying or not.

6.x.2	Potential Requirements

[PR.6.x.2-1] The 6G OAM system shall be able to support intelligent network issues identification and prediction including Network performance issues and fault issues.

[PR.6.x.2-2] The 6G OAM system shall be able to support mechanisms for operator to provide the on-demand diagnotic and recovery from network issues.

[PR.6.x.2-3] The 6G OAM system shall be able to monitor the progress or status for the network issue diagnostic and provide the network issue diagnostic and recovery result report for the operator.

* * * End of Changes * * * *