3GPP TSG SA5 Meeting #165	S5-260298

Goa, India, 9-13 February 2026

Source:	Samsung

Title:	Autonomous software management.docx

Document for:	Approval

Agenda item:	6.20.10

Spec:	3GPP TR 28.884

Version:	V0.2.0

Work Item:	FS_SBMA_Ph4

Comments

This pCR provides the UC for autonomous software management for VNF

Proposed Changes

* * * First Change * * * *

## 5.x	Use case#4 on policy driven software management for 5G

### 5.x.1	Description

In large-scale 5G vRAN deployments, a typical site comprises hundreds of Distributed Units (DUs), multiple Centralized Units (CUs), and mixed hardware generations operating with diverse software baselines, as well as multi-vendor NEs. For instance, a single urban macro site may deploy over 200 DUs (O-DUs) across multiple radio sectors, 4-8 CUs (O-CUs) serving these DUs, and varying hardware platforms (Intel Xeon, AMD EPYC, Arm-based edge servers). Additionally, multi-vendor NEs (e.g., Vendor-x DU, Vendor-y DU) may run software baseline versions ranging from v18.0.x and v25.1.y. This complex deployment is depicted in the figure below:

DU (20K)

Control Plane 1

Control Plane n

Management

Interface

User Plane 1

(Intel Xeon)

User Plane n

Management

Interface

…

User Plane 2

(AMD EPYC)

…

DU

Software Version

= v18.0.x

DU

Software Version

= v25.1.y

…

ACPF

AUPF

NG-Uu

X2-C

E1

F1-C

F1-U

S1-U

gNB

5G UE

VSM

SGW/MME

CU

ACPF: Access Control Plane Function

AUPF : Access User Plane Function

CU: Centralized Unit

DU: Distributed Unit

gNB: Next Generation Node B

MME: Mobility Management Entity

S-GW: Serving Gateway

VSM: Virtual Software Management

Architecture with various Software Components

Applying the Software Management IRP (SwM IRP, 3GPP TS 32.532) concepts defined for 4G to 5G VNFs software management implies that during a software upgrade campaign, the Orchestrator (IRPManager or MnS Producer) must identify the target software package for each MF type/model and maintain awareness of each MF's current software state using operations like `SwMProfile` and `listSwMCapabilities`. The Orchestrator must maintain real-time awareness of each MF's state, and explicit per-MF control is required for each upgrade operation. In the scenario where a single urban macro site deploys over 200 DUs across multiple radio sectors and is managed by a single orchestrator, the process becomes particularly demanding. If a new version of the vDU package becomes available, the Orchestrator must connect to each MF individually to initiate/configure the package upgrade trigger. The Orchestrator also need to ensure the successful completion of the upgrade process for each MF, including performing retry or repair operations as needed.

Further, since the logic is centralized at the orchestrator, if the Orchestrator crashes during an upgrade, all MFs may freeze mid-process. The recovery would necessitate manual state reconstruction for each MF making the entire process of software management inefficient.

This use case calls for a distributed and more resilient software management process when network nodes are made self-aware about when and how to upgrade the software. This involves configuring the conditions (based on time, network performance, exiting traffic), when satisfied, would result in an initiation of software upgrade process. This would also involve, configuring policies (e.g automatic activations, version constraints, rollback criteria) to be considered for an efficient software upgrade.

### 5.x.2	Potential requirements

Note: All the requirements defined for PNF, in clause 5.4.2.2, are valid here too.

REQ-SBMA-VNFSWM-1 The 3GPP management system shall support the capability to enable policy based autonomous SW management including (not limited to) SW initiation, download, activate, monitor, rollback.

REQ-SBMA-VNFSWM-2 The 3GPP management system shall support the capability of distributed software management.

### 5.x.3	Potential solutions

The proposed solution is to introduce a policy‑driven software management model. The orchestrator (or SMO) continues to act as the system of record for topology and high‑level policy, but instead of orchestrating step‑by‑step upgrades on each NE, it performs the following:

The Orchestrator (as MnS Consumer) publishes new software packages to a central software repository (as MnS Producer) and update a manifest/metadata describing packages and their constraints. The following information is included in the repository.

- repositoryId: Unique repository identifier

- repositoryUri: HTTPS/Git endpoint URL

- availablePackages: Indexed packages with metadata of type packageInfo. The availablePackages list should not merely point to binaries; it must include package packageInfo metadata necessary for autonomous NE decisions and validation.

- accessControlPolicy: Authentication/authorization rules

- packageManifest: Declarative list of all packages (CICD/GitOps-style)

- repositoryAccessCredentials: TLS cert/auth token for repository access

The Orchestrator configures Managed Function (MF) with the upgrade policies that are used to automate the software upgrade process.

  - Each MF is configured with upgrade triggers. Upgrade triggers defines the conditions that, when fulfilled, activate the software upgrade process. Various trigger conditions are defined as follows:

    - If all health checks pass AND every-30mins

    - After particular Health Recovery Events: NE recovers from critical alarm state (e.g. resource usage).

    - At a particular Network Condition Improvements: Backhaul bandwidth availability > threshold (e.g., >100Mbps free).

    - At a particular Maintenance Window Entry: Current time enters configured Maintenance window.

    - After receiving an External Signals:  Peer NE (e.g., CU) completes the upgrade.

  - When the conditions are fulfilled the software upgrade process is activated to check if a newer, applicable package exists. The UpgradePolicyProfile is used to configure information that is used to determine the suitable software package. If a suitable package is found the NE autonomously activate the upgrade policies and identify the appropriate software version, downloads the package from the repository, validates and installs it. The UpgradePolicyProfile would include the following information:

    - policyId: Policy identifier

    - nEFilters: NE types/models this policy applies to NE. This would apply when the UpgradePolicyProfile is name contained in Subnetwork including multiple NE/MF. This attribute can be used to find the appropriate MF, within Subnetwork, on which the policy shall be activated.

    - maxUpgradeFrequency: Min time between upgrades (prevent churn). If a CI/CD pipeline pushes multiple minor builds (v1.1.0 → v1.1.1 → v1.1.2) in quick succession, unbounded autonomy would make NEs upgrade repeatedly in hours, consuming bandwidth, causing instability, and saturating maintenance windows. So, maxUpgradeFrequency param can be used to avoid such churn.

    - preUpgradeValidations: Health checks before upgrade. Defines health gates before upgrade operations proceed. Examples: Resource health: CPU < 70%, memory < 80% . System readiness: No critical alarms active. Network readiness: Backhaul ≥ 100 Mbps free. Storage checks: Minimum free disk ≥ image size × 1.5. These validations prevent upgrades on nodes that are degraded, overloaded — essentially embedding safety preconditions into local NE logic.

    - versionConstraints: Allowed version transitions (e.g., v1.x → v2.x only)

    - maxUpgradeWindow: Max time available for upgrade from NE perspective. This defines maximum time allowed to finish one upgrade once started. If the upgrade can’t be finished in that specified time then abort and rollback should be auto-performed.

    - rollbackThreshold: Conditions triggering auto-rollback. Defines health gates after upgrade operations is completed. Examples: Resource health: CPU < 70%, memory < 80% . System readiness: No critical alarms active. These validations prevent upgrade to package with issues and rollback to previous version if any issue is found.

### The overall procedure is depicted in the figure below:

- Create New SW Repository (SoftwareRepository IOC)

- The Orchestrator initiates the process by creating a new software repository.

- The Central SW Repository (Git/Artifact Store) acknowledges the creation.

- Publish Software Package (SoftwareRepository IOC)

- The Orchestrator publishes various software packages and updates the manifest in the Central SW Repository (Git/Artifact Store).

- The Central SW Repository acknowledges the publish and confirms the manifest update.

- Configure Policy and Trigger Condition (SoftwareManagementCtrl IOC)

- The Orchestrator defines the `upgradePolicyProfile` and `triggerConditions`.

- The Orchestrator sends the configured policy and trigger conditions to all available NEs individually (DU1 and DU2).

- New Software Package Version becomes available (SoftwareRepository IOC)

- The Orchestrator publishes the new software package version along with the manifest in the Central SW Repository (Git/Artifact Store).

- The Central SW Repository acknowledges the publish and confirms the manifest update.

- Wait for Trigger Conditions to be Satisfied

- #Trigger Condition 1: Periodic Polling

  - DU #1 and DU #2 independently query the Central SW Repository to check for all available software packages.

  - Central SW Repository responds with the list of available packages.

- #Trigger Condition 2: New Package Notification

  - Both DUs (DU1 and DU2) subscribe to the Central SW Repository for notifications about new package availability.

  - When a new package is uploaded to the Central SW Repository, it notifies list of available packages to all subscribed NEs (DU1 and DU2).

- NE Local Decision: Upgrade or Not. In case of upgrade, find the appropriate package.

- Each NE evaluates the upgrade policy and its current health status from SoftwareManagementCtrl IOC.

- If a new applicable version is found and the health status and upgrade window are acceptable:

  - DU #1 and DU #2 decide to proceed with the upgrade based on the UpgradePolicyProfile.

- Else (no applicable version or policy/health blocking):

  - DU #1 and DU #2 defer the upgrade.

- Autonomous Download

- DU #1 autonomously downloads the package from the Central SW Repository using the provided `packageId` and `repoUri`.

- DU #1 verifies the checksum and signature of the downloaded package.

- DU #2 performs the same steps: autonomously downloads the package, verifies the checksum, and signature.

This flow highlights a decentralized approach where each NE autonomously decides and performs the upgrade based on predefined policies and conditions, reducing dependency on a centralized Orchestrator and mitigating risks associated with orchestrator failures.

### 5.x.4	Evaluation of potential solutions

* * * Next Change * * * *

* * * End of Changes * * * *