3GPP TSG-SA5 Meeting #165	S5-260306

Goa, INDIA 9 - 13 Febrary 2026

Source:	CATT

Title:	Pseudo-CR on Add new use case and requirements on distributed inference

Document for:	Approval

Agenda item:	 6.20.2

Spec:	3GPP TR 28.882

Version:	0.2.0

Work Item:	FS_AIML_MGT_Ph3

Comments

This pCR proposes to add use case and requirements on distributed inference to TR 28.882.

Proposed Changes

* * * First Change * * * *

### 5.5.1	Use cases

#### 5.5.1.x	Management capabilities enhancements for AI/ML inference

#### 5.5.1.x.1	Management support for distributed inference

#### 5.5.1.x.1.1	Description

When AI/ML inference is performed in a centralized way, a substantial volume of real-time operational data needs to be collected and transported to a central inference point. Such centralization may lead to data transport bottlenecks and increased end-to-end latency, which can adversely affect the timeliness and efficiency of analytics and closed-loop automation.

Distributed inference refers to a process of collaborative execution of a trained ML model or a set of ML models across multiple AI/ML inference functions. AI/ML inference functions, such as MDAF, NWDAF or RAN functions,  use one or more ML models which support distributed inference may in cooperation with other inference functions, in a hierarchical ordisaggregated or disaggregated manner, where inference is performed on locally available data and inference outputs are produced as part of a distributed inference task.

Management of distributed inference can be applied to several AI/ML-based use cases. For instance, a NWDAF acting as a FL server may operate within a central core network function, while distributed inference may be used at edge gateways or regional NWDAFs acting as FL clients to process local traffic data and then provide summaries to the central NWDAF.  In the OAM and RAN, to improve network performance and user experience, e.g., for Network Energy Saving, Load Balancing and Mobility Optimization, the AI/ML inference function and the associated ML model(s) may be utilized in a distributed way.

When receiving an AI/ML inference request, the MnS producer may evaluate whether distributed inference is needed according to the inference requirements provided by the AI/ML inference consumer, and it is up to the MnS producer to determine, based on the potentail requirements provided by the consumer, appropriate inference function(s) which may need to participate in the AI/ML model inference.

Distributed inference introduces additional considerations, including the propagation of errors or delays between inference functions and the impact of heterogeneous computational capabilities across different nodes.

Given the inherently distributed nature of the 3GPP mobile network architecture, the management system is required to provide management support for distributed inference.

NOTE 1: 		How ML model(s) and data exchange between different inference functions depends on implementation which is not in scope for standardization.

NOTE 2: 		Data exchange between involved inference functions should be in the security tunnel with appropriate authentication and authorization mechanisms.

#### 5.5.1.x.1.2	Potential Requirements

REQ-DIST_MGMT-01: The AI/ML inference MnS Producer should have a capability allowing an authorized consumer to quiry the capability pertaining to distributed inference.

REQ-DIST_MGMT-02: The AI/ML inference MnS Producer should have a capability allowing an authorized consumer to provide distributed inference requirements to the MnS.

REQ-DIST_MGMT-03: The AI/ML inference MnS Producer should have a capability to provide AI/ML inference report on AI/ML inference function(s) participating in distributed inference.

#### 5.5.1.x.1.3	Possible solutions

TBD

#### 5.5.1.x.1.4	Possible solutions evaluation

TBD

* * * End of Change * * * *