3GPP TSG SA5 Meeting #165	S5-260314

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	Rel-20 pCR on TR 28.889 Add use case for enhancement of metric-value conflicts avoidance and detection

Document for:	Approval

Agenda item:	6.20.10

Spec:	3GPP TR 28.889

Version:	0.2.0

Work Item:	FS_CCLM_Ph2

Comments

In CCL, when the metrics are derived from an intent, the conflict resolution mechanisms mentioned in TS 28.312 can also be used in avoiding concurrent and non-concurrent metric-values conflicts. Thus, it proposes to add use case for metric-value conflicts avoidance and detection.

Proposed Changes

* * * First Change * * * *

## 4.3	Enhancement of metric-value conflicts avoidance and detection

### 4.3.1	Description

Each CCL has a control scope including a set of metrics (e.g. derived from an intent, in this case the metrics and the expected outcomes can be considered as targets). When the metrics are derived from an intent, the conflict resolution mechanisms (mentioned in TS 28.312[3], including conflict resolution based on intent priority, conflict resolution based on intent utility functions and conflict resolution based on pre-emption) can also be used in avoiding concurrent and non-concurrent metric-values conflicts. For example, the CCL (Coordination MnS consumer) may have preference among the metrics. Furthermore, The CCL can have preference for these conflict resolution mechanisms, which includes a preferred conflict resolution mechanism or the application order of two or more conflict resolution mechanisms.

### 4.3.2	Potential requirements

REQ-Monitor-CCL -1: The 3GPP management system should have the capability to allow MnS consumer to specify the preference for different conflict resolution mechanism.

### 4.3.3	Potential solutions

This solution proposes to enhance the existing CCL information models defined in 3GPP TS 28.567 [2].

To support detection and avoidance of potential non-concurrent metric-value conflicts, if the CCL has been pre-configured e.g., by the operator with the expected outcomes, the CCL may register its desired metrics, outcomes, its conflict resolution mechanisms and the preference for these mechanisms with the CCLCoordinationEntity (CCL Coordination MnS producer). This triggers the first evaluation for potential conflict, i.e. whether these metrics and outcomes are likely to conflict with those of another CCL. Then, the CCL Coordination MnS producer can handle the conflicting metrics and outcomes based on the received conflict resolution mechanisms.

To support detection and avoidance of potential non-concurrent metric-value conflicts, it is proposed to:

-	introduce an attribute "preferredConflictResolutionMechanism" needs to be introduced in ClosedControlLoop IOC to represent the preference for different conflict resolution mechanism derived from the intent. If the CCL has been pre-configured e.g., by the operator with the expected outcomes, the CCL may register its desired metrics, outcomes, its conflict resolution mechanisms and the preference for these mechanisms with the CCLCoordinationEntity. This triggers the first evaluation for potential conflict, i.e. whether these metrics and outcomes are likely to conflict with those of another CCL. Then, the CCL Coordination MnS producer can handle the conflicting metrics and outcomes based on the received conflict resolution mechanisms.

### 4.3.4	Evaluation of solutions

### TBD

* * * End of Changes * * * *