3GPP TSG SA5 Meeting #165	S5-260315

# Goa, India, 9-13 February 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.567 | CR | 0012 | rev | - | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.567 Update clause 4.3 to align with the approved pCR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CCLM |  |  |  |  |  | Date: |  |  | 2026-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | R19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | In the approved pCR S5-253861, CCL goals are replaced by the CCL requirements. However, CCL goals still exists in the current TS. Thus, this CR proposes to update clause 4.3 to align with the approved pCR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update clause clause 4.3 to align with the approved pCR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The inconsistence between the approved pCR and TS 28.567. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

## 4.3	Characteristic information of a CCL

### 4.3.1	Overview

A CCL is associated with a set of Characteristic information that describes its properties, behaviours and impact. This information includes CCL requirements, CCL Triggers, CCL actions and action plans as well as CCL scopes.

### 4.3.2	CCL requirements

The CCL requirements are responsible for a set of outcomes that need to be achieved or realized by the CCL. Each expected outcome is called a CCL requirement. A CCL requirement contains for example a metric.

### 4.3.3	CCL actions and actions plans

A CCL action is a configuration change that a CCL can perform over a managed entity such as configuring an attribute of  managed object. A CCL may decide to perform several actions which can be combined in a single CCL action plan.

### 4.3.4	CCL scopes

The scope is the set of managed objects, their properties and network outcomes that are associated with the CCL for measurement, configuration and impact. The scopes for the different CCLs can be managed by the MnS consumer, i.e. they can be defined on to the CCL or revised by the MnS consumer. A CCL may have four scopes: the measurement scope, targeted  scope, control scope, monitored scope and impact scope, defined as follows:

-	measurement scope: the measurement scope is where related measurements are collected

-	control scope: control scope is the scope to which the CCL's actions are desired to be applied, e.g., the set of network functions and attributes that are the planned candidates to be modified by the CCL. The control scope is also called the action-space as it describes the set of candidate actions that the CCL can (is configured to be able to) execute.

-	targeted scope: which relates to purpose of the CCL, it represents the scope to which the CCL's actions are desired to have influence, e.g., it is both the network functions and attributes as well network outcomes like coverage areas that are planned to be influenced by the configuration’s actions of the CCL. -

-	Monitored scope: Monitored scope is the scope which a CCL monitors to see if there are conflicts.

-	impact scope: impact scope is the scope to which the CCL's actions have influence, e.g., it is both the network functions and attributes as well network outcomes like coverage areas that are influenced by the configuration actions of the CCL. This is different from the measurement scope, i.e. the scope where the CCLs measure and control scope, i.e. the scope where they act.

The impact scope may be known and bounded or unbounded and thus unknown - see figure 4.3.4-1. The bounded scope indicates that the area known by the CCL is the scope where its actions will impact. The unbounded impact-scope is the full network scope where the CCL’s action will have impact, but the CCL does have information that its action will have that impact to that scope.

Figure 4.3.4-1: Exemplification of known/bounded vs. unknown/unbounded impact scope: CCL A takes action in cell A expecting impact in cells A, B, C and D. if the impact is strictly in cells A, B, C and D, then the impact scope is known and bounded. However, if the impact scope includes cells E and F, then for the CCL, the true impact scope is unknown and thus unbounded.

==============End of change==============