3GPP TSG-SA5 Meeting #164	S5-260368

Goa, India, 9-13 February 2026

Source:	Ericsson Korea Partners Co Ltd, ZTE Corporation

Title:	Rel-20 pCR TR 28.881 Add evaluation and conclusion to UC#8 (Intent Guarantee)

Document for:	Approval

Agenda item:	6.20.1

Spec:	3GPP TR 28.881

Version:	1.0.0

Work Item:	FS_IDMS_MN_Ph4

Comments

In this contribution, we propose to add the evaluation and conclusions to Use Case #8 (Intent Guarantee)

This pCR is related to WT-3.1.

Proposed Changes

* * * First Change * * * *

#### 4.8.3.2	Potential solution #2

Enhancement Aspect 1:

A new attribute whose type is Context <<datatype>>  is proposed to be added to the Intent <<IOC>> and IntentExpectation <<dataType>>, respectively.

The presence of this new context indicates that the associated requirements should be guaranteed by the MnS producer. When the context is set at the intent level (intentContext), it indicates all expectation within that intent should be guaranteed. When it is set only to some of the intent expectations (expectationContext), it indicates only those expectations should be guaranteed.

When the new context is present at both the intent level (intentContext) and the intent expectation level (expectationContext), both should be taken into account. A guaranteePeriod defined at the intent level applies to all guaranteed intent expectations within that intent. If additional guaranteePeriod values are defined for a specific intent expectations, they are considered in addition to the intent-level guarantee period.

The new guarantee period context specifies the time intervals for which the MnS Producer shall evaluate the confidence level of fulfilment, as explained in clause 4.8.1.

Allowed values:

-	contextAttribute: "guaranteePeriod"

-	contextCondition: "IS_ALL_OF"

- contextValueRange: a list of SchedulingTime <<choice>> defined in TS 28.622 [6].

Enhancement Aspect 2:

The confidence level that is calculated by the MnS Producer considering all guarantee periods should be reported as part of the intentFulfilmentReport, following the reporting period set by observationPeriod attribute of IntentReportControl <<dataType>>.

A new optional attribute guaranteeConfidenceLevel is proposed to be added to the IntentFulfilmentReport <<dataType>> and to the ExpectationFulfilmentResult <<dataType>>, to indicate the guarantee confidence level calculated by the MnS Producer.

Even though the guarantee level is reported as part of the fulfilment report, the reported value shall be interpreted as the confidence level associated with the guaranteed expectations. For example, if an intent expectation is defined as a guarantee expectation for a maximum delay of 10 ms, and the corresponding reported confidence level is 90%, this indicates that there is a 90% confidence that the latency will remain below 10 ms during the guarantee periods, even if the current observed fulfilment temporarily exceeds 10 ms.

The new attribute is provided when the corresponding intent or intent expectation is set for guarantee, i.e., when guarantee period is set either for the whole intent or for specific intent expectations. When the guarantee period is defined at intent level, the guaranteeConfidenceLevel reported at the intent level represents the MnS Producer’s confidence in fulfilling all intent expectations within that intent during the applicable guarantee period. When additional guaranteePeriod values are defined for specific intent expectations, the guaranteeConfidenceLevel reported for those expectations represents the confidence level calculated for those individual expectations, considering all guarantee periods applicable to them (including both the intent-level and expectation-level guarantee periods, if both are present).

* * * Next Change * * * *

### 4.8.4  Evaluation of potential solutions

For UC#8, two potential solutions have been identified. Both the present solutions are feasible, but potential solution #2 can cover potential solution #1 with some future enhancements.

* * * Next Change * * * *

## 5.8 Use case #8: Support to express guarantee requirements in an intent

The use case of Support to express guarantee requirements in an intent is described in clause 4.8 and following management capabilities are identified:

-	The capability enabling an MnS consumer to express an intent to be guaranteed and which expectations requirement in an intent should be guaranteed.

-	The capability enabling an MnS consumer to specify the guarantee period.

-	The capability enabling an MnS consumer to obtain the confidence level of fulfilling the guaranteed expectations at time of reporting.

It is recommended to enhance the generic intent model as per the potential solution #2 in the clause 4.8.3.2 to support above identified management capabilities:

- Add a new guarantee period context (of type Context <<datatype>>) to the Intent <<IOC>> and IntentExpectation <<dataType>>, respectively. The new guarantee period context specifies the time intervals for which the MnS Producer shall evaluate the confidence level of fulfilment.

  - Allowed values:

    - contextAttribute: "guaranteePeriod"

    - contextCondition: " IS_ALL_OF "

    - contextValueRange: a list of SchedulingTime <<choice>> defined in TS 28.622 [6]

- Add a normative text clarifying that the confidence level shall be calculated by the MnS producer and reported as part of the intentFulfilmentReport, following the reporting period defined by the observationPeriod attribute of IntentReportControl <<datatype>>.

- Add new attribute guaranteeConfidenceLevel to the IntentFulfilmentReport <<dataType>> and to the ExpectationFulfilmentResult <<dataType>> to indicate the predicted guarantee confidence level calculated by the MnS Producer for the next observation period. Reuse of FulfilmentInfo << dataType >> may be considered, since both IntentFulfilmentReport <<dataType>> and ExpectationFulfilmentResult <<dataType>> have attributes of this type.

* * End of Changes * * * *