3GPP TSG-SA5 Meeting #165	S5-260405

Goa, India, 9-13 February 2026

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.158 | CR | 0153 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Type and reason definition for Conflict error code |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSBMA |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of Type and reason definition for Conflict error code 409 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Incase of attribute modification conflict error code 409 shall return type and reason and for resource deletion shall return type. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No clarity on consumer probable action on receiving conflict error code |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.4, 6.6.5.3.1 , 6.6.5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

### 6.6.4	The "type" property

The "type" property provides high level error information allowing to complement HTTP 4xx and 5xx error codes in case this is necessary or desired. It provides more details on the nature of the problem than the HTTP error codes. Problem types are specified for the following error response codes.

-	400 Bad Request

-	403 Forbidden

-	422 Unprocessable Content

-	500 Internal Server Error

-	503 Service Unavailable

Note that some error codes convey already all information that can be conveyed. For example, the "404 Not Found" status code indicates that the target resource does not exist or has no current representation. It is hard to see which information should be added to make the error response more helpful for the MnS Consumer.

The "type" property is an enumeration of string values. A MnS Producer should use the following values. Other values may be used as well if deemed more appropriate for specific errors.

-	VALIDATION_ERROR (HTTP error code: 400 Bad Request): The request message does not validate and cannot be processed. Validation refers to two aspects: Validation of the received request message against the JSON schema definition of the request message, and validation of the information model state after applying the requested changes against the JSON schema definition of the information model, for example, if a new instance of a certain object class is allowed to be contained under the class of the specified parent object.

-	REQUEST_OBJECT_TREE_MISMATCH (HTTP error code: 422 Unprocessable Entity): The request message is well formed and understood but cannot be completed due to the current state of the object tree on the MnS Producer. For example, this reason is used when an object is requested to be created below a parent object that does not exist.

-	IE_NOT_FOUND (related error code: 400 Bad Request): The information element (object, attribute, attribute field, attribute element) requested to be modified does not exist.

-	MODIFICATION_NOT_ALLOWED (HTTP error code: 403 Forbidden): The requested modification is correct and understood but not allowed.

-	RETRIEVAL_NOT_ALLOWED (HTTP error code: 403 Forbidden): The retrieval request is well formed and understood but the retrieval of the specified information is not allowed.

- MODIFICATION_CONFLICT (HTTP error code: 409 Conflict): The requested modification is well formed and understood by the MnS Producer, but it is not allowed due to a conflict with another modification.

- DELETION_CONFLICT (HTTP error code: 409 Conflict): The requested deletion is well formed and understood by the MnS Producer, but it is not possible to delete due to conflict arises from resources other than leaf resources since they are structured with parent-child relations in a hierarchical tree

-	SERVER_LIMITATION (HTTP error code: 500 Internal Server Error): The request is well formed and understood by the MnS Producer, but the MnS Producer cannot satisfy the request due to server limitations.

-	SERVICE_DISABLED (HTTP error code: 503 Service Unavailable): The MnS Producer has disabled itself and is currently unable or unwilling to handle the request. This condition may occur, for example, in overload situations.

-	APPLICATION_LAYER_ERROR (HTTP error code: 500 Internal Server Error): The request is well formed and understood by the MnS Producer, but the MnS Producer cannot satisfy the request due to application layer issues.

End of 1st change

2nd change

#### 6.6.5.3	Error reasons for attribute manipulations

#### 6.6.5.3.1	JSON Patch and 3GPP JSON Patch

This clause specifies reasons for errors that may occur when attempting to manipulate attributes of existing resources with JSON Patch and 3GPP JSON Patch. JSON Patch and 3GPP JSON Patch are used for partial resource updates.

This specification defines the following error reasons for use with JSON Patch and 3GPP JSON Patch:

-	NEW_ATTRIBUTE_VALUE_INVALID (related type: VALIDATION_ERROR, 400 Bad Request): The attribute, attribute field or attribute element, as specified in the "path" property, cannot be added, or its value cannot be replaced, as requested, because the value, as specified in the "value" property, is invalid. Valid values are determined by the attribute properties "type", "allowedValues", "multiplicity", "isOrdered", "isUnique", and "isNullable".

-	NEW_ATTRIBUTE_NAME_INVALID (related type: VALIDATION_ERROR, 400 Bad Request): The attribute or attribute field cannot be added as requested, because its name, as specified in the "path" property, is invalid.

-	NEW_ATTRIBUTE_PARENT_NOT_FOUND (related type: REQUEST_OBJECTS_MISMATCH, 422 Unprocessable Entity): The attribute or attribute field cannot be added as requested, because its parent, as specified in the "path" property, does not exist.

-	ATTRIBUTE_NOT_FOUND (related type: IE_NOT_FOUND, 400 Bad Request): The attribute or attribute field cannot be removed, moved, copied, or is value cannot be replaced, as requested, because the "path" or "from" property identifies an attribute or attribute field, that does not exist.

-	ATTRIBUTE_ELEMENT_NOT_FOUND (related type: IE_NOT_FOUND, 400 Bad Request): The attribute element cannot be replaced, removed, moved, or copied, because the "path" or "from" property identifies an attribute element, that does not exist.

-	ATTRIBUTE_INDEX_BAD (related type: IE_NOT_FOUND, 400 Bad Request): The attribute element cannot be added at the specified array location as requested, because the array element index specified in the "path" property is greater than the number of elements in the array.

-	FINAL_MV_ATTRIBUTE_VALUE_INVALID (related type: REQUEST_OBJECTS_MISMATCH, 422 Unprocessable Entity): The attribute element, as specified in the "value" property cannot be added to or removed from the multi-valued attribute as requested, because this would result in an invalid value, according to the attribute properties "multiplicity" or "isUnique". The attribute element itself is valid.

-	ATTRIBUTE_NOT_WRITABLE (related type: MODIFICATION_NOT_ALLOWED, 403 Forbidden): The attribute or attribute field cannot be added, removed, or moved, or its value cannot be replaced, as requested, because the attribute or attribute field is not writable by MnS Consumers, according to the attribute property "isWritable".

-	ATTRIBUTE_INVARIANT (related type: MODIFICATION_NOT_ALLOWED, 403 Forbidden): The attribute or attribute field cannot be added, removed, or moved, or its value cannot be replaced, as requested, because the attribute or attribute field is invariant, according to the attribute property "isInvariant".

- OP_UNKNOWN (related type: VALIDATION_ERROR, 400 Bad Request): The patch operation specified by the "op" property is not known by the MnS producer. This situation may occur, for example, when a patch operation is not supported or wrongly spelled.

- RESOURCE_LOCKED_FOR_CHANGES (related type: MODIFICATION_CONFLICT, 409 conflict): The attribute or attribute field cannot be added, removed, or moved, or its value cannot be replaced, as the attribute or attribute field is locked for changes

End of 2nd  change

3rd change

##### 6.6.5.3.2	JSON Merge Patch, 3GPP JSON Merge Patch and PUT

This clause specifies reasons for errors that may occur when attempting to manipulate attributes of existing resources with JSON Merge Patch, 3GPP JSON Merge Patch and PUT. JSON Merge Patch and 3GPP Merge JSON Patch are used for partial resource updates. PUT is used for complete resource updates.

The following error reasons are defined for use with JSON Merge Patch, 3GPP JSON Merge Patch, and PUT:

-	NEW_ATTRIBUTE_VALUE_INVALID (related type: VALIDATION_ERROR, 400 Bad Request): One or more attributes or attribute fields cannot be added, or their values cannot be replaced, as requested, because the received value is invalid. Valid values are determined by the attribute properties "type", "allowedValues", "multiplicity", "isOrdered", "isUnique", and "isNullable". The "badAttributes" property provides the path to these attributes and attribute fields.

-	NEW_ATTRIBUTE_NAME_INVALID (related type: VALIDATION_ERROR, 400 Bad Request): One or more attributes or attribute fields cannot be added as requested, because the received attribute or attribute field name is invalid. The "badAttributes" property provides the path to these attributes and attribute fields.

-	ATTRIBUTE_NOT_WRITABLE (related type: MODIFICATION_NOT_ALLOWED, 403 Forbidden): One or more attributes or attribute fields cannot be added or removed, or their values cannot be replaced, as requested, because the attributes or attribute fields are not writable by MnS Consumers, according to the attribute property "isWritable". The "badAttributes" property provides the path to these attributes and attribute fields.

-	ATTRIBUTE_INVARIANT (related type: MODIFICATION_NOT_ALLOWED, 403 Forbidden): One or more attributes or attribute fields cannot be added or removed, or their values cannot be replaced, as requested, because the attributes or attribute fields are invariant, according to the attribute property "isInvariant". The "badAttributes" property provides the path to these attributes and attribute fields.

- RESOURCE_LOCKED_FOR_CHANGES (related type: MODIFICATION_CONFLICT, 409 conflict): The attribute or attribute field cannot be added, removed, or moved, or its value cannot be replaced, as the attribute or attribute field is locked for changes

The following additional error reasons are defined for use with JSON Merge Patch and 3GPP JSON Merge Patch:

-	ATTRIBUTE_NOT_FOUND (related type: IE_NOT_FOUND, 400 Bad Request): One or more attribute or attribute fields cannot be removed as requested, because they do not exist. The "badAttributes" property provides the path to these attributes and attribute fields.

End of 3rd   change