3GPP TSG-SA5 Meeting #165	S5-260460

Goa, India, 9 - 13 February 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.540 | CR | 0058 | rev |  | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | - x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.540 Add OAM requirements for RAN3 MWAB features |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AdNRM_Ph4-OAM |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN3 has developed a number of Rel-19 functionalities for the WAB feature, which are documented in TS 38.401. NG connection management (TS 38.401, clause 12.3) and Xn connection management (TS 38.401, clause 12.8) are functionalities that require OAM support, and therefore the definition of new requirements at the 3GPP management system. These requirements are missing. In addition, the procedures for WAB-node configuration (TS 28.531) and WAB-node connectivity to OAM (TS 28.314) are missing. These procedures are however refererred for IAB feature (see clause 4.14). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add WAB-node definition in clause 3.1 Add WAB acronym in clause 3.2. In clause 4.12: - add reference to the WAB-node configuration and OAM connecitivity procedures specified in TS 28.531 and TS 28.314/315, respectively. - add reference to the following functionalities: NG connection management and Xn connection management. In clause 5.1, add the missing requirements on WAB-gNB management for these functionalities. A note is added to clarify that there mobility component may be involved, pending RAN3 LS clarification (S5-255678). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of OAM support for WAB feature functionalities specified in RAN3. Missing references to WAB management procedures describes in other SA5 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2, 4.12, 5.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G Core Network: Defined in 3GPP TS 23.501 [2].

AMF Region: Defined in 3GPP TS 23.501 [2].

AMF Set: Defined in 3GPP TS 23.501 [2].

en-gNB: Defined in 3GPP TS 37.340 [5].

gNB: Defined in 3GPP TS 38.300 [3].

gNB Central Unit (gNB-CU): Defined in 3GPP TS 38.401 [4].

gNB-CU-Control Plane (gNB-CU-CP): Defined in 3GPP TS 38.401 [4].

gNB-CU-User Plane (gNB-CU-UP): Defined in 3GPP TS 38.401 [4].

gNB Distributed Unit (gNB-DU): Defined in 3GPP TS 38.401 [4].

ng-eNB: Defined in 3GPP TS 38.300 [3].

NG-RAN: Defined in 3GPP TS 23.501 [2].

RedCap UE: defined in 3GPP TS 38.331 [10].

eRedCap UE: defined in 3GPP TS 38.331 [10].

AIoT: defined in 3GPP TS 23.369 [11]

AIOTF: defined in 3GPP TS 23.369 [11]

ADM: defined in 3GPP TS 23.369 [11]

IAB-node: as defined in TS 38.401 [4]

WAB-node: as defined in TS 38.401 [4]

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core network

5GS	5G System

ADM	AIoT Data Management

AIOTF	Ambient IoT Function

AMF	Access and Mobility Management Function

EN-DC	E-UTRA-NR Dual Connectivity

EPS	Evolved Packet System

IAB	Integrated Access and Backhaul

MR-DC	Multi-Radio Dual Connectivity

NG-RAN	NG Radio Access Network

NR	New Radio

PCF	Policy Control Function

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF				Unstructured Data Storage Function

WAB				Wireless Access Backhaul

Next change

## 4.12		Management of WAB-gNB

WAB architecture is specified in clause 6.1.7 of 3GPP TS 38.401 [18]. The management of WAB-node includes the node connectivity to management system as specified in TS 28.314 [13] and node configuration as specified in TS 28.531 [6].

A WAB-node consists of a WAB-gNB and a WAB-MT. The WAB-gNB is based on the gNB functionality specified in TS 38.300 [3] and serves UEs by means of a terrestrial NR Uu radio link. The WAB-MT provides PDU sessions for backhauling all WAB-gNB traffic, including Xn, NG and OAM traffic.

The 3GPP management system should provide the configuration parameters to the WAB-node to support management of WAB-gNB. The 3GPP management system should provision configuration parameters based on the current location of the WAB-node. In that case, the continuity of OAM connectivity needs to be ensured as the WAB-node moves (see clause 12.2.1 of TS 38.401 [4]). NG connection management (see clause 12.3 of TS 38.401 [x]) and Xn connection management (see clause 12.8 of TS 38.401 [4]) involving WAB-nodes are also performed based on information configured by the 3GPP management system.

The OAM provides configuration parameters for WAB-gNB including the configuration for activating/deactivating the WAB-gNB operation, and to assist the WAB-gNB providing information used by WAB-MT for the BH PDU Session(s) management via URSP processing.

Note that the name of WAB-gNB and MWAB-gNB are used interchangeably.

Second change

## 5.11	Requirements for management of WAB-gNB

REQ-VMR-CON-001: The 3GPP management system should support IP configuration for WAB-gNB OAM connectivity.

REQ-VMR-CON-002: The 3GPP management system should support TAC/RANAC (re)configuration for WAB-gNB management.

REQ-VMR-CON-003: The 3GPP management system should support activating/deactivating the WAB-gNB operation.

REQ-VMR-CON-004: The 3GPP management system should support configuring policies for service authorization of the WAB-gNB.

REQ-VMR-CON-005: The 3GPP management system should support receiving location information from WAB-gNB.

REQ-VMR-CON-006: The 3GPP management system should ensure continuity of the OAM connectivity for the WAB-gNB, when the WAB-node location is updated.

REQ-VMR-CON-00x: The 3GPP management system should support configuring WAB-gNB with the IP of AMF(s), for the WAB-gNB to establish NG connection(s) towards the AMF(s) serving the UEs.

REQ-VMR-CON-00y: The 3GPP management system should support configuring WAB-gNB to accept/reject Xn connection requests from other WAB-gNBs.

REQ-VMR-CON-00z: The 3GPP management system should support configuring gNB to accept/reject Xn connection requests from WAB-gNBs.

NOTE: Whether REQ-VMR-CON-00y and REQ-VMR-CON-00z need to consider mobility is FFS.

End of Changes