3GPP TSG SA5 Meeting #165	S5-260485

# Goa, India, 9-13 February 2026

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0768 | rev | - | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-20 CR TS 28.552 Measurements related to LTM cell-switch |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | New measurement definitions added related to LTM cell switch for intra and inter DU scenarios. RAN3 SON-MDT WI has agreed on BFR related failures and a network based solution to handle the same (based on support from a large number of companies). So, there is a need and interest in operators to monitor and improve the BFRs during LTM. In order to provide a standardized solution, we propose counters for both success and failure scenarios in the inter-DU and intra-DU cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following measurements are added: - Number of beam failure recoveries shortly after successful inter DU LTM cell switch per beam pair’ - Number of successful inter DU LTM cell switch per beam pair - Number of beam failure recoveries shortly after successful intra DU LTM cell switch per beam pair - Number of successful intra DU LTM cell switch per beam pair |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will not be any possibility for operators to monitor and improve BFR related failures during LTM procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.6.x, 5.1.1.6.x.a, 5.1.1.6.x.b, 5.1.1.6.x.c, 5.1.1.6.x.d |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

5.1.1.6.x Measurements related to LTM cell-switch (intra and inter DU scenarios)

5.1.1.6.x.a   Number of beam failure recoveries shortly after successful inter DU LTM cell switch per beam pair

a)	This measurement provides the number of beam failure recoveries (BFR) shortly after successful inter DU LTM cell switch from beam in the source to beam in the target cell.

b)	CC

c)	On BFR which was executed to beam with id (represented via srb as dedicated sub-counter) which occurred in the time interval since successful inter DU LTM cell switch less than configured threshold (vendor specific). The successful inter DU LTM cell switch is triggered by successful reception of the RRC Reconfiguration complete or first UL message by target DU for previously sent Cell switch command (TS 38.331) from source DU to UE which is signalled by CU to target DU via CU-DU CELL SWITCH NOTIFICATION message (TS 38.473). The measurement is reported in the target DU per beam pair, i.e. beam in the source and beam in the target cell.

d)	A single integer value.

e)	MM.InterDULTMSuccBFRSSB.Ssb, where Ssb represents the sub-counter associated with beam in target cell where BFR was executed.

f)	Beam.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for MRO LTM.

==============Next change==============

5.1.1.6.x Measurements related to LTM cell-switch (intra and inter DU scenarios)

5.1.1.6.x.b   Number of successful inter DU LTM cell switch per beam pair

a)	This measurement provides the number of successful inter DU LTM cell switch from beam in the source to beam in the target cell.

b)	CC

c)	The successful inter DU LTM cell switch is triggered by successful reception of the RRC Reconfiguration complete or first UL message by target DU for previously sent Cell switch command (TS 38.331) from source DU to UE which is signalled by CU to target DU via CU-DU CELL SWITCH NOTIFICATION message (TS 38.473).  The measurement is reported in the target DU per beam pair, i.e. beam in the source and beam in the target cell.

d)	A single integer value.

e)	MM.InterDULTMSuccSSB

f)	Beam.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for MRO LTM.

==============Next change==============

5.1.1.6.x Measurements related to LTM cell-switch (intra and inter DU scenarios)

5.1.1.6.x.c   Number of beam failure recoveries shortly after successful intra DU LTM cell switch per beam pair

a)	This measurement provides the number of beam failure recoveries (BFR) shortly after successful intra DU LTM cell switch from beam in the source to beam in the target cell.

b)	CC

c)	On BFR which was executed to beam with id (represented via srb as dedicated sub-counter) which occurred in the time interval since successful intra DU LTM cell switch less than configured threshold (vendor specific). The successful intra DU LTM cell switch is triggered by successful reception of the RRC Reconfiguration complete or first UL message by DU for previously sent Cell switch command (TS 38.331) from the DU to UE. The measurement is reported in the DU per beam pair, i.e. beam in the source and beam in the target cell.

d)	A single integer value.

e)	MM.IntraDULTMSuccBFRSSB.Ssb, where Ssb represents the sub-counter associated with beam in target cell where BFR was executed.

f)	Beam.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for MRO LTM.

==============Next change==============

5.1.1.6.x Measurements related to LTM cell-switch (intra and inter DU scenarios)

5.1.1.6.x.d   Number of successful intra DU LTM cell switch per beam pair

a)	This measurement provides the number of successful intra DU LTM cell switch from beam in the source to beam in the target cell.

b)	CC

c)	The successful intra DU LTM cell switch is triggered by successful reception of the RRC Reconfiguration complete or first UL message by DU for previously sent Cell switch command (TS 38.331) from DU to UE.  The measurement is reported in the DU per beam pair, i.e. beam in the source and beam in the target cell.

d)	A single integer value.

e)	MM.IntraDULTMSuccSSB

f)	Beam.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for MRO LTM.

==============End of change==============