3GPP TSG SA5 Meeting #165	S5-260541

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	Rel-20 pCR 32.872 Handling of charging information stored in V-CHF

Document for:	Approval

Agenda item:	7.5.2

Spec:	3GPP TR 32.872

Version:	TR V0.2.0

Work Item:	FS_RoamRE_CH

Comments

This pCR proposes to introduce a new solution on Local Breakout Inter CHFs scenario key issue2.

Proposed Changes

* * * First Change * * * *

#### 5.1.4.b	Solution #1.b: Handling of charging data requests stored in V-CHF

The solution for key issue #2 covering requirements REQ-3GPPCH-LBIC-2, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF.

A pre-set conditions (e.g., stored data size threshold or stored duration threshold) may be pre-configured in the V-CHF according to operator's policy. When failure is detected, the V-CHF stores the charging data requests received from V-SMF(CTF) if failure handling is set to 'Continue'.

Editor's Note: Needs further clarification and addition of a message flow.

* * * End of Changes * * * *