3GPP TSG SA5 Meeting #165	S5-260235rev1

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	Rel-20 pCR 32.872 Handling of charging information stored in V-CHF

Document for:	Approval

Agenda item:	7.5.2

Spec:	3GPP TR 32.872

Version:	TR V0.2.0

Work Item:	FS_RoamRE_CH

Comments

This pCR proposes to introduce a new solution on Local Breakout Inter CHFs scenario key issue2.

Proposed Changes

* * * First Change * * * *

#### 5.1.4.b	Solution #1.b: Handling of charging data requests stored in V-CHF

The solution for key issue #2 covering requirements REQ-3GPPCH-LBIC-2, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF.

A pre-set conditions (e.g., stored data size threshold or stored duration threshold) may be pre-configured in the V-CHF according to operator's policy. When failure is detected, the V-CHF stores the charging data requests received from V-SMF(CTF) if failure handling is set to 'Continue'.

-	When the charging session remains active and the pre-set conditions in the V-CHF have not been reached, once the connection with H-CHF is restored, V-CHF sequentially forwards the stored Charging Data Requests to H-CHF.

-	When the charging session remains active but the pre-set conditions in the V-CHF are reached or the charging session is terminated, the V-CHF forwards the stored Charging Data Requests and subsequence requests to local storage.

-

Figure 5.1.4.b-1: Handling flow of charging data requests stored in V-CHF

* * * Next Change * * * *

A.1.x	Figure 5.1.4.b-1

@startuml

<style>

element {

BackGroundColor: white;

}

</style>

hide footbox

participant "V-SMF(CTF)" as CTF

participant "V-CHF" as VCHF

participant "H-CHF" as HCHF

VCHF <--> HCHF : 1. Failures detected between the V-CHF and the H-CHF

rnote over VCHF: 2. V-CHF decides to store\nthe Charging Data Request(s)

CTF -> VCHF : 3. Charging Data Request[Update]

note right : this is the start of  pre-condition

activate VCHF

opt connection re-established between V-CHF and H-CHF

VCHF -> HCHF: 4a. send the stored requests in sequence

end

opt The pre-set conditions in V-CHF are met or the charging session is terminated

VCHF ->? : 4b1. send datas to local storage

deactivate VCHF

note left : this is the end of pre-condition

CTF -> VCHF : Charging Data Request[Terminated]

VCHF ->? : 4b2. send datas to local storage

end

@enduml

* * * End of Changes * * * *