3GPP TSG SA5 Meeting #165	S5-260237rev1

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	Rel-20 pCR 32.872 Use triggers from V-CTF in V-CHF charging request

Document for:	Approval

Agenda item:	7.5.2

Spec:	3GPP TR 32.872

Version:	TR V0.2.0

Work Item:	FS_RoamRE_CH

Comments

This pCR proposes to introduce a new solution on Local Breakout Inter CHFs scenario key issue3.

Proposed Changes

* * * First Change * * * *

#### 5.1.4.c	Solution #1.c: Use triggers from V-CTF in V-CHF charging request

The solution for key issue #1 covering requirements REQ-3GPPCH-LBIC-3, identify the trigger conditions in V-CHF for the Local Breakout N40+N107 charging scenario.

When a Charging Data Request is sent from V-CHF to H-CHF for inter CHFs communication, the trigger condition that triggers the Charging Data Request is from V-SMF(CTF). The value of the Triggers and other parameters are derived from the Charging Data Request previously received from the V-SMF(CTF).

When a Charging Data Response is sent from V-CHF to V-SMF(CTF) for inter CHFs communication, the value of the Triggers and Unit Count Inactivity Timer parameters included in this response are derived from the Charging Data Response previously received from the H-CHF.

* * * End of Changes * * * *