3GPP TSG SA5 Meeting #165	S5-260241rev1

Goa, India, 9-13 February 2026

Source:	Huawei

Title:	Rel-20 pCR 32.872 Solutions evaluation for Key issue #1.2

Document for:	Approval

Agenda item:	7.5.2

Spec:	3GPP TR 32.872

Version:	TR V0.2.0

Work Item:	FS_RoamRE_CH

Comments

This pCR proposes to introduce the solution evaluation on Local Breakout Inter CHFs scenario key issue1.2.

Proposed Changes

* * * First Change * * * *

#### 5.1.5.y	Solutions evaluation for Key issue #1.2

Solutions #1.4 solves Key issue #1.2, providing the handling method of the charging session between V-CHF and CTF when V-CHF detected failure of charging session with H-CHF in inter CHFs scenario. The benefit of the solution is that the V-CHF synchronously releases or maintains the charging session with the CTF and H-CHF, avoids long-term resource consumption for unilateral inactive charging session.

Solutions #1.6 solves Key issue #1.2, describing in inter CHFs scenario, the H-CHF handles abnormal messages in the same way as the CHF as specified in clause 5.5.1.2 of 3GPP TS 32.290[3]. The benefit of the solution is that it increases the robustness of charging system.

Solutions #1.7 solves Key issue #1.2, describing the V-CHF handle the Charging Data Request received from alternative V-CHF as a valid request in inter CHFs scenario. The benefit of the solution is that it increases the service continuity in inter CHFs scenario.

Editor's Note: Further evaluation is currently in progress.

* * * End of Changes * * * *