3GPP TSG-SA5 Meeting #164	S5-255173rev1

Dallas, USA, 17 - 21 November 2025

# Source:	Huawei

# Title:	Rel-20 pCR 32.872 Failure handling of V-CHF when V-CHF detected failure of charging session with H-CHF

# Document for:	Approval

Agenda Item:	7.5.2

1	Decision/action requested

This is a pCR to TR 32.872 Introduce solutions on Local Breakout Inter CHFs scenario key issue2.

2	References

[1]	3GPP TR 32.872: "Study on 5GA roaming charging reliability enhancement".

3	Rationale

This pCR proposes to introduce solutions on Local Breakout Inter CHFs scenario key issue2.

4	Detailed proposal

Propose to incorporate the following change into the TR 32.872 [1]

First change

5.1.4.b	Solution #2.x: V-CHF release the charging session with CTF when V-CHF detected failure of charging session with H-CHF

A possible solution for key issue #2 covering requirements REQ-3GPPCH-LBIC-2, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF.

When the V-CHF, acting as an NF consumer, detects a failure of the H-CHF, it determines how to handle the charging session with the NF (CTF) based on the operator's agreement, e.g. V-CHF may continue the session with NF (CTF) and store the charging information for settlement, as specified in clause 5.5.1.3 of 3GPP TS 32.290[3].

In the case of V-CHF decides to release the charging session between the V-CHF and CTF, the V-CHF sends a Charging Data Response or Charging Notify Request to CTF to indicate that the charging session should be released.

Figure 5.1.4.b-1: V-CHF terminates the charging session with CTF

5.1.4.c	Solution #2.y: V-CHF suspend quota management when V-CHF detected failure of charging session with H-CHF

A possible solution for key issue #2 covering requirements REQ-3GPPCH-LBIC-2, enhanced failure handling for scenarios where a failure is detected between the V-CHF and the H-CHF.

When the V-CHF, acting as an NF consumer, detects a failure of the H-CHF, it determines how to handle the charging session with the NF (CTF) based on the operator's agreement, e.g. V-CHF may continue the session with NF (CTF) and store the charging information for settlement, as specified in clause 5.5.1.3 of 3GPP TS 32.290[3].

In the case of V-CHF decides to continue the charging session between the V-CHF and CTF in online charging scenario, the V-CHF instructs the NF consumer (CTF) to suspend quota management as specified in clause 5.3.2.5 of 3GPP TS 32.290[3], and stores the Charging Data Request(s) or charging information marked with ‘QUOTA_MANAGEMENT_SUSPENDED’ from the NF consumer (CTF).

Annex A (informative):
PlantUML

# A.1	Figures

## A.1.b	Figure 5.1.4.b-1

@startuml

participant "V-SMF(CTF)" as CTF

participant "V-CHF" as VCHF

participant "H-CHF" as HCHF

VCHF <--> HCHF : 1. Failures detected between the V-CHF and the H-CHF

rnote over VCHF #white

2. V-CHF decides to release the charging session with V-SMF(CTF)

endrnote

VCHF -> CTF : 3. V-CHF sends a Charging Notify Request to V-SMF(CTF)

@enduml

End of change