3GPP TSG-SA5 Meeting #164	S5-255486d1

Dallas, USA, 17 – 21 November 2025

Title:	LS reply to IETF Network Slice Application in 3GPP 5G End-to-End Network Slice

Response to:	S5-255025 (LS on IETF Network Slice Application in 3GPP 5G End-to-End Network Slice)

Release:	Rel-19

Work Item:	AdNRM_Ph3

Source:	SA5

To:	IETF Traffic Engineering Architecture and Signaling Working Group (teas)

Cc:	SA2, SA3, RAN3

Contact person:	Jose Antonio Ordoñez jose.antonio.ordonez@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

[1]	https://datatracker.ietf.org/doc/html/draft-ietf-teas-5g-network-slice-application

[2]	3GPP TS 28.531, “Management and Orchestration; Provisioning”, v19.2.0

[3]	3GPP TS 23.003, “Technical Specification Group Core Network and Terminals; Numbering, addressing and identification”; v19.3.0.

[4]	3GPP TS 29.571, “5G System; Common Data Types for Service Based Interfaces;

Stage 3, (Release 19)”, v19.4.0

[5]	3GPP TS 28.541, “Management and Orchestration; 5G Network Resource Model (NRM); Stage 2 and 3”, v19.5.0

[6]	3GPP TS 28.201, "Charging management; Network slice performance and analytics charging in the 5G System (5GS); Stage 2".

[7]	3GPP TS 28.202, "Charging management; Network slice management charging in the 5G System (5GS); Stage 2".

[8]	3GPP TS 28.530, "Management and orchestration; Concepts, use cases and requirements".

[9]	3GPP TS 23.501, "System architecture for the 5G System (5GS)".

[10]	3GPP TS 28.533, "Management and orchestration; Architecture framework".

[11]	IETF RFC9543, "a Framework for Network Slices in Networks Built from IETF Technologies".

# 1	Overall description

3GPP SA5 would like to thank IETF TEAS Working Group for the information provided. 3GPP SA5 has reviewed the informational document “IETF Network Slice Application in 3GPP 5G End-to-End Network Slice” [1], and would like to provide the following feedback.

CSMF (Communication Service Management Function), NSMF (Network Slice Management Function) and NSSMF (Network Slice Subnet Management Function) are mentioned multiple times throughout the document in [1]. SA5 would like to clarify that CSMF, NSMF and NSSMF are non-standardized slicing management functions, but deployment examples of Provisioning MnS producers and consumers for network slicing management. Annex A.8 (informative) of 3GPP TS 28.533 [10] provides a typical deployment example for management of a mobile network including network slicing. For further information on network slicing management, see 3GPP TS 28.531 [2].

The concepts of “RAN Slice” and “Core/CN Slice” are mentioned multiple times throughout the document  [1] (e.g., Figure 2, Figure 6, Figure 7). SA5 would like to clarify these concepts are not defined in normative specifications, and suggests their alignment with the 3GPP definition of Network Slice Subnet, as specified in clause 3.1 of 3GPP TS 28.530 [8].

With regards to Relationship Between IETF Network Slices and 3GPP Network Slices (clause 3.4):

- In page 9, there is a reference to Figure 6. SA5 would like to clarify that this reference should be Figure 5 instead.

- Figure 5 illustrates the relationship between 3GPP domain controllers and IETF Network Slice Controller. SA5 suggests aligning this figure with the terminologies used in the IETF Network Slice Management Architecture defined in RFC9543 [11] and with the terminologies used in 3GPP TS 28.531 [2] (e.g., avoid using non-3GPP concepts such as “RAN/AN NSSMF” and “CN NSSMF”). In addition, SA5 suggests IETF to indicate in the figure what parts are out of IETF scope, like as proposed in Annex A.8 of TS 28.533 [10] (e.g., see TN domain manager box in Figure A.8.1 in [10]).

- It is quoted that “3GPP identifies each E2E network slice using an integer called S-NSSAI”. Charging for Network Slicing in 3GPP is based on the use of S-NSSAI and specified in TS 28.201 [6] and TS 28.202 [7]. SA5 would like to clarify that S-NSSAI is not an integer, but a specific type consisting of an Integer (SST) and 3-octet String (SD); SA5 would also like to note that the example values for S-NSSAI in the document [1] (e.g., 01111111) do not comply with the S-NSSAI format defined by 3GPP.

- It is quoted that “Note that 3GPP uses the terms NSI and NSSI which are a set of network function and required resources (e.g. compute, storage and networking resources) which corresponds to network slice Instance […]”. SA5 would like to clarify the following:

  - NSI and NSSI are terms introduced in 3GPP TS 28.531 [8]. These terms are used interchangeably with NetworkSlice instance (i.e., managed object instance of NetworkSlice IOC) and NetworkSliceSubnet instance (i.e., managed object instance of NetworkSliceSubnet IOC) respectively, as defined in TS 28.530 [8]. An NSI/NSSI is uniquely identified by a SA5 defined management identifier referred as to Distinguished Name (DN).

  - The network slice instance defined in 3GPP TS 23.501 [9] (uniquely identified by a SA2 defined signalling identifier) can be reflected via the NetworkSliceSubnet instance (uniquely identified by an DN) and the allocated resources. For further details, see clause 3.1 of TS 28.530 [8].

With regards to 5G E2E Network Slice Mapping Procedure (clause 4.2,):

- In Figure 7, the following annotations are included: “(e.g., 4)”, “(e.g., 6)” and “(e.g., 1)”. SA5 would like to know whether there is any description for these numbers in the steps of the figure. .

- Steps 1, 2, 3, 4 and 5 allocates 3GPP management system functionalities to NSMF (e.g., receiving requests for network slice instance allocation, determining the network functions and resources, S-NSSAI assignment, etc). Since NSMF is not a standardized management function, SA5 suggests replacing “3GPP NSMF” with “3GPP network slice MnS producer”.

- Step 6 quotes the following: “3GPP NSMF sends a request to an IETF NSC (acting as an NSSMF for transport network, from the perspective of the 3GPP Management System)) for creation of a RFC9543 Network Slice service”. SA5 would like to note that IETF NSC is not in scope of 3GPP management system. For coordination with transport network, 3GPP specifies the “Procedure of TN coordination supporting network slicing” in clause 7.9 of TS 28.531 [1] and the "Relation between GSMA GST, ServiceProfile and SliceProfile" in normative Annex L.2 of TS 28.541 [5]. This annex specifies how TN requirements (input to the TN domain) are derived from TopSliceSubnetProfile.

- Step 8 quotes the following: “The 3GPP NSMF could maintain the mapping relationship between S-NSSAI and IETF Network Slice Service ID”. SA5 would like to clarify that the maintenance of relationship between S-NSSAI and IETF Network Slice Service ID is a functionality not currently in scope of 3GPP network slicing management and charging.

With regards to 5G E2E Network Slice Mapping in Management and Control Planes (clause 5):

- It is quoted that “Build up mapping relationship between NSI identifier and RFC9543 Network Slice Services”: SA5 would like to know whether the NSI defined in IETF refers to the identifier of a NetworkSlice instance, i.e. the Distinguished Name (DN) of a managed object instance of NetworkSlice IOC (see TS 28.541 [5]).

With regards to Mapping EP_transport to IETF NS CE Endpoints (clause 5.1), the document in [1] notes that the translation of EP_Transport parameters into the corresponding parameters for the CE within the IETF network slice is not straightforward for certain scenarios. The document quotes the following: "In these scenarios, additional information are needed to identify the corresponding CE endpoint for example with other parameters defined in EP_transport, including: combined identification for NS CE endpoint […], next hop information […] and QoS profile […]". SA5 would like to clarify the following:

- For the combined identification for NS CE endpoint, the following attributes of EP_Transport IOC  (see clause 6.3.18 in TS 28.541 [5]) can be used: “localLogicalInterfaceInfo”, which is represented as LogicalInterfaceInfo <<datatype>> (see clause 6.3.35 of 3GPP TS 28.541[5]), and  externalEndPointRefList, which is represented as a list of ConnectionPointInfo <<datatype>> (see clause 6.3.41 of 3GPP TS 28.541[5]).

- For the next hop information, the attribute “ipAddress” in EP_Transport IOC attribute can be used for this purpose.

- For the QoS profile, the attribute “qoSProfile” in EP_Transport IOC attribute specifies a set of parameters locally provisioned on both sides of logical transport interface, e.g. DSCP, (see clause 6.4.1 of 3GPP TS 28.541 [5]). In addition, for the SLOs input to the TN domain (TN requirements), SA5 would like to clarify that these are derived from TopSliceSubnetProfile, as specified in Annex L.2 of TS 28.541 [5].

With regards to IETF Network Slice request through IETF Network Slice NBI (clause 7), the document in [1] quotes the following in page 28: "EP_Transport object class does not define a mechanism for active communication of EP_RP loopbacks to the IETF ingress PE device (e.g., no PE-CE protocols)". SA5 would like to clarify that the attribute “routingProtocol” in EP_Transport can be used for this purpose.

# 2	Actions

To IETF TEAS WG

ACTION: SA5 kindly requests IETF TEAS WG take the above feedback into consideration for updating the document in [1].

# 3	Dates of next TSG SA WG5 meetings

SA5#165		9 February – 13 February 2026		India

SA5#166		13 April – 17 April 2026			Malta